NodeRunner/SI(TM) Adapter Setup (NRSETUP) Information

-------------------------------------------------

(C) Copyright 1993 Artisoft(R), Inc.

(Edition 1.0)

Introduction

============

NOTE:  The information in this file does not apply to Micro Channel

users. Instead, refer to your computer manual and your

installation folder for information on using the Micro Channel

configuration program.

Your NodeRunner(TM) adapter is a software independent, jumperless,

software-configurable Ethernet adapter. By default, NodeRunner 2000

Series adapters use the following settings:

IOBASE=300h

IRQ=15

Boot ROM Address=Disabled

IOCS16=Normal

Ethernet Type=Coaxial  (2000/A adapter only)

Mode=Artisoft

This file describes NRSETUP and its use with NodeRunner 2000 Series

adapters.  You will also find descriptions of the NRSETUP switches and

a list of NRSETUP error messages.

About NRSETUP

==============

NRSETUP is the NodeRunner adapter setup utility. You have the following

three options within NRSETUP:

# Automatic NodeRunner Configuration

-----------------------------------

This option automatically sets the NodeRunner's IRQ line

to prevent conflicts with other installed devices. This option

also checks the integrity of the adapter and the network

connection.

# Manual NodeRunner Configuration

-------------------------------

This option allows you to manually set the NodeRunner

parameters to prevent setting conflicts with other devices.

It also lets you configure the adapter for specialized tasks such

as remote booting.

# NodeRunner Diagnostics

----------------------

This option runs full diagnostics on the NodeRunner adapter to

assess the adapter's integrity and check network connectivity.

For more information on the NodeRunner Diagnostics option, see

the section of this On-line manual titled "Using the NRDIAG

Hardware Diagnostics Utility".

Using NRSETUP

==============

All users should run NRSETUP's Automatic Configuration option. By doing

so you eliminate any IRQ conflicts with other installed devices and

detect any hardware problems early.

Run NRSETUP after installing the adapter and the NodeRunner and network

software, but BEFORE running the software.

NOTE:  Make sure no NodeRunner adapter drivers are loaded into memory

when you run NRSETUP.  If you are using DOS V5.0, you can do this

by typing MEM/C at the DOS prompt and viewing the list of programs

loaded. If the NodeRunner driver is running, exit any

applications, power down your PC, restart the PC and do not let

the NodeRunner drivers run from your startup batch file.

To run NRSETUP's Automatic Configuration:

1. Change to the directory containing your network program files.  Type:

NRSETUP  Enter

The NRSETUP Main Options screen appears displaying the NRSETUP options.

2. Select Automatic Configuration.

NRSETUP determines what IRQs are already occupied by other devices.

NRSETUP then makes the any necessary changes to the NodeRunner's

IRQ setting.

To make additional settings changes using Manual Configuration:

Select Manual Configuration.

Two screens appear. The top screen displays adapter information and

statistics. Values displayed in this screen are read-only and cannot be

modified. The bottom screen shows the NodeRunner's configuration

parameters. See steps 3 and 4 below for instructions on changing

the settings.

3. Highlight the setting you want to change and press Enter. Then select

the new value.

4. When you are finished, press Esc to exit NRSETUP and save your

changes.  If you do not want to save the changes, press F10.

Configuration Parameters

========================

The configuration parameters for the NodeRunner adapter are described

below.  Default values appear in parentheses.

I/O Base (300)

--------------

This setting indicates the start of the block of I/O ports used by the

adapter. The default setting of 300 usually does not conflict with an

I/O Base setting in use by previously installed hardware.  If this

setting does conflict with any existing hardware, change this option

to a different setting.

IRQ (15)

--------

This setting establishes the Interrupt Request Line to be used by the

adapter. The default setting of 15 usually does not conflict with an IRQ

setting in use by previously installed hardware.  If, however, you do

detect a conflict, highlight the IRQ setting and press Enter.  Some

commonly used IRQs include:

IRQ3 -- COM2:

IRQ4 -- COM1:

IRQ7 -- LPT1:

NOTE:  To use IRQ 10 or 15, you must place the adapter in a 16-bit

(long) expansion slot in your computer.

Boot ROM Address (Disabled)

---------------------------

This setting disables or specifies the memory address used for the

optional boot ROM chip onboard the adapter. If you have purchased this

chip, install and configure it according to the instructions included

with the chip.  If you did not purchase the boot ROM chip, leave this

setting in Disabled mode.

1OCS16 Timing (Normal)

----------------------

This setting selects the timing of the I/OCS16 bus control term.  If you

are unsure about this setting, leave it at Normal.  If you experience

problems such as intermittent disconnects, change this setting to Early.

Some PCs require early assertion of the IOCS16 signal for proper 16-bit

operation. If you know that you have such a PC, highlight the 10CS16

Timing parameter and press Enter to change the setting to Early.

Ethernet Type (Coaxial) (2000/A adapter only)

---------------------------------------------

Selects among the Ethernet cable types.  The options presented in

this field depend upon the type of adapter.  Possible cable types

include AUI (10Base5, DB-15, thick Ethernet), Coaxial (10Base2, BNC,

cheapernet, thin net), and UTP (10Base-T, twisted-pair).  This option

does not appear if the adapter supports only one cable type.

Mode (Artisoft)

---------------

This setting selects the compatibility mode of the adapter.  If you

are using the adapter in an Artisoft LANtastic/AI network, leave this

setting at the default.  If you are using the adapter in a Novell

network and are using the Artisoft NetWare drivers provided on your

NodeRunner diskette, use the default Artisoft setting.  If you are

using Novell-supplied NE1000 or NE2000 drivers, change this setting to

NE1000 or NE2000 emulation.  If you want to install your adapter in an

8-bit slot in an Artisoft network, choose Artisoft/8.

Configuring Multiple Adapters

=============================

When configuring multiple NodeRunner adapters, you must specify a unique

I/O Base and IRQ setting for each adapter.  Complete these steps:

1. Install the first adapter.

2. Run NRSETUP as instructed in the previous section and specify a

non-default I/O Base and IRQ setting.  The default settings for these

options are I/O Base 300 and IRQ 15.

3. Install the second adapter.

4. If necessary, configure the second adapter using NRSETUP.  Generally

this is not necessary unless settings for the second adapter conflict

with any other adapters installed before the NodeRunner cards.

To specify that adapter you would like to configure, enter the adapter's

I/O Base address on the NRSETUP command line using the IOBASE= switch.

For example, to configure an adapter that uses the default I/O Base

address (300) you would issue the command:

NRSETUP IOBASE=300

NRSETUP Command Line Switches

==============================

Below are the optional parameters that you can include on the NRSETUP

command line.  For each switch, you will find its name, the valid range

of options, the default value if any, and a description.

@ Range 0 - 3000 byte switch file

---------------------------------

Instructs NRSETUP.EXE to look for further options in an ASCII text file

that you have previously created.  You can place as many command line

switches in the file as you wish.  This is useful for reducing the length

of the command line if you will be using many switches.  Switches may be

used on the command line after the switch filename, and the switch values

will take precedence over those in the switch file.  You may invoke switch

files from within switch files as often as you wish.

You may also include comments for each line of the switch file.  Comments

may be at the beginning of each line or after switches, and must be

preceded by a semicolon (;).

HELP or ?

---------

Provides you with a list of command line switches for NRSETUP, along with

the valid range of values for each switch.  When you use the HELP or

? switch, the program does not install itself.

IOBASE= (300) 300 320 340 360 hex

----------------------------------

Allows you to specify the Input/Output port address for the adapter

without using the menu interface.  If you are using multiple adapters,

each card must have a unique IOBASE= setting.

MONOCHROME

----------

This switch specifies that the program will run in monochrome mode.  This

can help make the screen text easier to read on some monitors.

SHOW

----

Instructs NRSETUP to display the current settings of all installed

NodeRunner adapters.

NRSETUP Messages

=================

Below is a list of the messages returned by NRSETUP.  For each you

will find the message text, an explanation of the message, and any

corrective steps that you should take.

--------

Message:  Adapter may be in use!  Press Enter to proceed or Esc to abort.

Meaning:  NRSETUP has determined that the adapter is active.  This may be

the result of a warm boot while a network driver was active, or a

non-Artisoft driver may be active.

Remedy:   Press Enter to proceed.  This message warns you that the IRQ

the NodeRunner adapter is set for may be in use by another

device.  This message will occur if the NR driver was

loaded any time since the PC was turned on and can usually

be ignored. If the message occurs immediately after turning

on the computer and BEFORE loading any network software, you

should select another IRQ for the NodeRunner adapter to use.

--------

Message:  Corrupted EEPROM contents (CRC) - Call Artisoft Tech. Support

Meaning:  An error has occurred in the NodeRunner's EEPROM chip.

Remedy:   Turn the computer off then on and attempt to run NRSETUP

again.  If the message persists contact Artisoft Technical

Support.  The adapter may need service.

--------

Message:  Corrupted EEPROM contents (SIG) - Call Artisoft Tech. Support

Meaning:  An error has occurred in the NodeRunner's EEPROM chip.

Remedy:   Turn the computer off then on and attempt to run NRSETUP

again.  If the message persists contact Artisoft Technical

Support.  The adapter may need service.

--------

Message:  EEPROM verify has failed.

Meaning:  An error occurred reading information back from the adapter's

EEPROM chip.

Remedy:   Run NRSETUP again.  If the problem persists, contact Artisoft

Technical Support.

--------

Message:  Internal data structure error.

Meaning:  An error has occurred in the NodeRunner adapter EEPROM chip.

Remedy:   Turn the computer off then on and attempt to run NRSETUP

again.  If the message persists contact Artisoft Technical

Support.  The adapter may need service.

--------

Message:  Network driver detected -- Remove before attempting configuration.

Meaning:  You have attempted to configure the adapter with a network

driver loaded.

Remedy:   Power down the computer, or remove the network driver, then

restart it without running the network software.  You may then

configure the adapter.

-------

Message:  Network transmit failure -- possible bad connection or

termination.

Meaning:  The NodeRunner adapter cannot successfully transmit a packet.

Remedy:   Check all cable connections, terminators, T-connectors and hubs.

-------

Message:  No changes required to NodeRunner configuration.

Meaning:  No NodeRunner parameters have changed. Writing to the EEPROM

is not required.

Remedy:   Status message only. Press Enter to continue.

-------

Message:  NodeRunner controller failure -- possible I/O Base conflict.

Meaning:  You have another device already using the NodeRunner's

I/O Base address.

Remedy:   Relocate the conflicting device to another I/O Base address. Or

run the NRMOVE utility, followed by NRSETUP, to specify a

different I/O Base address for the NodeRunner adapter.

--------

Message:  No NodeRunner adapters detected.

Meaning:  You attempted to run NRSETUP or use the NRSETUP SHOW command, but

the program was unable to find any NodeRunner adapters, or an

I/O Base conflict exists between the NodeRunner adapter and

another card.

Remedy:   Make sure the adapter is installed in an expansion slot of your

PC before running NRSETUP, or remove conflicting hardware.

--------

Message:  NodeRunner at I/O Base hhh is currently active - Cannot access

configuration.

Meaning:  You have issued the NRSETUP SHOW command while a network driver

is loaded.

Remedy:   Power down the computer, then restart it without running the

network software.  You may then show NodeRunner status.

-------

Message:  Onboard memory failure -- possible I/O Base conflict or IOCS16

timing.

Meaning:  The diagnostic has failed the memory test.

Remedy:   Run the Automatic Configuration option. Or use Manual

Configuration to reset the IOCS16 timing parameter. If the

problem persists, check for an I/O Base conflict.

--------

Message:  Selected IRQ is in use.  Press Enter to proceed.  Esc to abort.

Meaning:  The IRQ you have selected is currently in use.

Remedy:   Select a different IRQ.

-------

Message:  Unable to communicate with adapter installed in 16-bit slot.

Meaning:  The Automatic Configuration option cannot get reliable data

transfers with either IOCS16 option.

Remedy:   Check for possible I/O Base conflict. If none found, contact

Artisoft Technical Support.

-------

Message:  Unable to determine cable type -- check network connection.

Meaning:  The Automatic Configuration option cannot successfully transmit a

packet on any of the adapter's ports.

Remedy:   Check to make sure the NodeRunner's network cable connections

are secure.

--------

Message:  Unable to locate NodeRunner Controller

Meaning:  NRSETUP cannot detect any NodeRunner adapters installed in

the PC, or there is an I/O Base conflict with another hardware.

Remedy:   Try using NRMOVE to eliminate the I/O Base conflict. See the

NRMOVE file in the On-line Manual for instructions. If NRMOVE

does not solve the problem, remove the conflicting hardware or

configure the NodeRunner in another PC.

--------

Message:  Unknown adapter type

Meaning:  The installed NodeRunner adapter is not known to this version

of NRSETUP.

Remedy:   Upgrade to a newer version of NRSETUP.