COMSETUP.DOC

INFORMATION FOR SETTING UP KT-FleaMarket with Non-Standard Com

Ports and High Speed Modems.

Create a file using your favorite text editor called KTFMNODE.x in

the KT-FleaMarket directory.  The 'x' is the node number.  If you're

running only one node, make it KTFMNODE.1.  Actually, the name of

the file is unimportant.  I just feel it would be easier to keep

track using the .<nodenumber> method.

In this file, you may set the following variables:

CTSRTS

FIFO

ADDRESS

IRQ

COMPORT

UNIT

STATUSPORT

MULTIPORT

If you eliminate any of these variables, they will be ignored.

Don't worry about setting all of them, only the ones you'll use.

Now for an explaination of each:

------

CTSRTS

------

If you're using a High Speed Modem (i.e. 2400/MNP or 9600b or

higher) insert the following command in your KTFMNODE.x file:

CTSRTS=ON

If you're NOT using a High Speed Modem, don't use the CTSRTS=

command.

----

FIFO

----

From the Telepathy Users Guide:

"...The 16550 is an advanced version of the 8250 and

16450 UART's used in most PC serial ports.  In addition

to all the standard features, it has a pair of 16-byte

first-in-first-out (FIFO) buffers for transmitting and

receiving, where the standard chips have one-character

buffers.  The FIFO's reduce the amount of work the

processor has to do to keep up with the UART, permitting

higher sustained baud rates.

It does no harm to try to enable the FIFOs if the UART

is not a 16550.  It is probably a good idea always to

enable the FIFOs, especially if you will be using baud

rates of 9600 or higher, or several ports at once..."

That pretty much sums it up...

If you wish to enable the FIFOs, insert the following command in

your KTFMNODE.x file:

FIFO=ON

If you do not wish to enable the FIFOs, don't use this command.

So...for a system with standard Com ports and a high speed

modem, your KTFMNODE.x file would look like this:

CTSRTS=ON

FIFO=ON

That's it. If you're using Non-Standard Com Ports, read on...

----------------------

NON-STANDARD COM PORTS

----------------------

�������������������������������������������������������������������͸

� If you'll be using a DigiBoard Multi-Port Serial Card, please     �

� read this section then proceed to the 'DIGIBOARD' section.  If    �

� you are not using a DigiBoard, read just this section.            �

�������������������������������������������������������������������;

�������������������������������������������������������������������͸

� This and the following section (DIGIBOARD), is assuming you have  �

� a decent understanding of Comports, IRQs, Addresses, etc.  In     �

� most cases (1 and 2 node BBS's), you'll not need these features.  �

�������������������������������������������������������������������;

This section will deal with nonstandard address and/or IRQ numbers.

Each serial port is identified by two numbers, an I/O base

address and an IRQ (interrupt request) number.  These values are

set by switches or jumpers before a serial card is installed, or

sometimes by the CMOS setup program when the serial port is on

the motherboard.  Initially, KT-FleaMarket will recognize the

first four standard serial ports:

PORT            ADDRESS         IRQ

-----------------------------------

1               1016            4

2                760            3

3               1000            4

4                744            3

�������������������������������������������������������������������͸

� NOTE: The address is listed as a DECimal NOT HEXadecimal          �

�������������������������������������������������������������������;

Beoynd the standard configurations above, you'll need to tell

KT-FleaMarket what's - what.  The commands to do this are:

COMPORT=xxxx    1-9999

ADDRESS=xxxx    Decimal Value Of Base I/O

IRQ=xxxx        Decimal IRQ Number

NOTE: If you use the COMPORT= command, you will override the

value sent by your BBS package through the <userfile> (i.e.

DOOR.SYS).

So...Let's say your COM3 was a standard ADDRESS but the IRQ was

5. The commands you would insert into the KTFMNODE.x file would

be:

ADDRESS=1000    (Decimal Value Of Base I/O)

IRQ=5

The COMPORT number sent from the <userfile> (i.e.

DOOR.SYS) would be 3, therefore, you don't need to issue

the COMPORT= command (why set ComPort to 3 when it's

already set to 3 ?).

If you were using COM9, you would insert these commands:

COMPORT=9

ADDRESS=xxxx    (DECIMAL BASE I/O ADDRESS)

IRQ=x           (IRQ NUMBER)

If the COMPORT number sent from the <userfile> (i.e.

DOOR.SYS) were different than the COMPORT=x command, the

COMPORT=x command will override it.  This shouldn't

ever happen though if your BBS is set up correctly.

ALWAYS USE THE COMPORT=x COMMAND IF THE COMPORT IS

GREATER THAN 4 (just to be sure).

�������������������������������������������������������������������͸

� When specifying an ADDRESS=, you MUST convert the HEXadecimal     �

� I/O address to DECimal.  I know this isn't standard practice,     �

� but for now, it's the easiest way (in programming terms).  Most   �

� scientific calculators have the ability to convert from Hex to    �

� Dec. If you're a Windows User, the Calculator in the Accessories  �

� Group has this ability.                                           �

�������������������������������������������������������������������;

----------

DIGIBOARD�

----------

!��������������������������������������������������������������������!

� THE DIGIBOARD FEATURES HAVE NOT BEEN FULLY TESTED.  PLEASE SEE THE �

� BETATEST.DOC FILE FOR MORE INFORMATION!                            �

!��������������������������������������������������������������������!

�������������������������������������������������������������������͸

� If you're NOT using a DigiBoard MC/X or PC/X Multi-Port Serial    �

� Card, you don't need to read this section. Instead, read the      �

� section entitled 'NON-STANDARD COM PORTS'.                        �

�������������������������������������������������������������������;

A combination of STATUSPORT=, UNIT=, COMPORT=, ADDRESS=, and

IRQ= commands configures KT-Shopper to recognize a DigiBoard

MC/X or PC/X multi-port serial card.  The command MULTIPORT=

command configures KT-FleaMarket whether you're using a PC/X or

MC/X type card.

Add the following commands to your KTFMNODE.x file:

MULTIPORT=xx/X          "PC/X" or "MC/X".

STATUSPORT=xxxx         Decimal Base I/O address for the

card's status port.

COMPORT=xxxx            The ComPort Number

ADDRESS=xxxx            Decimal Base I/O address of the

COMPORT.

IRQ=xxxx                IRQ of the COMPORT.

UNIT=xxxx               Identifies which of the physical

ports on the card is to be

assigned to this logical port

number.  Unit Numbers start from

0, so a four-port card has unit

number 0-3, an eight-port card

has 0-7, and a sixteen-port card

has 0-15, etc.

A typical KTFMNODE.x when using a DigiBoard PC/4 card installed

on IRQ 3, with status port at 320 (140 hex) and a serial port at

256 (100h), 264 (108h),  272 (110h), and 280 (118h) would be:

���������������������������������������������������������������ͻ

� KTFMNODE.1    � KTFMNODE.2    � KTFMNODE.3    � KTFMNODE.4    �

���������������������������������������������������������������͹

� CTSRTS=ON     � CTSRTS=ON     � CTSRTS=ON     � CTSRTS=ON     �

� FIFO=ON       � FIFO=ON       � FIFO=ON       � FIFO=ON       �

� MULTIPORT=PC/X� MULTIPORT=PC/X� MULTIPORT=PC/X� MULTIPORT=PC/X�

� STATUSPORT=320� STATUSPORT=320� STATUSPORT=320� STATUSPORT=320�

� IRQ=3         � IRQ=3         � IRQ=3         � IRQ=3         �

� COMPORT=1     � COMPORT=2     � COMPORT=3     � COMPORT=4     �

� ADDRESS=256   � ADDRESS=264   � ADDRESS=272   � ADDRESS=280   �

� UNIT=0        � UNIT=1        � UNIT=2        � UNIT=3        �

���������������������������������������������������������������ͼ

NOTE: The order in which the commands are placed in the file is not

important.

That's about it.  I would appreciate feedback on these new features

(Nonstandard/DigiBoard).  Let me know if you like the way I've set

it all up.  If not, tell me that also.

Final Note: My hard drive did the sudden stop thing the other day

and my Word Processor is now in the Text Heavens...Please excuse any

speelleeng errers I may have made....It's hard to believe how much

we rely on our spell checkers these days...<g>.