File MODEMS.DOC                                                  September 1993

MS-DOS KERMIT MODEM DIALING SCRIPTS

Christine M. Gianone, Frank da Cruz

Columbia University

John Klensin

United Nations University

The following modem dialing scripts are available for use with the DIAL

command in MS-DOS Kermit 3.12 and later.  Store the desired files in the same

directory as your MSKERMIT.INI file.

The DIAL command is a macro defined in the standard MSKERMIT.INI file on the

MS-DOS Kermit diskette.  It looks up the number (or name) in your dialing

directory, making the appropriate substitution, and passes the phone number to

your dialing script in a variable.  See "Using MS-DOS Kermit" and KERMIT.HLP

(MSKERM.HLP) for details about the DIAL command and the dialing directory.

If you lack the DIAL macro definition, you can define your own simple DIAL

macro (which does not use the dialing directory) as follows:

DEFINE DIAL TAKE xxx.SCR

where xxx is the modem type, described below.

The default modem type (if you are using the standard MSKERMIT.INI file) is

HAYES.  To select a different type of modem for dialing, do any one of the

following:

1. Type SET MODEM=xxx at the DOS prompt before starting MS-DOS Kermit, or

2. Add SET MODEM=xxx to your AUTOEXEC.BAT file and reboot your PC, or

3. Type DEFINE _MODEM xxx at the MS-DOS Kermit prompt, or

4. Add DEFINE _MODEM xxx to your MSCUSTOM.INI file.

xxx is name of the modem, corresponding to the part of the dialing script

filename before ".SCR".  For Telebit T3000 modems, for example, the script

file is called T3000.SCR, and "xxx" would be "T3000".  If you obtain these

files over the network, you should rename as shown in the DOS Filename column

below.  Here are the dialing scripts that are supplied with MS-DOS Kermit:

Modem Type             DOS Filename  Internet               BITNET/EARN/CREN

*  AT&T DataPort 14400    DATAPORT.SCR  kermit/a/msmdatap.scr    MSMDATAP SCR

Hayes 1200 or 2400     HAYES.SCR     kermit/a/msmhayes.scr    MSMHAYES SCR

*  Hayes Ultra 144        ULTRA144.SCR  kermit/a/msmultra.scr    MSMULTRA SCR

*  Multitech MT1432       MT1432.SCR    kermit/a/msmmt1432.scr   MSMMT1432 SCR

*  Penril Alliance V.32   PENRIL.SCR    kermit/a/msmpenril.scr   MSMPENRIL SCR

*  Practical Peripherals  PP14400.SCR   kermit/a/msmpp14400.scr  MSMPP1440 SCR

Rolm CBX DCM           ROLM.SCR      kermit/a/msmrolm.scr     MSMROLM SCR

*  SupraFAXmodem V.32bis  SUPRA.SCR     kermit/a/msmsupra.scr    MSMSUPRA SCR

*  Telebit QBlazer V.32   QBLAZER.SCR   kermit/a/msmqblazer.scr  MSMQBLAZER SCR

*  Telebit T3000 V.32bis  T3000.SCR     kermit/a/msmt3000.scr    MSMT3000 SCR

*  US Robotics Sportster  SPORT.SCR     kermit/a/msmsport.scr    MSMSPORT SCR

Vadic VA2400PA         VA2400PA.SCR  kermit/a/msmva2400.scr   MSMVA2400 SCR

If your modem does not appear in this list, feel free to adapt one of these

scripts to work with your modem (and send your new script back to Columbia so

others can use it too).

WHAT THE DIALING SCRIPTS HAVE IN COMMON

These scripts use your modem's default dialing method, pulse or tone; they do

not specify one or the other, since neither method is supported by all

telephones everywhere.  To force Tone dialing, begin your phone number with T,

for example:

DIAL T7654321

Similarly, to force pulse dialing, start the phone number with P.

If you give a DIAL command whose telephone number is simply = (equal sign),

the modem will be initialized, but no call will be placed.  In some cases,

the dialing script will also ask the modem to display its configuration.

When dialing a real phone number, you can include special characters in the

phone number to accomplish pauses, wait for secondary dialtone, etc.  See your

modem manual.

If you dial a number that is busy, most of these scripts will wait 30 seconds

and then redial automatically, up to 5 times.  You can cancel the redial

operation by pressing any key after you see the message:

Line is busy, will dial again in 30 seconds.

Press any key to cancel...

Each dialing script returns SUCCESS if dialing succeeds and FAILURE if it

doesn't, so you can use an IF FAIL or IF SUCCESS statement after a DIAL

command in a script.

THE ROLM DIALING SCRIPT

ROLM.SCR is for the Rolm/Siemens (formerly IBM) Computerized Branch Exchange

(CBX) data communications module (DCM).  It dials at your current speed, and

does not change speeds since the DCM is speed-sensitive.  It does not attempt

to redial if the line is busy.  For Rolm 244PCs, use HAYES.SCR, and always use

Tone dialing.

THE HAYES 1200/2400 AND VADIC 2400PA DIALING SCRIPTS

The Hayes 2400 script, HAYES.SCR, should work on any Hayes-1200, Hayes-2400,

or compatible modem.  It does not change any modem settings (S registers).  It

assumes that the modem changes its interface speed to match the negotiated

modulation speed, if the modulation speed is reported as 1200 or 2400;

otherwise, Kermit does not change speed.

The Vadic 2400PA script, VA2400PA.SCR, works in much the same way, except it

also recognizes a 300 bit-per-second connect message.  The Vadic modem

supports only pulse dialing.

HIGH-SPEED MODEM SCRIPTS

The entries marked with "*" above are for high-speed modems that include

error-correction and data-compression features.  These scripts attempt to use

these modems at a fixed interface speed of 57600 bps (or 38400 bps, if that is

the highest speed supported by the modem), to allow the modem's data

compression to operate at its full effectiveness, and they enable hardware

flow control (RTS/CTS) to prevent loss of data.  Note that flow control will

be fully effective only if the answering modem and computer also have an

effective flow control method between them.

The high-speed modem scripts:

. Configure the modem to echo commands, issue verbose result codes, and

hang up upon loss of DTR from the PC (as when you tell Kermit to HANGUP).

(If Kermit's HANGUP command doesn't work with your modem, you can find

a workaround in the KERMIT.BWR (MSKERM.BWR) file.)

. Start out at the highest supported modulation, V.32bis, with downwards

negotiation enabled (V.32, V.22bis, etc.)  If a lower modulation technique

is negotiated, the interface speed remains fixed and the modem does "speed

buffering", for which effective flow control is an absolute requirement.

. Enable error correction and compression, starting out at the highest

supported levels (V.42 and V.42bis) and falling back to lower levels

(usually MNP), or to none at all.

. Configure to modem to pass BREAK signals through transparently when you

type Alt-B.

Dialing     Interface   Highest      Error        Data

Script       Speed      Modulation   Correction   Compression

DATAPORT.SCR 57600      V.32bis      V.42->MNP    V.42bis->MNP

PENRIL.SCR   38400      V.32(bis?)   V.42->MNP    V.42bis->MNP

PP14400.SCR  57600      V.32bis      V.42->MNP    V.42bis->MNP

QBLAZER.SCR  38400      V.32         V.42->MNP    V.42bis->MNP

SPORT.SCR    57600      V.32bis      V.42->MNP    V.42bis->MNP

SUPRA.SCR    57600      V.32bis      V.42->MNP    V.42bis->MNP

T3000.SCR    57600      V.32bis      V.42->MNP    V.42bis->MNP

ULTRA144.SCR 38400      V.32bis      V.42->MNP    V.42bis->MNP

The Telebit T3000 script should also work on the Telebit WorldBlazer.

The Telebit QBlazer script should also work on the Telebit T1600.

The Hayes Ultra 144 gives you a failure code if the modem reports NO CARRIER:

0  Normal hangup

4  Physical carrier loss

5  Error control was required but was not negotiated

6  Other error-control modem did not respond to feature negotiation

7  Other modem is synchronous-only

8  Modems could not find a common framing technique

9  Modems could not find a protocol in common

10  Incorrect feature negotiation message sent by other modem

11  Timed out waiting for synchronous data

12  Normal disconnect initiated by other modem

13  Other modem did not respond after many retransmissions

14  Protocol violation

15  Compression failure

For further information, read the script files themselves.  They are ordinary

text files that you can TYPE, PRINT, or view with a text editor such as DOS

5.0 EDIT.

(End of MODEMS.DOC)