Seagate Technoloy, Inc.

FINDTYPE.DOC

Companion document file for FINDTYPE.EXE version 1.21

The FINDTYPE.EXE program is designed to aid in the integration of

a Seagate disc drive into an AT compatible computer system.

This program is designed to benefit those without an extensive

technical background as well as those more experienced systems

integrators.

---------------------------------------

DISCLAIMER

This software is provided, as-is, without warranty of any kind,

either expressed or implied, including, but not limited to, the

implied warranties of merchantability and fitness for a particular

purpose.  In no event shall Seagate Technology, Inc., be held liable

for any loss of profit or any other commercial damage, including but

not limited to special, incidental, or other damages.

Seagate Technology, Inc., retains all rights to the FindTYPE Program

but does grant the right to make unlimited, unaltered copies of the

FindTYPE program provided there is no charge to the users.

The three files you should have found within FINDT120.ZIP are:

FINDTYPE.EXE    the executable program

FINDTYPE.DOC    the accompaying document file

DRIVETYP.TXT    a short text file essay on drive types settings

README.1ST      text file describing file sizes and dates.

Thank you for your interest in Seagate Technology, Inc.'s FindTYPE

software.  Please submit your comments and suggestions to the

following address:

Seagate Technology, Inc.

P. O. Box 66360

Att: Technical Support

Scotts Valley, Ca. 95067-0360

For Seagate Technical Support Bulletin Board:

(408)438-8771 [300-9600 HST, V.32, V.42bis, MNP 3/5, N-8-1]

---------------------------------------

Swift, Elite, Sabre, MacWren, Wren Runner, Zone Bit Recording,

ZBR, FSD, RSD and FindTYPE are trademarks of Seagate Technology,

Inc.

Seagate, Seagate Technology, Wren and the Seagate logo are

registered trademarks of Seagate Technology, Inc.

IBM PC/XT/AT, and PC-DOS are registered trademarks of

International Business Machines Corporation.

MS-DOS is a registered trademark of Microsoft Corporation.

DISK MANAGER is a registered trademark of ONTRACK Computer

Systems, Inc.

All registered trademarks and unregistered trademarks are the

sole property of their respective companies.

====================================================================

DESCRIPTION

The purpose of this program is to help a systems integrator or

end-user to install a Seagate disc drive into any AT type computer.

The program reads the system BIOS to determine which drive types are

supported. If an exact match is found, the program will display the

correct type so that this information may be used in the computer's

CMOS setup program.

If an exact match is not found, the program will scan the BIOS

and select a drive type which will most efficiently use the

capacity of the drive being installed. It will also calculate the

percentage of utilization with the selected drive type. The

precompensation start track and reduced write current start track are

also compared and if an exact match cannot be found, the closest one

will be selected.

Some BIOS will support a custom or user defined drive type; if

available, it is usually drive type 47 or 48.  This allows the user to

input the number of cylinders, heads and sectors. This CUSTOM drive

type may be used when the closest type supported by the BIOS gives

less than 100% utilization of the drive. The program has the ability

to scan the BIOS for support of CUSTOM drive type. If that support is

detected, you will be prompted to that effect. However, refer to the

documentation for your computer to verify that support and for drive

parameter entry instructions.

In the case of ZBR (Zone Bit Recorded) drives, for AT(ide) interface

drives the true physical parameters cannot be set in CMOS, therefore

the suggested translation is only given. For SCSI drives that are ZBR,

an average of the Sectors per Track is given and is rounded down from

its actual value to a whole number.

Note: A "custom" or "user-definded" drive type may ask for a numerical

value for the Write Precompensation cylinder and for the Landing Zone

cylinder.  A basic rule-of-thumb for drive models that do not require

the old Write Precomp technique or a Landing Zone because they are

Auto-Parking is to add 1 to the cylinder value being used. As an

example: If cylinders equaled 820, then Write Precomp and Landing Zone

would be entered as 821.

============================

The FindTYPE ver. 1.21 program has the ability to reset the drive type

number in CMOS. After you have highlighted your drive model, read the

online information that pertains to your interface, and considered the

ramifications of changing the drive type in CMOS (which you DO NOT

WANT TO DO if there is data on the drive), then by pressing 0 for the

first physical drive or 1 for the second physical drive you begin the

reset process. There are Y/N prompts along the way to allow you to

abort this process. After the drive type has been reset, the system

will reboot as the program is exited.

============================

When installing AT interface (IDE) drives with more than 1024

cylinders and using a BIOS that supports a custom drive type, the

program can calculate the optimum translation parameters for the drive

being installed.

If you are installing a drive which is not supported by the BIOS,

and the BIOS does not support the CUSTOM or USER DEFINED drive

type, you have four options:

1. Use a controller with a ROM Bios on-board that will support

the interface being used.

2. Replace the system BIOS with one that supports the drive or that

supports a CUSTOM drive type.

3. Use a third party software package, such as Disk Manager, to

install the drive and provide a device driver solution.

4. Use the closest drive type which is supported by your BIOS and

achieve only partial utilization of the capacity of the drive. You may

find that the percentage of utilization is acceptable.

There are also options which allow the printing of the system

BIOS drive tables and a list of parameters for Seagate drives.

============================================================

Help screen notes for Findtype v1.21    F1 from the Main Menu

"Find the BEST 'Standard' or 'Custom' Type"

This selection will display, on the right hand side of the screen, a

column of Seagate disc drive model numbers. Using the up/down arrow

keys, highlight the model of your choice and the two other boxes will

display specific information about that drive. This information

includes the Seagate model #, old Imprimis model # (if applicable),

the interface, form factor (size), true physical drive geometry, a

'custom' translation geometry (only for AT interface drives), and the

BEST 'standard' drive type number from your system's BIOS. You should

ALWAYS double check the number from inside the CMOS setup, as some odd

BIOS's have caused the numbering to be off by 1.

When you have highlighted the model of your choice, you can press the

Enter key to pop-up additional information that pertains to your drive

and the setting drive types for that particular interface.

As the program was started, FindTYPE made an attempt to determine

whether or not a 'custom' or 'user definable' drive exists in your

system BIOS.  If it was found, a "tweedle" tone was given.  It is VERY

important to note that this search for a 'user definable' drive type

is not foolproof; rarely, at times, a 'user definable' might get

missed or indicated as found when it really isn't.

"Output the BIOS Drive Type Table"

This selection will "output" to your choice of three devices: screen,

printer, or drive (save as a file).  The result is a listing of your

system's BIOS Drive Type Table. This is a simple read function.

"Output Seagate Drive Parameters"

This selection will "output" to your choice of three devices: screen,

printer, or drive (save as a file). The result is a listing of all

Seagate disc drives and specifications that are supported by this

version of FindTYPE.

"Display Seagate Model # Information"

This selection will display an "anatomy" chart of Seagate model

numbers. This chart will describe the various components that make up

a Seagate model number.

=======================================================

Revision History

10-21-90 v1.10  Initial release, using simple comand line switches,

detect of user-definable drive type.

06-03-91 v1.20  Color added, new model drives added, user menu

interface changed, added ability to reset CMOS,

support for PS/2 family systems, help screens added.

12-13-91 v1.21  New Models added: 1400a 1401a 1481n 1581n 1980n 3051a

3144a 3283a 3283n 9052a 9077a 9096a 9144a 11200n

41601n 41651n 42100n 42101n 42400n 43400n 43401n.

Glossary removed from FINDTYPE.DOC.

****    End-of-File   FINDTYPE.DOC    ****