Asynchronous Communication Test Software (ACT) for NS16450/8250A UARTs

======================================================================

National Semiconductor Corporation

Greg DeJager      February 8, 1989

Adapted from LBT.C -- Loopback Test Rev 1.1

Developed by:  Brian A. Berg

BERG SOFTWARE DESIGN

Summary of Files

--------------------------

ACT.C		C source code: main program, subroutines and interrupt handler

ACT.EXE		executable ACT program; only this file is needed to run the ACT

SERIO.H		header file containing UART definitions for C code use

STDHDR.H	header file containing "standard" definitions for C code use

Intro, Hardware Setup and Program Operation

===========================================

This test was developed to run on all IBM AT or AT compatible architectures

as well as IBM PS/2 Model 30 - 80 machines.  System requirements include

two separate machines with an NS16450 or INS8250A UART located at COM1, 2 or 3

and an RS-232 DTE to DTE interface between them.  The DTE-DTE interface must

have the following connections:

Tx----------------------------Rx

Rx----------------------------Tx

RTS--------------------------CTS

CTS--------------------------RTS

GND--------------------------GND

The executable software is titled ACT.EXE.  To run the test, the above

hardware must be set up and after entering the proper directory, the user

must type ACT <enter> on both machines.  The program will prompt the user

to start both machines and then hit a key to begin the test.  This procedure

is necessary to ensure that all modem control outputs are deasserted before

program start-up so that a machine won't begin transmitting before the

other is ready to receive.  Full duplex transmission begins after a key is

hit on both machines and each machine has exchanged initial RTS signals.

Proper test operation is indicated by a '|' and '_' character being printed

to the screen every 2560 characters received.

All operations are interrupt driven.  UART line status, receiver and transmitter

interrupts are enabled.  When not servicing interrupts, the CPU sits in a

loop checking for a keyboard hit (means for user to stop test) and for errors

produced within the interrupt service routines.  A transmitter interrupt is

generated whenever the transmit holding register empties.  The CPU services

the interrupt by writing the next character to the holding register ONLY if CTS

is asserted.  If CTS is not asserted a timer variable is incremented and compared

to a large number (50).  If 50 transmit interrupts occur in succession

without CTS being asserted, the program assumes that the other machine has

stopped and the program is ended.

A receiver interrupt is generated when the receiver buffer gets a new character.

The CPU immediately deasserts RTS to stop any further transmission from the

other machine.  It then reads the byte out of the receiver buffer comparing

it to an expected value.  Data transmitted and received sequences between 00

and FFH. If a mismatch occurs, the expected and received data are displayed

and the program stops.  A line status interrupt results from an Overrun, Framing

or Parity error or Break condition.  Upon receiving the status interrupt, the

program prints the corrupted data and the line status register value and exits

the program.  Inconsistencies between the Interrupt ID and Line Status Registers

also cause error messages to be printed and the program to stop.

There is a restriction associated with porgram operation.  The maximum baudrate

that can be selected is 56k; however, this baudrate is limited by the processor

speed and the host system.  A 10Mhz AT compatible and 12Mhz PS/2 Model 30

operate reliably at a maximum of 19.2k baud.  At higher baud rates operation

is system dependent due to the overhead involved with processing the interrupts.

The default com port is COM1 and default baudrate is 9600.  However, the user

can control these options by specifying arguments on the command line.  Arguments

must be separated by spaces.  The first argument is a selection of COM1, COM2

or COM3 (eg.  ACT 2  selects COM2).  The second argument is a decimal

divisor from 2 through 2304 which specifies a baudrate from 56k to 50 (the

default is 12, 9600 baud).  The baudrate arguement must be proceeded on the

command line by the COM port designation.  Again, specifying no arguments

causes a default of COM1, 9600 baud to be used.  A command line which causes

the program to use COM3 and 19.2k baud is: ACT 3 6.  A help screen is printed

if any improper argument is used or if ACT H is entered.

Programmer's Guide for ACT.C -- Rev 1.0

=======================================

Help Screen for ACT

-------------------

NOTE: The following is displayed if any incorrect arguments are supplied

to the ACT (this includes entering "act help")

HELP Screen for NS16450/INS8250A Asynchronous Communications Test (ACT)

USAGE: act [<COM-number> [<baudrate-divisor>]]

arg1              arg2

('act' may be followed by no args, arg1 or arg1+2)

ARGUMENT     MINIMUM VALUE    MAXIMUM VALUE    DEFAULT VALUE

============  ===============  ===============   ==============

COM-number    1 (for COM1)     3 (for COM3)           1

(serial line 0)  (serial line 2)       (COM1)

baudrate-   2 (56000 baud)   2304 (50 baud)         12

divisor     (based on 1.8432 MHz crystal)      (9600 baud)

*  Sample baudrate divisors: for baud= 1200, use 96  *

*                                      2400      48  *

*                                      4800      24  *

*                                      9600      12  *

*                                     19200       6  *

Page 2 of 4

Programmer's Guide for ACT

===========================

Summary of ACT.C Subroutines

-----------------------------

NOTE: As the code itself is heavily commented, it should be inspected if

further clarification is necessary.

The following subroutines are contained in this file:

void main(int, char **);		/* main program */

void interrupt far serint(void);	/* serial interrupt handler */

void procarg(int, char **);		/* process invocation arguments */

void savepar(void);			/* save our environment parameters */

void rstrpar(void);			/* restore our environment parameters */

void dspstr(UCHAR *);			/* display a string on the console */

void usage(void);			/* display usage info and exit */

1. main(): main program

The main program initializes the system as follows:

processes invocation arguments by calling procarg(),

uses the CLEAR_REGS() macro to clear the RBR, LSR, IIR and MCR registers,

saves environment parameters by calling savepar(),

sets the requested (or default) baud rate and LCR register,

prints name of test to screen

prints "Hit a key when other program has been started" and waits for keyboard hit,

enables IRQ by setting OUT2,

enables UART receiver and line status interrupts,

asserts RTS and after receiving CTS (RTS from other machine),

it prompts the operator to "Hit any non-control key to stop" the program.

It then starts full duplex communication by enabling transmitter interrupts

and then loops until either the global variable "errflag" changes from

FALSE (0) to TRUE (a specific error code), or for the operator strikes any

non-control keyboard key.  The errflag variable is set by the interrupt

handler as described for serint().

While this loop is active, the global variable "tick" is inspected.  If it is

non-zero, either "|" or "_" is displayed at one place on the screen in order

to inform the operator that the program is active.  This is facilitated by

use of a flip-flop variable to index one of two ASCII strings.  The character

is changed every 2560 bytes received.

After exiting this loop, the program waits for the TEMT bit in the LSR to be

set to indicate an empty transmitter.  The CLEAR_REGS() macro is again

invoked.  An 'End of program' message is printed and, if the interrupt

handler found an error, an error message corresponding to the code in the

variable errflag is output.

Page 3 of 4

Programmer's Guide for the ACT

===============================

Summary of ACT.C Subroutines (continued)

----------------------------------------

2. serint(): serial port interrupt handler

The interrupt handler first checks if the global "errflag" is TRUE.  If so,

the interrupt is ignored by disabling UART interrupts and by sending the

PIC an EOI and exiting.

Then the IIR is read and each interrupt type is handled:

NO Interrupt Pending:

errflag <- False Interrupt error message code

Receiver Line STatus interrupt:

read and store LSR

read and store RBR

errflag <- Status error message code

Received Data AVailable (RDAV):

clear RTS

if Data Ready bit is set

read byte and compare to expected value else errflag <-

Receiver Error code

if mismatch is found, errflag <- Mismatch Error code

else increment byte counter variable and set 'tick if

2560 characters were received

if no errors were found RTS is reasserted

Transmitter Holding Register Empty:

the LSR is read

verify THRE bit set, errflag <- Tx Error message code if not

if CTS is asserted

write next byte to transmitter and clear timeout counter

else increment timeout counter and if it is > 50 (arbitrary

large number), errflag <- End of Program code (this means

that other machine has stopped requesting more data

(ie.  CTS = 0) )

Any other value for the IIR:

errflag <- IIR Error code

All UART interrupts are disabled (IER <- 0)

If no errors were found, all interrupts are reenabled.  This creates an edge

on the INTR line if multiple interrupts are pending.  It also fixes the

full duplex bug involving simultaneous interrupts which is present in

16450 and 8250A UARTs

An EOI is sent to the PIC.

(NOTE: an IRET instruction is automatically generated to exit this int. handler)

Page 4 of 4

Programmer's Guide for ACT

===========================

Summary of ACT.C Subroutines (concluded)

----------------------------------------

3. procarg(): process invocation arguments

The invocation arguments as defined in the help screen are validated

by this routine.  The help screen itself is displayed by calling

usage() if any illegal arguments are found.

Based on the COM argument (or its default), the interrupt vector number

and UART base address are set up for global use.  The existence of

the requested COM port is verified; its non-existence causes program

termination.

4. savepar(): save our environment parameters

This routine saves the original interrupt vector, ctrl-break/ctrl-c

status and PIC mask, and then sets these up as required for FACT use.

It also saves some of the UART register values.

5. rstrpar(): restore our environment parameters

This routine is the complement for savepar(): it restores the saved

registers, interrupt vector, ctrl-break/ctrl-c status and PIC mask.

6. dspstr(): display a string of characters on the console

This routine uses the INT 10H software interrupt to display a string a

character at a time.

7. usage(): display program usage information and terminate (never return)

This routine displays the help screen and terminates the program.

#################################################################################################################################################################