Asynchronous Communication Test Software (ACTB) for INS8250/8250N-B UARTs

=========================================================================

National Semiconductor Corporation

Greg DeJager      February 8, 1989

Adapted from LBT.C -- Loopback Test Rev 1.1

Developed by:  Brian A. Berg

BERG SOFTWARE DESIGN

Summary of Files

--------------------------

ACTB.C		C source code: main program, subroutines and interrupt handler

SERIO.H		header file containing UART definitions for C code use

STDHDR.H	header file containing "standard" definitions for C code use

ACTB.EXE	executable file

Intro, Hardware Setup and Program Operation

===========================================

This test was developed to run on all IBM PC or PC compatible architectures.

System requirements include two separate machines at least one being a PC

with an INS8250 or INS8250N-B located at COM1, 2 or 3.  The other machine may

be an AT or PS/2 with an NS16450/16550 driving its COM port.  The COM ports on

the machines must be connected by the following RS-232 DTE to DTE interface:

Tx----------------------------Rx

Rx----------------------------Tx

RTS--------------------------CTS

CTS--------------------------RTS

GND--------------------------GND

The executable software to be run on the PC is titled ACTB.EXE.  If the other

machine is not a PC and contains a 16450/550, it must run the program ACT.EXE

(ACT.EXE is a similar communication program which tests 16450 UARTs on AT machines).

To test the 8250/N-B, the above hardware must be set up and after entering the

proper directories, the user must type ACTB <enter> on the PC and either ACTB

or ACT on the other machine.  The programs will prompt the user to start both

machines and then hit a key to begin the test.  This procedure is necessary to

ensure that all modem control outputs are deasserted before program start-up

so that a machine won't begin transmitting before the other is ready to receive.

Full duplex transmission begins after a key is hit on both machines and each

machine has exchanged initial RTS signals.  Proper test operation is indicated

by a '|' and '_' character being printed to the screen every 2560 characters

received.

All operations are interrupt driven.  UART line status, receiver and transmitter

interrupts are enabled.  When not servicing interrupts, the CPU sits in a

loop checking for a keyboard hit (means for user to stop test) and for errors

produced within the interrupt service routines.  A transmitter interrupt is

generated whenever the transmit holding register empties.  The CPU services

the interrupt by writing the next character to the holding register ONLY if CTS

is asserted.  If CTS is not asserted a timer variable is incremented and compared

to a large number (50).  If 50 transmit interrupts occur in succession

without CTS being asserted, the program assumes that the other machine has

stopped and the program is ended.

A receiver interrupt is generated when the receiver buffer gets a new character.

The CPU immediately deasserts RTS to stop any further transmission from the

other machine.  It then reads the byte out of the receiver buffer comparing

it to an expected value.  Data transmitted and received sequences between 00

and FFH. If a mismatch occurs, the expected and received data are displayed

and the program stops.  A line status interrupt results from an Overrun, Framing

or Parity error or Break condition.  Upon receiving the status interrupt, the

program prints the corrupted data and the line status register value and exits

the program.  Inconsistencies between the Interrupt ID and Line Status Registers

also cause error messages to be printed and the program to stop.

There is a restriction associated with program operation.  The maximum baudrate

that can be selected is 56k; however, this baudrate is limited by the processor

speed and the host system.  The COMPAQ PC compatible tested operated at a

maximum baud rate of 1800.  At higher baud rates operation is system dependent

due to the overhead involved with processing the interrupts.

The default com port is COM1 and default baudrate is 1200.  However, the user

can control these options by specifying arguments on the command line.  Arguments

must be separated by spaces.  The first argument is a selection of COM1, COM2

or COM3 (eg.  ACTB 2  selects COM2).  The second argument is a decimal

divisor from 2 through 2304 which specifies a baudrate from 56k to 50 (the

default is 96, 1200 baud).  The baudrate arguement must be proceeded on the

command line by the COM port designation.  Again, specifying no arguments

causes a default of COM1, 1200 baud to be used.  An example command line which

causes the program to use COM1 and 300 baud is: ACTB 1 384.  A help screen is

printed if any improper argument is used or if ACTB H is entered.

Programmer's Guide for ACTB.C -- Rev 1.0

========================================

Help Screen for ACTB

-------------------

NOTE: The following is displayed if any incorrect arguments are supplied

to the ACTB (this includes entering "actb help")

HELP Screen for INS8250/INS8250N-B Asynchronous Communications Test (ACTB)

USAGE: actb [<COM-number> [<baudrate-divisor>]]

arg1              arg2

('actb' may be followed by no args, arg1 or arg1+2)

ARGUMENT     MINIMUM VALUE    MAXIMUM VALUE    DEFAULT VALUE

============  ===============  ===============   ==============

COM-number    1 (for COM1)     3 (for COM3)           1

(serial line 0)  (serial line 2)       (COM1)

baudrate-   2 (56000 baud)   2304 (50 baud)         96

divisor     (based on 1.8432 MHz crystal)      (1200 baud)

*  Sample baudrate divisors: for baud= 300, use 384  *

*                                      600      192  *

*                                      1200      96  *

*                                      2400      48  *

*                                      4800      24  *

Page 2 of 4

Programmer's Guide for ACTB

===========================

Summary of ACTB.C Subroutines

-----------------------------

NOTE: As the code itself is heavily commented, it should be inspected if

further clarification is necessary.

The following subroutines are contained in this file:

void main(int, char **);		/* main program */

void interrupt far serint(void);	/* serial interrupt handler */

void procarg(int, char **);		/* process invocation arguments */

void savepar(void);			/* save our environment parameters */

void rstrpar(void);			/* restore our environment parameters */

void dspstr(UCHAR *);			/* display a string on the console */

void usage(void);			/* display usage info and exit */

1. main(): main program

The main program initializes the system as follows:

processes invocation arguments by calling procarg(),

uses the CLEAR_REGS() macro to clear the RBR, LSR, IIR and MCR registers,

saves environment parameters by calling savepar(),

sets the requested (or default) baud rate and LCR register,

prints name of test to screen

prints "Hit a key when other program has been started" and waits for keyboard hit,

enables IRQ by setting OUT2,

enables UART receiver and line status interrupts,

asserts RTS and after receiving CTS (RTS from other machine),

it prompts the operator to "Hit any non-control key to stop" the program.

It then starts full duplex communication by enabling transmitter interrupts

and then loops until either the global variable "errflag" changes from

FALSE (0) to TRUE (a specific error code), or for the operator strikes any

non-control keyboard key.  The errflag variable is set by the interrupt

handler as described for serint().

While this loop is active, the global variable "tick" is inspected.  If it is

non-zero, either "|" or "_" is displayed at one place on the screen in order

to inform the operator that the program is active.  This is facilitated by

use of a flip-flop variable to index one of two ASCII strings.  The character

is changed every 2560 bytes received.

After exiting this loop, the program waits for the TEMT bit in the LSR to be

set to indicate an empty transmitter.  The CLEAR_REGS() macro is again

invoked.  An 'End of program' message is printed and, if the interrupt

handler found an error, an error message corresponding to the code in the

variable errflag is output.

Page 3 of 4

Programmer's Guide for the ACTB

===============================

Summary of ACTB.C Subroutines (continued)

----------------------------------------

2. serint(): serial port interrupt handler

The interrupt handler first checks if the global "errflag" is TRUE.  If so,

the interrupt is ignored by disabling UART interrupts and by sending the

PIC an EOI and exiting.

Then the IIR is read and each interrupt type is handled:

NO Interrupt Pending:

errflag <- False Interrupt error message code

Receiver Line STatus interrupt:

read and store LSR

read and store RBR

errflag <- Status error message code

Received Data AVailable (RDAV):

clear RTS

if Data Ready bit is set

read byte and compare to expected value else errflag <-

Receiver Error code

if mismatch is found, errflag <- Mismatch Error code

else increment byte counter variable and set 'tick if

2560 characters were received

if no errors were found RTS is reasserted

Transmitter Holding Register Empty:

the LSR is read

verify THRE bit set, errflag <- Tx Error message code if not

if CTS is asserted

write next byte to transmitter and clear timeout counter

else

increment timeout counter and if it is > 50 (arbitrary

large number), errflag <- End of Program code (this means

that other machine has stopped requesting more data

(ie.  CTS = 0) )

Any other value for the IIR:

errflag <- IIR Error code

if THRE is asserted (ie transmit interrupt pending)

transmitter interrupt is disabled and immediately reenabled  (This fixes

full-duplex bug involving missing a transmitter interrupt)

An EOI is sent to the PIC.

(NOTE: an IRET instruction is automatically generated to exit this int. handler)

Page 4 of 4

Programmer's Guide for ACTB

===========================

Summary of ACTB.C Subroutines (concluded)

----------------------------------------

3. procarg(): process invocation arguments

The invocation arguments as defined in the help screen are validated

by this routine.  The help screen itself is displayed by calling

usage() if any illegal arguments are found.

Based on the COM argument (or its default), the interrupt vector number

and UART base address are set up for global use.  The existence of

the requested COM port is verified; its non-existence causes program

termination.

4. savepar(): save our environment parameters

This routine saves the original interrupt vector, ctrl-break/ctrl-c

status and PIC mask, and then sets these up as required for FACT use.

It also saves some of the UART register values.

5. rstrpar(): restore our environment parameters

This routine is the complement for savepar(): it restores the saved

registers, interrupt vector, ctrl-break/ctrl-c status and PIC mask.

6. dspstr(): display a string of characters on the console

This routine uses the INT 10H software interrupt to display a string a

character at a time.

7. usage(): display program usage information and terminate (never return)

This routine displays the help screen and terminates the program.

#######################################################################################################################