WHATPORT.COM

Version 2.0

October 24, 1989

For detecting installed serial ports and type of mouse

Copyright 1989, Life Sciences Editorial Services

1236 River Bay Road

Annapolis, Maryland 21401

AUTHOR: Peter Petrakis

IMPROVEMENTS IN THIS VERSION:

Version 2 of WHATPORT.COM provides a more graphic display

of installed COM ports and their associated interrupt request

(IRQ) lines.  In addition, unlike the original version, it displays

the IRQ line being used by any installed mouse (except PS/2), not

just a serial mouse, thus increasing the usefulness of the program

for avoiding IRQ line conflicts.

DESCRIPTION:

This program detects what serial ports are installed in an MS/DOS

or PC/DOS system.  The type of mouse, if installed, is also shown

(serial, bus, InPort, HP) together with the interrupt request (IRQ)

line it's using.  If it's a serial port mouse, the program indicates

whether it's on the IRQ line for COM1/COM3 or the IRQ line

for COM2/COM4, to guide a user in selecting an available COM

port for other devices when more than two serial ports are

installed.  Because PCs have only two IRQ lines for serial

communications, COM3 must share COM1's IRQ line and COM4

must share COM2's IRQ line.  Unfortunately, two devices using

the same IRQ line can't be active at the same time.  COM3 can't

be used for a printer or modem if a serial mouse is active on

COM1, for example.  Likewise, COM4 can't be used if a serial

mouse is active on COM2.  The following table shows what ports

are available for other devices when a serial port mouse is present

on a multiple-port system:

Serial port mouse on        COM ports available for other devices

--------------------        -------------------------------------

COM1 or COM3 (IRQ 4)               COM2 or COM4

COM2 or COM4 (IRQ 3)               COM1 or COM3

HOW TO USE THE PROGRAM:  Just run it.

DISTRIBUTION:

WHATPORT.COM and this document are provided free by Life

Sciences Editorial Services, Annapolis, Maryland.  They may be

copied and distributed promiscuously provided no money is

charged for them, they are not modified, and they are not

separated.  Although the program has been extensively tested,

Peter Petrakis makes no warranty concerning its usefulness on all

systems and he accepts no responsibility or liability for any mishap

resulting from its use.