Modifying Modem Programs to Operate at 450 Baud

As you are probably aware,  most 300 Baud  modems will operate at

speeds	greater  then 300 Baud.  Since source code is not  easily

available on most modem programs, below is a method to modify the

ones that do not support 450 Baud. (I haven't found any that do).

The  method  I	use is to modify one of the Baud  rates  that  is

supported to 450 Baud. I picked 1200 in both examples below, if I

had a 1200 Baud modem I would not be doing this.

Each  of the examples are done somewhat differently but the basic

procedure  remains the same.  It is necessary to know the  values

of the various Baud rates used by the hardware. They are:

9600 BAUD = 000C

4800 BAUD = 0018

2400 BAUD = 0030

1200 BAUD = 0060

600 BAUD = 00C0

450 BAUD = 0100

300 BAUD = 0180

110 BAUD = 0417

GENERAL PROCEDURE

1.  Make  a  copy of the program to modify and rename it with  an

extension other then COM, EXE, or HEX.

2. Load the program under debug

Example: A>debug xtalk.mod

3. Use	search	feature of debug to find value of  Baud  rate  to

change.

Example: -s100 f000 60 00

Note:  that the value you are looking for will be in  memory

in reverse order from the above table.

4. In most cases debug will report several places where the value

occurs	in  memory,  use the unassemble comand to  determine  the

correct  value	to  change.  If there are several that	might  be

correct my only suggestion would be to change them one at a  time

and test between changes. (Not very fool proof)

5.  After  necessary changes then use the write feature to  store

changes on the disk.

Example: -w

6. Rename the file back to the correct extension.

7. Run the file to test it and good luck.

Caution:  While in debug do not attemp to run the file.  It  will

not work and the system will get lost.

Modifing Crosstalk

1. Load file under debug

2.  Search for 60 00. There will probably  be  two areas at  B6AA

and C866.  The C866 is the correct area. If you dump the area you

will  see  all the Baud rates followed by the two bytes  for  the

hardware setup.

3. Back up four bytes to where the 1200 starts and enter 0450 and

the 0 , 1

Example: -ec863

XXXX:C863 31.30	 32.34	   30.35     30.30     60.0

00.1	 36.

Note:  The  value	to the left of the decimal point  is  the

value in memory, to the right is the value to change it to.

4. Write file back to disk.

Note:  Since it is known that two	ASCII  zeroes  are before

the hardware value, if the search string is changed to -s100 f000

30  30	60  00, then it should only report one occurance of  that

string.

Modifying Minitel

1. Load file under debug

2. Search for 60 00. There will likely	be several locations. The

area around 1700 is the correct area. If you unassemble this area

you  will see some Mov	 BX instructions with the hardware values

being loaded.

3. Change the 60 00 to 00 01.

4. Write file back to disk.

Note:  Since  it  is  now known that MOV BX  is  before  the

hardware value, if the search string is changed to -s100 f000  BB

60 00, it should only report one occurrence of that string.

Misc Notes:

1.  Crosstalk  will  show	0450  Baud  on	the  screen  when

selected.  In Minitel you  have to  select 1200 but  it will work

at 450.

2.  With both of the systems only the first character  needs

to be input when selecting a Baud rate. A 0 for Crosstalk and a 1

for Minitel.

David Paul

#he first character  needs

to be input when selecting a Baud rate. A 0 for Crosstalk and a 1

for Minitel.