11/12/88 11:46am

MULTI-TECH MT696E 9600 BAUD MODEM

=================================

This text file describes the settings required to use the MULTI-TECH MT696E

9600 baud modem for RBBS CPC 17.1B.

Roy's RBBS (312) 779-3976 uses the Multi-Tech MT696E 9600 baud modem for

fast communications.  The MT696E is a MNP level 6 modem, capable of data

compression, which allows upload/download speeds in excess of 1200 cps using

ZMODEM.

At present, it is only compatible with other MT696E's and the MICROCOM 9600

baud modem with MNP level 6 error correction.

===============================================================================

RBBS SETTINGS:

The following screens show how I have RBBS CPC 17.1B setup:

RBBS-PC (Version CPC17-1B) Configuration  Page 12 of 17

221. Communications port to be used by RBBS-PC ---- COM1

222. # of seconds to wait for modem to initialize - 3

223. Seconds to wait before issuing modem commands- 2

224. Number of rings to wait before answering ----- 1

225. Use the RBBS-PC default Hayes commands? ------ NO

226. Is MICROCOM's MNP protocol available? -------- YES

227. Issue modem commands between rings ----------- NO

228. Baud rate to initially open modem at --------- 9600

229. Log off user who are idle for ---------------- 180 seconds

230. Are you using a 'DUMB' auto-answer modem? ---- NO

231. Initialize Hayes 2400 firmware for RBBS-PC.

232. # seconds to wait after dropping DTR --------- 3

233. File with PROTOCOL definitions --------------- E:PROTO.DEF

234. Always check caller for AUTODOWNLOAD support - NO

235. Require non-ascii protocol for BASIC files --- YES

236. Recycle if no calls are received within ------ 30 minutes

237. Leave modem at initial baud ------------------ YES

PARAMETER 225:

RBBS-PC to use modem commands you specified as follows:

1. Reset the modem                : ATZ

2. Initialize the modem           : ATM0Q1S2=255S10=30E0Q0X1S30=255&D3S0=254

Note: End item 2 with:

S0=1Q0X1 if answer on 0 rings

S0=254 if answer on >0 rings (no ring-back)

S0=255 if answer on >0 rings (with ring-back)

3. Count the number of rings      : ATS1?

4. Answer the phone               : ATQ0X1V1A

5. Take the phone off the hook    : ATQ1E1H1M0

6. Clear the modem's firmware     : AT&F

7. Initialize modem's firmware    : AT&C1&D3B1E0V1M0S0=0&T5

Note: End item 7 with:

Q1 if item 2 ends with S0=255

8. Write to modem's firmware      : &W

Specify command to change (1 to 8), CR to end, or 0 to reset to defaults)

RBBS-PC (Version CPC17-1B) Configuration  Page 13 of 17

241. Restore initial parms. after change to N/8/1 - NO

242. Minimum baud required of new callers --------- 0

243. Minimum baud required of old callers --------- 0

244. Modem flow control uses Clear-to-Send (CTS)--- NO

245. Modem flow control uses XON/XOFF ------------- YES

246. Seconds to wait for carrier after answering -- 30

===============================================================================

MT696E SETTINGS:

There are three (3) sets of settings required for the MT696E,  1) an 8 position

switch; 2) a 4 position switch and 3) internal jumpers.  The following details

these settings:

8 Position Switch:

#1 - UP    -- DTR dependent on interface

#2 - UP    -- Verbose responses

#3 - DOWN  -- Enable command responses

#4 - DOWN  -- Disable command character echo

#5 - DOWN  -- Disable automatic answer

#6 - UP    -- CD & DSR function normally, Pins 6 & 8

#7 - UP    -- Single line setting for RJ11

#8 - DOWN  -- Enable command mode

4 Position Switch:

#1 - DOWN  -- CTS dependent on interface

#2 - UP    -- Dial-up operation

#3 - DOWN  -- Multi-Tech responses

#4 - DOWN  -- Asynchronous operation

Internal Jumper Settings:

RTS Forced ON  -- Standard factory setting

CTS Follows RTS  -- Optional setting

DSR Follows CD -- Normal Factory setting

Following is a listing of the software settings as shown by the commands

AT L5 L6 L7:

B0 E0 M1 Q0 R0 V1 X4 &E1 &E4 &E6 &E8 &E10 &E13 &E15

$MB9600 $SB9600 $BA0 &W1

S0   S2   S3   S4   S5   S6   S7   S8   S9   S10  S11  S12  S24  S25  S30

000  043  013  010  008  002  255  002  006  007  060  050  020  000  000

$A0 &A0 &B0 &BS1 &C1 $D0 &D2 $F1 &G0 &M0 &P0 &Q0 $R0 &R1 &S1 $EP1

$MB9600 $SB9600 $BA0 &W1

NOTE: The important settings are $MB, $SB and $BA0

===============================================================================

TELIX 3.11 SETTINGS

I use TELIX 3.11 to communicate with my board and found that the proper

settings are as follows:

�͵ Terminal options ���������������������������������������������������͸

�                                                                        �

� A - Baud 19200   Parity None    Data length 8    Stop bits 1           �

� B - COM1                                                               �

�                                                                        �

� C - Default terminal type ........... ANSI-BBS                         �

� D - Status line ..................... Bottom                           �

� E - Local echo ...................... Off                              �

� F - Add Line Feeds after CRs ........ Off                              �

� G - Strip high bit (incoming data) .. Off                              �

� H - Received Backspace destructive .. On                               �

� I - XON/XOFF software flow control .. Off                              �

� J - CTS/RTS hardware flow control ... On                               �

� K - DSR/DTR hardware flow control ... Off                              �

� L - Compuserve Quick B transfers .... On                               �

� M - Zmodem auto-downloads ........... On                               �

� N - Answerback string (ENQ) .........                                  �

�                                                                        �

�                                                                        �

� Change which setting?       (Return or Esc to exit)                    �

������������������������������������������������������������������������;

�͵ Modem and dialing parameter setup ��������������������������������������͸.

�                                                                            �

� A - Init string ......... AT M1 &Q0E0 S7=255 S2=43 S0=0 V1 X4 Q0 S11=45 ^M �

� B - Dialing prefix 1 .... ATDT                                             �

� C - Dialing prefix 2 .... ATDT                                             �

� D - Dialing prefix 3 .... ATDT                                             �

� E - Dialing suffix ...... ^M                                               �

� F - Connect string ...... CONNECT                                          �

� G - No connect strings .. NO CARRIER             BUSY                      �

�                           VOICE                  NO DIAL TONE              �

� H - Hang-up string ...... ~+++~ATH0M1^M                                    �

� I - Auto answer string .. ~~~+++~~~ATS0=1^M                                �

� J - Dial cancel string .. ^M                                               �

�                                                                            �

� K - Dial time ........... 60                                               �

� L - Redial pause ........ 4                                                �

� M - Auto baud detect .... Off                                              �

� N - Drop DTR to hangup .. On                                               �

�                                                                            �

�                                                                            �

� Change which setting?       (Return or Esc to exit)                        �

����������������������������������������������������������������������������;

===============================================================================

Please refer any questions to:

ROY HERMAN

ROY'S RBBS (312) 779-3916  EVENINGS/WEEKENDS ONLY

VOICE -- OFFICE (800) 532-8488 OR (312) 239-7700

HOME (312) 957-2767