Reprinted Article From The Oct 17, 1989 PC Magazine Advisor Column

By: Lori Grunin                 Title: Beyond COM2

LETTER:

I've  been stumped by IBM's design limit for serial  ports.   I'd

like  to add a COM3 and COM4 beyond the two already installed  in

my 386 clone.  I put the new board in and properly set the  jump-

ers,  but when I attempted to set the  communications  parameters

via DOS's MODE command, I got "illegal device" error messages.

I've looked for books to tell me how to get around this  problem,

but to no avail.  Is there a software driver that I must  install

to effect valid operation of these ports?

Thomas Emery

Goleta, California

RESPONSE:

IBM's original PC architecture supported only two COM ports,  and

although compatible makers, and eventually IBM, added more serial

capability, there are still no standard addresses or IRQ  (Inter-

rupt Request) lines for the additional ports.

The addresses for COM1 and COM2 are 3F8h and 2F8h,  respectively,

with COM1 on IRQ4 and COM2 on IRQ3.  When you install more serial

ports, COM3 usually uses 3E8h and IRQ4, while COM4 uses 2E8h  and

IRQ3.   Unfortunately, some I/O card makers like to  blaze  their

own  trails; IBM even added uncommon COM port addresses and  IRQs

to the long list of PS/2 eccentricities.

The  DOS MODE command doesn't work with serial ports  above  COM2

because  DOS gets its I/O information from the BIOS, which  finds

out  what's  installed where in your system during  the  Power-On

Self-Test  (POST).   The POST checks only for the first  two  in-

stalled ports.

To  get  around this problem, communications  software  and  some

serial  peripherals  (such  as mice)  will  occasionally  support

higher COM ports by addressing them directly, rather than  making

DOS function calls.  Of course, if your peripherals don't support

these  extra  ports or you need to redirect data using  the  MODE

command, that's where trouble arises.

There  are a couple of public-domain utilities that allow you  to

append  your COM port information to the BIOS, making  the  ports

DOS-accessible.   COMSET (COMSET.ARC) and Port  Finder  (PF2.ARC)

are available in the "general hardware" data library of the IBMHW

forum  on Compuserve; Port Finder can also be obtained  from  its

author,  mcTRONic Systems (7426 Cornwall Bridge Ln., Houston,  TX

77041;(713)462-7687).

Both  packages activate COM3 and COM4 (if they're  installed,  of

course),  and  require DOS 3.3 or later.  They basically  do  two

things: activate the extra ports by giving the BIOS the  address-

es,  and provide utilities for swapping the addresses  among  the

different  ports.   Address-swapping allows programs  that  don't

support  COM3  and COM4 to access them.  Software  that  directly

addresses the ports will usually be unaffected.

Keep  in mind, however, that you probably won't be able  to  swap

two ports that share an IRQ (such as COM1 and COM3) if you've got

a  mouse  attached to one of them, because the  system  will  get

confused when the interrupt triggers.

Port Finder is the more flexible of the two.  In addition to  the

aforementioned  features, it provides the same  capabilities  for

LPT3,  including swapping among printer ports.   Furthermore,  it

lets you easily restore all your original settings.

Even if you think you've correctly configured the board, it can't

hurt  to  double check a couple of things.  If you've got  a  bus

mouse,  for instance, make sure that you're not stepping  on  its

IRQ; although interrupt sharing is one of the touted features  of

the PS/2, PCs can't do it very well.

Also, don't leave any port holes-if you've got three, they should

be COM1,COM2, and COM3, not COM1,COM2, and COM4. [END]

Excerpt from July 1990 issue of "Vulcan's Computer Monthly"

10 Indispensable Online Utilities

Commentary by Ron Albright

Heaven  knows  that  the communication  voyager  has  enough

problems  to deal with to find the jewels that  await  successful

connections.   Incompatible  baud speeds, word length,  and  stop

bits,  noisy  phone lines, busy signals, and  other  frustrations

stand  in our way almost at every turn.  We battle  on,  however,

knowing  that once we succeed, a wealth of information and  soft-

ware awaits.

Fortunately, there are several tools that every good  commu-

nications  should  be aware of and keep in  their  armamentarium.

This  article  will  examine 10 of these gems.   Each  handles  a

specific problem the electronic journeyman faces, and  alleviates

it to a large degree.  Here are my picks.

PORT FINDER ...

As you are probably well aware, setting up a PC for communi-

cations is not a walk in the park.  The first battle you face  is

getting  a modem to coexist with your current hardware.   If  you

are  like me, and have more hardware hanging from  your  computer

than  you can manage, you can run into problems right  there.   I

for  example,  currently (it changes from week to  week)  have  a

Logitech  ScanMan  Plus and Trackman connected  as  Bus-  devices

(their own cards rather than using my serial ports), a voice mail

board,  and an internal modem configured as COM3.  Since  the  PC

handles  any hardware device through an "interrupt" ("IRQ"),  you

can quickly run into conflicts that immobilize devices.  You  end

up  doing  the "jumper shuffle," changing those  little  jumpers,

replacing  the  boards, re-booting, and  crossing  your  fingers-

hoping to find a montage that works.

Well  with  "Port Finder" (mcTRONic Systems,  7426  Cornwall

Bridge  Ln.,  Houston,  TX   77041;  (713)  462-7687;  Shareware,

$13.95),  at  least some of the guesswork  is  eliminated.   PORT

FINDER  is  a  very versatile program.  It   will   display  your

computer    type, BIOS date, DOS version, display adapter   type,

total  base   memory , extended memory, expanded memory  and  EMS

version.    It will  detect  and  display  COM1 to COM4 and  LPT1

to  LPT3  ports  that are  responding  with  the actual  HEX  ad-

dress.   Because  PORT  FINDER displays  the  HEX port  addresses

you  can also use PORT  FINDER  as  a diagnostic tool.   You  can

swap   COM or LPT ports addresses instead  of  swapping   cables.

On  AT systems Port Finder will check to see  if  a game port  is

present and if so it will be added to the equipment list.

If   you are using DOS 3.3 or higher  and have COM3   and/or

COM4, PORT FINDER will  activate  both  COM3  and/or  COM4.   DOS

will then recognize these ports and you will be able to  use  the

DOS  MODE command.  Most system information type programs such as

the  Norton  Utilities  System  Information   (SI)  program  will

correctly   indicate  the number  of serial ports that  you  have

including  the  COM3   and/or  COM4 ports   after  you  run  PORT

FINDER.   Any other  programs that   operate through  DOS  should

also  accept  COM3 and/or  COM4 after you run  PORT  FINDER   (if

the  program does not allow you to enter COM3 or COM4   then  you

will need to swap COM3 or COM4 with COM1 or COM2 before  you  run

such  a program then select either COM1 or COM2 in said program).

If  you   have  a new IBM PS/2 computer COM3  and/or   COM4  will

already   be activated  .  You can use PORT FINDER to reset  both

COM  and   LPT   port  addresses  in    case  you  forget    what

addresses  have  been  swapped.

PORT FINDER can also be used in a BATCH file to detect which

COM  ports are  present. An  Errorlevel is  set for all  possible

combinations.   An example batch file COMMTEST.BAT  is  included.

Port  Finder will work under Microsoft Windows and a .PIF in  now

included  with Port Finder.  Due to the fact that Desqview  moves

the  COM and LPT port addresses to a nonstandard  location,  Port

Finder will not be able to properly work when Desqview is loaded.

For those, like me, who find themselves swapping hardware in

and  out  of a machine with great regularity, Port Finder  is  an

indispensable  tool  for managing your ports.   I  couldn't  live

without it.

PKZip ...                       TELEDISK ...

PKware, Inc.                    Sydex

7545 N. Port Washington Rd.     153 North Murphy Ave.

Glendale, WI  53217             Sunnyvale, CA  94086

414-352-3670                    408-739-4866

Shareware, $47                  Shareware, $25

LIST ...                        MSGVU ...

Vernon Buerg                    Dave Briccetti and Associates

139 White Oak Circle            P.O. Box 1713

Pateluma, CA  94952             Lafayette, CA  94549

$29.95                          Shareware, $30

VDE ...                         Elsewhere ...

Eric Meyer                      INTERLINK TECHNOLOGIES

401 12th Ave SE - # 139         1222 5th Street

Norman, OK  73071               Sarasota, FL  34236

405-329-5777                    Updates $20

Site Licenses $50

ZMODEM ...                      Checkup ...

Omen Technology Incorporated's  Levine & Associates

17505-V                         9405 Bustleton Ave.

Northwest Sauvie Island Road    P.O. Box 14546

Portland,  Oregon  97231        Phila., PA  19115

Shareware, $20                  215-333-8274

$24.94

Procomm 4.2 ...

DataStorm Technologies

P.O. Box 1471

Columbia, MO  65205

Reprinted From PC Computing January 1991, Prime Time Section

Going Fishing For Shareware? Pack an Online Tackle Box

by: Wendy Taylor

. . .

Some communications programs let you use only  COM1 or 2, Port

Finder, a utility from  mcTRONic Systems  ($13.95;  7426 Cornwall

Bridge Lane,  Houston,  Texas;  713-462-7687)  lets your software

address   COM3 and 4   as well.    It  saves  you  the  hassle of

reconfiguring your software or swapping cables.

. . .

Reprinted From The Power User's Tool Kit 1.0 - August 1990

MicroWAREHOUSE Presents The Power Users's Tool Kit 1.0

Compiled by: Rick Ayre

. . .

PORT FINDER

James McDaniel's  Port Finder is one of my favorite Shareware pro-

grams.  Couple  my love for  communications  with this easy to use

program and its ability to solve port conflicts on  almost any PC,

and you can see why.  This is one of those  programs that was ori-

ginally written  because the  author  needed a tool,  and couldn't

find it.  Today, I'm happy for all the frustration  that drove him

to create Port Finder.  In this recent release, it not only sniffs

out ports, both serial and parallel, but allows the initialization

of com 3 and 4 if available.  It will also let you  swap any legal

ports.  This version comes with a device driver that can be loaded

into your config.sys file to configure your ports as you like.

. . .

Reprinted From The CPC Newsletter - New Canaan, CT - January 1991

The PC Chronicles

by: Edward L. Kivell

. . .

One of the most useful shareware programs that I've run across

in quite some time is Port Finder.    This is a Comm (serial) and

LPT (parallel) port utility program.   It will activate Comm 3 or

4 on  those computers  with those  ports and  DOS 3.3  or higher.

Additionally,  it allows all possible Comm to Comm and LPT to LPT

port swaps.  The program works flawlessly.  Through this program,

I have been  able to activate Comm 3 on my PC and assign my fax /

modem to that port.  By placing a single command in my "AUTOEXEC.

BAT" file,  Comm 3  is turned  on without  my worrying  about it.

Rather than pull  and switch serial cables because several serial

devices have  to share  Comm 1 or 2,  you can use  Port Finder to

swap  the  ports.  The shareware fee  is only $13.95 so you don't

want to be without this utility.

. . .

*****************************************************************

NOTICE  NOTICE  NOTICE  NOTICE  NOTICE  NOTICE  NOTICE  NOTICE

*****************************************************************

All  Port Finder  articles are  unmodified except for their price

reference, which will be the  latest price including  shipping in

the USA.    The prices  were changed  to avoid  confusion.   Most

articles included information about other programs which has been

left out here.  Only that portion about Port Finder is included.