USER'S INSTALLATION MANUAL

FOR

"QTERM"

(QUICK TERMINAL)

POP UP COMPUTER COMMUNICATIONS PROGRAM

REVISION A.1 MAY 1988

Any comments would be appreciated:

Compuserve 76276,61

or

Ken Partridge

325 Aliento

Rancho Santa Margarita, Ca.

92688

TABLE OF CONTENTS                                           PAGE

SECTION 1 - INTRODUCTION

1-A  DESCRIPTION  . . . . . . . . . . . . . . . . . . .    1

1-B  SYSTEM REQUIREMENTS  . . . . . . . . . . . . . . .    1

SECTION 2 - INSTALLATION AND OPERATION

2-A  INSTALLING QTERM . . . . . . . . . . . . . . . . .    2

2-B  OPERATION  . . . . . . . . . . . . . . . . . . . .    3

SECTION 3 - CHANGING THE PROGRAM

3-A  COM PORT PARAMETERS/MODIFYING PARAMETERS . . . . .    4

SECTION 4 - APPENDICES

4-A COPYING THE QTERM PROGRAM  . . . . . . . . . . . . .   6

4-B TROUBLE SHOOTING . . . . . . . . . . . . . . . . . .   6

4-C NONRESIDENT TERMINAL PROGRAM . . . . . . . . . . . .   7

SECTION 1     INTRODUCTION

1-A DESCRIPTION

"QTERM" (Quick terminal) is a memory resident program for IBM PCs

or compatibles which can be used to communicate with any Serial device

without exiting from the current applications program.

QTERM is activated by a re-definable ACTIVATE KEY which halts

execution of your current program such as LOTUS 123, WORDPERFECT,

etc. With QTERM activated your computer is now in the Terminal

Emulation Mode which will send and receive data through its COM1

or COM2 serial port. Pressing a re-definable EXIT KEY will exit the

QTERM program and restore the screen of your applications program.

The purpose of this program is to allow easy connection and

control of any serial device without exiting your current

application program.

1-B SYSTEM REQUIREMENTS

You will need:

-> IBM PC,PC/XT, PC/AT, PS/2 or a true compatible

-> DOS Version 2.0 or later

-> IBM monochrome, IBM color, IBM enhanced, MCGA,

VGA or equivalent display

-> QTERM program diskette

-> COM1 or COM2

SECTION 2 INSTALLING QTERM

2-A INSTALLING QTERM

The COM port parameters of the program are preset for the

following:

COM1, 9600 baud, no parity, 8 data bits, 1 stop bit, half duplex,

no linefeed added with carriage return. The ACTIVATE KEY is set

for <ALT> E and the EXIT KEY is set for ESC (escape). To change

any of these parameters refer to section 3.

To install the program, insert the supplied program diskette into

the current disk drive (or if stored on a hard drive bring up the

corresponding directory) and at the DOS prompt, enter:

QTERMS

You may now load your applications programs.

NOTE: QTERM is now in the computer until you turn the power off.

QTERM only needs to be loaded once; while your computer is on.

QTERM stays in memory even if you quit one applications program

for another.

If you are installing the program using the AUTOEXEC.BAT file,

insert the following line into the batch file:

QTERMS

NOTE: ALTHOUGH SOME MEMORY RESIDENT PROGRAMS ARE NOT COMPATIBLE

WITH EACH OTHER, QTERM IS 99% COMPATIBLE WITH OTHER MEMORY

RESIDENT PROGRAMS.

If this is the first time you have installed the program or you

have made changes to your setup parameters, it is recommended

that you activate QTERM (Press your ACTIVATE KEY) and communicate

with the device connected to the COM port before you load your

applications program. Doing this insures that you have loaded the

proper setup parameters and that the cabling and hardware are

working.

2-B OPERATION

The QTERM memory resident terminal emulation program resides in

the high memory of your PC's system RAM while your applications

program resides in lower memory. The program can be activated at

anytime (provided it is loaded into memory) by the ACTIVATE KEY

and deactivated by the EXIT KEY. An <alt> E is used as the

default value for the ACTIVATE KEY and ESC (escape) for the EXIT

KEY.

Each keystroke from the keyboard is checked for a match with the

ACTIVATE KEY value. If it is not the proper value, the

applications program being run goes about its normal routine. If

the ACTIVATE KEY is pressed, the applications program is then

paused, its current screen saved, the monitor is cleared and the

serial communications port is opened with the specified parameter

settings.

The PC is now in a terminal emulation mode which can send and

receive serial data through the COM port.

To go back to your applications program, simply press the EXIT

KEY. Your original screen will be restored and you will now be

back in your program. The QTERM program will not pop up in most

high resolution graphics screen pictures. You must first go to a

normal low resolution screen, pop up QTERM then go back to the

graphics screen.

SECTION 3 CHANGING PARAMETERS

3-A COM PORT PARAMETERS

Parameters required by the program to select the desired COM

port, parity, databits, etc. are selected at the DOS prompt when

QTERM is first loaded. The complete syntax is

QTERM B=nn L D Cn Px H=x E=n

NOTE: ONLY THOSE PARAMETERS THAT NEED TO BE CHANGED NEED TO BE

ENTERED OTHERWISE THE DEFAULT VALUE WILL BE SELECTED.

WHERE

C1 = COM1

C2 = COM2

* DEFAULT = COM1

B=19200 = 19.2K BAUD

B=9600  = 9600 BAUD

B=4800  = 4800 BAUD

B=2400  = 2400 BAUD

B=1200  = 1200 BAUD

B=300   = 300 BAUD

* DEFAULT = 9600 BAUD

PE = EVEN PARITY, 7 DATA BITS

PO = ODD PARITY, 7 DATA BITS

PN = NO PARITY, 8 DATA BITS

* DEFAULT = NO PARITY

L = ADD A LINEFEED WITH EACH RECEIVED

CARRIAGE RETURN

* DEFAULT = NO EXTRA LINEFEED

D = CHANGE TO HALF DUPLEX  (ECHO)

* DEFAULT = FULL DUPLEX (NO ECHO)

H= ANY LETTER FROM A - Z WILL CHOOSE THAT LETTER

AND THE ALT KEY AS THE HOT KEY

* DEFAULT = <ALT>E  AS THE HOT KEY

E=1  CHOOSES <CTRL>G  AS THE EXIT KEY

E=2  CHOOSES <CTRL>A  AS THE EXIT KEY

E=3  CHOOSES ANY EXTENDED KEY (FUNCTION, ALT, ETC)

AS THE EXIT KEY.

* DEFAULT = <ESC>  AS THE EXIT KEY.

For example, if you wanted all the default values, at the DOS

prompt, enter:

QTERMS

If you wanted an added linefeed and 19.2k baud enter:

QTERMS L B=19200

Or if you wanted to use COM2, even parity at 300 baud with the

HOT KEY as <ALT> Q and the EXIT KEY as <CTRL>A enter:

QTERMS C2 PE B=300 H=Q E=2

APPENDICES

APPENDIX-A COPYING THE QTERM PROGRAM

To copy the QTERM program, insert the QTERM program into disk

drive A. Insert a disk into disk drive B or ensure the hard disk

(C) is in the right directory. Change the default disk drive to

either B or C (to do this enter a B: or C: at any DOS prompt).

Enter:

COPY A:*.*

All the files from disk drive A should be in a parent directory

on disk drive B or C.

To copy the QTERM program, and you do want its own directory,

insert the QTERM program into disk drive A. Change the default

disk drive to either B or C and ensure that the

default disk drive is in the right directory.

Enter:

MD QTPROG

Now change to that directory by entering:

CD QTPROG

Now enter the copy command:

COPY A:*.*

All the files from disk drive A should be in their own directory

on drive B or C.

APPENDIX-B TROUBLE SHOOTING

Why can't I get the communications screen to come up ?

1. Did you remember to load QTERM ?

2. Are you sure you are using the right ACTIVATE KEY ?

Why can't I communicate with my switch ?

1. Are the program and the SWITCH coordinated (ie. matching COM

port, baud rate, etc.) ?

2. Are you using the right log on codes to your switch ?

3. Did you remember to disconnect from your switch if you had

connected previously ?

When I am communicating with the switch and enter a code to do a

command, why do I get switched back to my applications program

that I originally popped out of.

1. You should change the EXIT KEY for the QTERM program

because your device must be using the same key to do a

function.

Why can't I get out of the terminal emulation screen and back to

my application program ?

1. Are you sure that you entered the correct key for your

EXIT code ?

Why does the message "PROGRAM TO BIG TO FIT IN MEMORY" come up ?

1. With QTERM loaded there apparently is not enough room for all

the programs. Use the NON RESIDENT program (TERMCOM) to set up

your device. QTERM only uses 9k of memory.

APPENDIX-C NON RESIDENT TERMINAL PROGRAM

There is a simple terminal program on the QTERM disk that is not

memory resident. You may use it to see if you have connected the

SWITCH to your PC correctly or if the memory resident program is

to large to work with your applications program. The program is

set to COM1, 9600 baud, no parity, 8 databits, no extra linefeed,

half duplex, with an <ESC> as the exit key.

To load this program  at any DOS prompt type:

TERMCOM

The parameters for the non-resident terminal program are entered

the same way the QTERM parameters are, through the command line.

Valid parameters are:

B=19200  = 19.2K BAUD

B=9600   = 9600 BAUD

B=4800   = 4800 BAUD

B=2400   = 2400 BAUD

B=1200   = 1200 BAUD

B=300    = 300 BAUD

C1       = COM1

C2       = COM2

PE       = EVEN PARITY, 7 DATA BITS

PO       = ODD PARITY, 7 DATA BITS

PN       = NO PARITY, 8 DATA BITS

L        = ADDED LINE FEED WITH CARRIAGE

RETURN

D        = HALF DUPLEX

E        = <CTRL>A   AS THE EXIT KEY

There is one more program on the diskette called TELL.COM since DOS

really had no way of telling you the current parameters of a COM port

this program goes in and JUST reads the serial port parameters (and also

tells you a few extras) to read COM 2 enter the program name then C2 after

it.

AUTHORS NOTE:

Although this program has no uploading or downloading features, it does

possess a rock solid design, has not crashed, runs with other TSRs (yes

even sidekick) and only takes up about 8k of data. I hope these programs

are helpful to someone and I would love to have feed back.

thanks,

KEN PARTRIDGE