SPM.DOC    Copyright (c) 1991 Information Technology, Ltd.

What is LiteSPM?

---------------

LiteSPM(tm), or SPM for short, is a background(TSR) Serial Port

Monitor. Once started, it sits quietly in the background, and

listens to one of the system's serial ports. SPM writes the

information that it receives to a disk file at either intervals

you choose, or at a predefined, by SPM, point, whichever comes

first.

Why Use LiteSPM?

----------------

We originally designed LiteSPM for use with the newer generation

of telephone systems. Many telephone systems in use today have a

provision for periodically sending data to a serial (RS232)

device. Frequently, a standard printer records this information.

At some later time, the printed information will be entered into

a computer for the purposes of analysis and billing.

With SPM you can bypass the data entry step. SPM records the

information directly on your disk, but doesn't tie up your

system. The result is a saving in both time and money.

SPM has other uses. Since SPM will record anything, it can

monitor and record any type of serial information. The

information does not have to be ASCII characters. And SPM does

not interpret or alter the data it receives in any way. It can

monitor and record information from laboratory systems,

manufacturing equipment, scales, or anything, in fact, that can

produce serial (RS232) output.

IMPORTANT NOTE:

----------------

The above does not apply to modems. In general, modems require

special initialization sequences to function properly. Modem

support MAY be added in a future version of SPM.

What Is Required?

-----------------

SPM requires about 24K of free memory and 1 serial port and at

least MS-DOS 2.1, nothing more, nothing less. If you are

fortunate and have MS-DOS 5.XX, you may load SPM into upper

memory. See your MS-DOS documentation for details on the LOADHIGH

command.

SPM is flexible as well. You can configure SPM using command line

switches, selecting the serial port, baud rate, parity, end of

line characters, and path name of the file that SPM should use.

Once SPM is running, you can instruct SPM to rename its file so

that you can process it. Of course, SPM can be unloaded on

command.

Installation Instructions

Page 1

SPM.DOC    Copyright (c) 1991 Information Technology, Ltd.

-------------------------

You may install SPM by copying the file SPM.EXE into any

directory on your system's disk. We recommend that you place SPM

in a directory that is usually on the DOS path, but that is not a

requirement, it will just make your life somewhat easier. Once

you have copied SPM.EXE, you are ready to go.

Starting SPM

------------

To use SPM, you may simply type

SPM

to start SPM with its default parameters. Normally, however, you

will want to change one or more of these parameters. The command

line switches to do this are described below.

The more general form of the SPM command is

SPM -cn -bnnnn -pX -eXXX -hX -fXXXXXXXXXXXXXXX | -u

where:

-a      turns on the optional alert signal. As each log

record is written, a tone is sounded. Once activated,

the alert signal can only be deactivated by unloading

and reloading SPM.

-cn     specifies the COM port SPM is to monitor, values

of n may range from 1 to 4. If you intend to use COM3

or COM4, please see the information about these ports

below.  The default value is 1.

-bnnnn  specifies the baud rate SPM is to use. The rate

may range from 300 to 9600. The default value is 2400.

-pX     specifies the parity of the data that SPM will

monitor. Values of X may be E - even parity, O - odd

parity, or N - no parity.  The default value is N.

-eXXX   defines the character string that SPM is recognize

as the end of a record. By default, SPM recognizes

CR-LF as the end of record. To specify a new end of

record string, use the -e switch followed by up to 6

characters enclosed in quotes ("). Control characters

are specified by using the common notation ^ followed

by the ASCII character to be used.  To specify a CR,

for example, you would use ^M. Control sequences like

^M count as a single character.

Page 2

SPM.DOC    Copyright (c) 1991 Information Technology, Ltd.

-hX     specifies the type of handshaking that SPM is to

use. By default, there is no handshaking. If you wish

to use XON-XOFF handshaking, specify X. To use DSR/DTR

handshaking specify D. To use RTS/CTS handshaking

specify R. Note that to use handshaking, the device

that SPM is monitoring must support handshaking.

-fXXXXXXXXXXXX defines the full path name of the file

into which SPM will place the information it captures.

By default, the name of the file is SPM.LOG. SPM.LOG

will be placed in the default directory in use when

SPM was started. We STRONGLY recommend that you

specify a full path name to avoid problems.

-u Unload SPM from memory. As with any TSR, SPM will only

unload successfully if it is the last TSR in memory.

Examples

--------

SPM -c1 -b1200

This command starts SPM on COM1 using 1200 baud. All other

parameters are defaulted.

SPM -c3

This command starts SPM on COM3, with all parameters defaulted.

If you intend to use COM3 or COM4, please review the section on

COM3 and COM4.

SPM -c1 -fd:\logfiles\spm.log

In this example, SPM will monitor COM1 at 2400 baud. The data

that SPM receives will be placed in SPM.LOG in the logfiles

directory on drive D.

SPM -c1 -e"^M"

Start SPM on COM1. The end of line string is Ctrl-M (or Carriage

Return).

SPM -c2 -hr -f\logfiles\spm0630.log

Here,  SPM is started on COM2.  The device that SPM is monitoring

recognizes RTS/CTS hardware flow control.  All data is placed in

the file spm0630.log in the logfiles directory of the current

drive.

Page 3

SPM.DOC    Copyright (c) 1991 Information Technology, Ltd.

SPM -fC:\logfiles\spm0630.old

With SPM already running, the -f argument renames the existing

logfile to SPM0630.old. We specify the full path name in case the

logfile is in another working directory or on another drive. The

next time SPM needs to write data, a new log file with the

original path name will be created.

Page 4

SPM.DOC    Copyright (c) 1991 Information Technology, Ltd.

SPM -u

Unload SPM when it is safe to do so. Flush any unwritten data to

the log file.

Using COM3 and COM4

-------------------

SPM can be used with COM3 and COM4.  To do so successfully, your

serial ports must be setup so that the base port address of COM3

is 3e8H (2e8H for COM4); In addition, the COM3 interrupt must be

IRQ4 (IRQ3 for COM4). If your serial ports can be set to these

values, then you should have no difficulty using COM3 or COM4.

Tips and Techniques

-------------------

In general, the loading of SPM is best done in your system's

autoexec.bat file. If this is not convenient or not sufficiently

flexible, you may use another method to load SPM.

If the data you are attempting to capture with SPM is

identifiable as discreet records, we STRONGLY urge you to use the

-e switch to define the characters that identify the end of the

record. While SPM will never loose information, including the -e

switch helps insure that SPM will place discreet records in the

log file. Put another way, when you use the -e switch correctly,

the log file should never contain partial records.

SPM can survive in a network environment. In a network

environment, you may specify that the SPM log file be placed on a

network drive.

DO NOT attempt to directly process the current SPM log file. We

have included the rename facility so that you can keep SPM

active, yet access the data in a convenient fashion.  Of course,

the simplest way to accomplish this is through the use of a .bat

file. Typically, the bat file would instruct SPM to rename the

log file, process the data, then delete the file.

DO specify the complete path name of the SPM log file when using

the -f pathname switch. This is particularly important when you

use the -f switch to rename the current log file.

DO NOT delete the current SPM log file while SPM is active. While

you might do so successfully, on occasion, there is no way to

insure that SPM will not attempt to write to the file while you

are deleting it. If this should happen, the results are

unpredictable. At best there would be some data loss. At worst, a

system hang might result.

Page 5

SPM.DOC    Copyright (c) 1991 Information Technology, Ltd.

DO pay attention to the hardware flow control signal restrictions

when you select either DTR/DSR or RTS/CTS flow control. LiteSPM

assumes that these signals are active HIGH signals. Check your

hardware reference carefully if you have difficulty that seems to

be related to hardware flow control.

Serial port wiring can be one of the most difficult areas to

understand, since some equipment manufacturers have chosen to

bend the standards. If you plan to do a lot of work in this area,

we recommend that you acquire a good quality breakout box to help

you make the right connections. A breakout box will let you

monitor and experiment with the various RS232 signals quickly and

conveniently without spending time soldering unknown connections.

One such device that we use is the Blue Box 100, manufactured by

International Data Sciences, 7 Wellington Road, Lincoln, RI

02865. You can contact them at (401) 333-6200 for current pricing

and availability.

Problems And Questions

------------------------

If you are having difficulty using LiteSPM, and you are a

registered user, please feel free to contact us by telephone. If

you want to register LiteSPM, see the file REGISTER.DOC for

additional information. We will also respond to Electronic Mail

on Compuserve (ID 70166,1152) or GEnie (ID I.TECH). Finally, we

will be happy to respond to mail inquiries by conventional

first-class mail. Regrettably, we are not equipped for FAX

service at this time.

And if you have special requirements for SPM, please feel free to

contact us as well.

Page 6