UARTCHK  --  detect UARTs installed in the computer and display their

type (8250, 16450, 16650, or 16650A)

Usage:  uartchk [address...]

Description:

By itself (nothing else on the command line), UARTCHK reports on

the I-O addresses usually associated with COM ports 1 through 4; the

output shows the port number, along with the I-O address.

With one or more I-O addresses on the command line, UARTCHK reports

on those addresses; use this for checking UARTs assigned to non-usual

I-O addresses.  UARTCHK interprets the command-line parameters as base

16 numbers; these may (but don't need to) start with "0x" or end with

"h".  UARTCHK ignores any parameters which cannot be interpreted as

base 16 numbers.  It also ignores any addresses less than 100h, since

the DMA controller, interrupt controller, and other critical system

hardware have registers there.

CAUTIONS:

Use UARTCHK only when the serial port(s) involved are idle.  Any

operation in progress (for example, a background data transfer) could

be disrupted.

Use command-line addresses judiciously.  UARTCHK saves and restores

the value of any I-O port it modifies; however, access to arbitrary I-O

addresses (possibly occupied by devices other than UARTs) may have

unpredictable results.

Examples:

COMMAND                             OUTPUT

------------------------------------------------------------------------

uartchk

3F8h (port 1):  16450

2F8h (port 2):  16450

3E8h (port 3):  8250

2E8h (port 4):  none

uartchk 2e8

2E8h:  none

uartchk 3f8h 3f8 0x3f8

3F8h:  16450

3F8h:  16450

3F8h:  16450

uartchk no numbers here

------------------------------------------------------------------------

Acknowledgement:

The methods used here for detecting UARTs and distinguishing UART

types were described in Jeff Duntemann's "Structured Programming"

column, in Dr. Dobb's Journal, July 1991.

Program and documentation by

Thomas W. S. Smith

September 26, 1991