The Tandy 1000 Sound Digitizer

presented by

KSH Software

Copyright (C) 1991  Version 0.3

All Rights Reserved

2

TABLE OF CONTENTS

1.   INTRODUCTION . . . . . . . . . . . . . . . . . . .  3

1.1. Required Equipment . . . . . . . . . . . . . . . .  3

1.2. Optional Equipment . . . . . . . . . . . . . . . .  3

1.3. Recommended Equipment  . . . . . . . . . . . . . .  3

1.4. WARNING! . . . . . . . . . . . . . . . . . . . . .  3

1.5. Shareware/Copyright Notice . . . . . . . . . . . .  4

1.6. Licensing Agreement  . . . . . . . . . . . . . . .  4

1.7. Thanks . . . . . . . . . . . . . . . . . . . . . .  4

2.   INSTALLATION . . . . . . . . . . . . . . . . . . .  5

2.1. Installation . . . . . . . . . . . . . . . . . . .  5

2.2. Running Tandy Sound  . . . . . . . . . . . . . . .  5

3.   USING TANDY SOUND  . . . . . . . . . . . . . . . .  6

3.1. Command Line Options . . . . . . . . . . . . . . .  6

3.2. CTRL-Break . . . . . . . . . . . . . . . . . . . .  6

3.3. On-line Help . . . . . . . . . . . . . . . . . . .  6

3.4. Recording Sound  . . . . . . . . . . . . . . . . .  6

3.4.1.  Volume  . . . . . . . . . . . . . . . . . . . .  7

3.4.2.  Speed . . . . . . . . . . . . . . . . . . . . .  7

3.4.2.1.  Record  . . . . . . . . . . . . . . . . . . .  7

3.4.2.2.  Playback  . . . . . . . . . . . . . . . . . .  7

3.4.3.  Max File Size . . . . . . . . . . . . . . . . .  7

3.4.4.  Filename  . . . . . . . . . . . . . . . . . . .  8

3.5. Playing Sound  . . . . . . . . . . . . . . . . . .  8

3.5.1.  Volume  . . . . . . . . . . . . . . . . . . . .  8

3.5.2.  Speed . . . . . . . . . . . . . . . . . . . . .  8

3.5.2.1.  Record  . . . . . . . . . . . . . . . . . . .  8

3.5.2.2.  Playback  . . . . . . . . . . . . . . . . . .  8

3.5.3.  Filename  . . . . . . . . . . . . . . . . . . .  8

3.6. Utilities  . . . . . . . . . . . . . . . . . . . .  9

3.6.1.  Compression . . . . . . . . . . . . . . . . . .  9

3.6.1.1.  Compressing Files . . . . . . . . . . . . . .  9

3.6.1.2.  Decompressing Files . . . . . . . . . . . . .  9

3.6.2.  Filtering . . . . . . . . . . . . . . . . . . .  9

3.6.2.1.  Filter Level  . . . . . . . . . . . . . . . .  9

3.6.2.2.  Filter Block Size . . . . . . . . . . . . . .  9

3.6.3.  Converting Deskmate Files . . . . . . . . . . . 10

4.   REGISTRATION INFORMATION . . . . . . . . . . . . . 11

5.   HOW TO CONTACT ME  . . . . . . . . . . . . . . . . 12

3

1.  INTRODUCTION

This program was created because I felt that the SOUND

program sold with Deskmate was lacking in many many many

places.  I thought it was fun to be able to record tiny

snippets of sound, but I wanted to record whole songs.  The

result is this program.

1.1.  Required Equipment

This program requires a Tandy 1000 SL or Tandy 1000 TL

(/2s also).  You will also need a microphone or a mono to

mono line out to mini-plug input.

1.2. Optional Equipment

The internal speaker on these machines is not the best

in the world.  I suggest using a set of Walkman headphones.

1.3. Recommended Equipment

I suggest that you use the program with a Hard Disk.

You can attempt to use it with a floppy drive.  If you do, I

suggest that you use a 3-1/2" drive.  The files created by

this program will get VERY LARGE!  Four minutes of music fit

in a file about 2.5 Megs in size!

1.4. WARNING!

The MIC input port on the Tandy 1000 has 2 settings.

The first setting is MIC.  The MIC setting is the default

that Tandy ships these PCs with.  On the sound satellite

board (located directly behind the speaker), the jumper

positioned in the E2-E3 position gives you this mode.  This

setting is for low-power input signals.  The signal goes to

the A/D converter UNATTENUATED.  If you connect a LINE OUT or

any high-power output to this setting, you could seriously

damage the satellite board (or more).

If you plan to use a LINE OUTPUT to the computer, you

must move the jumper from E2-E3 to the E1-E2 position.  This

setting causes the input signal to be ATTENUATED.  A

microphone will not work in this mode.

DO NOT UNDER ANY CIRCUMSTANCES CONNECT AN AMPLIFIED

OUTPUT (Speaker or headphone output) TO THE MIC INPUT.

The previously mentioned jumper is a minor design flaw

of the Tandy.  Here is my suggestion to remove this problem.

Purchase a SPDT switch and replace the jumper with the

switch.  Mount the switch externally.  Remember that E2 is

the common connection.  For more information on how to do

this, send me mail and I'll send you a diagram.

4

1.5. Shareware/Copyright Notice

This program is NOT FREE.  This program is distributed

as shareware or user-supported software.  I am asking for a

mere $25 as a registration fee.  Your registration fee will

put you on a mailing list that will inform you of later

updates.  If you decide to keep and use this program, I would

greatly appreciate you sending some form of reimbursement.

Please make checks payable to Sean Haynes.

Mail payments to:

KSH Software Registration

P.O. Box 952

Naples, ME 04055

This program is Copyright 1990 by Sean A.R. Haynes.  This

program may be freely distributed among friends [or enemies

I'm not picky] and BBS systems under the sole condition that

all files included in this archive remain intact.

The following files are included in this archive:

TNDSOUND.EXE  (The executable file)

TNDSOUND.HLP  (The on-line help file)

INSTALL.BAT   (The installation batch file)

TNDSOUND.DOC  (This file)

REGISTER.FRM  (The registration form)

GODIR.COM     (Used in installation)

REVISION.DOC  (Revision history)

1.6.  Licensing Agreement

By using this program, you agree to abide by the above

rules.  You also agree that if you use any of the routines in

this program, or any of the sound files created by this

program in any other program, you MUST send in your

registration fee and you MUST give credit where credit is due

in your documentation.

1.7.  Thanks

A special thanks to John Brandolini and Robert Pomeroy

for there help with attempting to design a compression

routine.

5

2.   INSTALLATION

2.1. Installation

This program should have been delivered with a program

called 'INSTALL.BAT'.  This program should automatically

create all necessary directories and files.  (Make sure that

all the files have been de-archived).

The correct usage of INSTALL is as follows:

INSTALL targetdrive: sourcedrive:\path

Example:

C:\> INSTALL A: C:\TNDSOUND (This would install the

program on the A: floppy drive, using the files in

the C:\TNDSOUND directory.)

After typing in the above command, the following output

should appear on the screen:

C:\> echo off

1 File(s) copied

1 File(s) copied

1 File(s) copied

Tandy Sound installed

If anything else appears on the screen, an error has

occurred.  Check to make sure you have followed the correct

format and that all the files have been de-archived.

You must de-archive TNDSOUND.EXE, TNDSOUND.HLP, and

INTRO.SND for the program to operate successfully.

2.2. Running Tandy Sound

To run Tandy Sound, make sure that you are either in the

right directory, or the directory exists in your search path.

Type the word TNDSOUND at the DOS prompt.

6

3.  USING TANDY SOUND

This program is really easy to use.   All you need is a

Tandy 1000 and a microphone and you're all set.  Plug your

microphone into the MIC input on the front of the Tandy and

away you go!  If you want to record from a CD, Tape, Stereo,

or Television, read the WARNING section to learn how to

change the internal jumper.

I have found that you will either need a stereo to mono

jack, or a mono to mono jack to record from a stereo.  I will

also suggest that you try not to make one, because it can

cause problems.  I am using my Realistic portable CD player

with a mono to mono jack.  You can also hook up to most rack

systems with a RCA to mini-jack.

Again, I REPEAT, DO NOT UNDER ANY CIRCUMSTANCES CONNECT

TO AN AMPLIFIED OUTPUT!

3.1. Command Line Options

As of version 0.3, if you specifiy /O on the command

line, the program will bypass the machine detection routine.

Use this option if you feel that your computer has the

digitization capabilities of the Tandy SL/TL.  If you use

this option and the computer does not have the capabilities,

the computer may hang making a reboot necessary.

3.2. CTRL-Break

If for some reason the sound channel hangs up and the

computer locks up, pressing CTRL-Break will clear the sound

channel.  Doing this causes both recording and playing to

stop immediately.

3.3. On-line Help

Most of what is said in this file is repeated in the On-

line Help file.  To view this file, press F1 from the Main

Menu.

3.4. Recording Sound

To record sound, press the F2 key from the Main Menu.

This will pull up the record sound window.

When 'B' is pressed, the program will start recording

sound.  A window will appear on the screen telling you the

filename and filesize of the sound being recorded.  Recording

will stop when either the Max Filesize is reached or when a

key is pressed.  Recording will also stop if there is any

kind of disk error.

7

After recording has finished, the program will playback

the sound just recorded at the Volume and Speed specified.

If you want to change these options, you can use the playback

menu.

The following options are also available:

3.4.1.  Volume

The Volume can be set anywhere from 1-7, where 7 is

loudest.  This has no affect on how the sound is recorded.

Sound will always be recorded at the same level, but will be

played back at the level that volume is set to.

By pressing 'V', you can change the volume at which the

sound will be played back.  You can also change this setting

in the playback menu.

3.2.2.  Speed

The speed selections are comparable to the sampling rate

at which the A/D works.   The lower the number, the higher

the sampling rate.  I have not been able to get an exact

correlation between the two, but the following suggested

speeds for recording and playing back seem to work.  You can

play around with the numbers until you find one that suits

your needs.

3.4.2.1.  Record

By pressing 'R', you can change the speed at which data

is recorded.  For voice recording, values of 40-60 are

usable.  I have been able to go over 100 however.  The higher

the number, the more you can record in less space.  Music,

however, requires speeds of around 30-40.  You can try higher

numbers, but you might begin to lose data.

3.4.2.2.  Playback

By pressing 'P', you can change the speed at which data

is played back.  This can also be done in the playback menu.

This has no effect on how the program is record, it is the

rate at which the data is played back through the D/A.  I

have found that it is approximately 10 times higher than the

Record speed (i.e. if you record at 30, you playback at 300).

3.4.3.  Max File Size

By pressing 'M', you can specify a filesize that

recording will stop at.  Being that the program records in

32K blocks, the actual filesize may be slightly larger than

the size entered.  This is good for recording to floppies

(which is unrecommended anyway).

8

3.4.4.  Filename

By pressing 'F', you can change the filename that you

wish to record.  The '.SND' extension is assumed if no

extension is entered.

3.5. Playing Sound

When a file is recorded, an ID is written to the

beginning of the sound file which stores the Record Speed,

Playback Speed, and Volume.  When a sound file is loaded,

this information is read in.  If you want to change these

values, you would use the Playback Sound Menu.  To get to

this menu, press F3 from the Main Menu.

When 'B' is pressed, the program will playback the file.

If the program does not see the previously mentioned ID, then

it will not let you play the file.

Once you make a change to the Volume or Playback Speed,

that value is stored in the ID of the file.  The Record

Speed, for obvious reasons, cannot be changed once the file

has been recorded.  It is displayed on the Playback screen as

a reference.

The following options are also available in this menu:

3.5.1.  Volume

This selection performs the same function as in the

Record Sound Menu, except the ID will be rewritten

immediately to the file.

3.5.2.  Speed

This selection performs the same function as in the

Record Sound Menu, except the ID will be rewritten

immediately to the file.

3.5.2.1. Record

This value cannot be changed from the playback menu.  It

is displayed on the screen as a reference.

3.5.2.2.  Playback

This selection performs the same function as in the

Record Sound Menu, except the ID will be rewritten

immediately to the file.

3.5.3.  Filename

Same as Record Sound, with the following exception: You

must enter a valid filename.  If the program does not see the

9

previously mentioned ID, then it will not let you play the

file.  Pressing <RETURN> with nothing entered will change the

filename to the default SAMPLE.SND.

3.6. Utilities

This menu can be reached by pressing F4 from the Main

Menu.  This menu contains the following options:

3.6.1.  Compression

The Compression Utilities are currently under

development.  Please look for them in later versions.

3.6.1.1.  Compressing Files

This option is currently under development.

3.6.1.2.  Decompressing Files

This option is currently under development.

3.6.2.  Filtering

Filtering will attempt to remove hiss from a file.  The

one problem that exists is that as more hiss is removed, you

also risk the possibility that data may be lost.

Your best bet is to start with a Filter Level of about 6

and a block size of 6 and slowly raise the Level and/or lower

the block size.

The filtering process may cause data to be lost that might

make the file sound really bad, so be careful not to lose the

.OLD file if you have butchered the original file.

3.6.2.1.  Filter Level

This is the level of filtering that you want to perform

on the file.  The higher the number, the more filtering is

performed.  Also, the higher the number the more possibility

that data will be lost.

3.6.2.2.  Filter Block Size

This number determines the size of the block of hiss

that must exist in order to be removed, where 0 is the

smallest block.  The higher the bigger the block.  I suggest

that you play with these two options to find levels that suit

your needs.

10

3.6.3.  Converting Deskmate Files

Being that most of the .SND files out there are stored

in a format compatible with Deskmate, I have made a selection

that will take a Deskmate file and convert it into a format

that my program can deal with.  By pressing 'M' from the

Utilities menu, you can convert the currently listed

filename.

11

4.  REGISTRATION INFORMATION

Please register your copy of the Tandy Sound Digitizer.

Below is a copy of the Registration form:  (you can also

print out REGISTER.FRM).

------------------------------------------------------------

KSH SOFTWARE

TANDY SOUND DIGITIZER V0.3  (C) 1991

-------------------------------------------------------------

Name: ______________________________________________________

Company: __________________________ Title: _________________

Address: _____________________________________ Apt. #: _____

City: ____________________________ State: ____ ZIP: ________

------------------------------------------------------------

Comments:  _________________________________________________

____________________________________________________________

____________________________________________________________

____________________________________________________________

------------------------------------------------------------

ELECTRONIC ADDRESSES

[  ]  BBS: Telephone #: (____) ____-______ Name: ____________

[  ]  Internet Address:  ____________________________________

[  ]  Bitnet Address:  ______________________________________

(I prefer BBS'S in the 617 area code.  Also, if at all

possible, leave a BITNET or Internet/ARPAnet address)

-------------------------------------------------------------

Please enclose $25 registration fee.  Programmers wishing to

get the TNDSOUND.TPU Unit add an extra $10.

Make check or money order payable to:  Sean A. Haynes

Mail this form to:    KSH Software Registration

P.O. Box 952

Naples, ME 04055

12

5.  HOW TO CONTACT ME

I can be reached at the following addresses:

USnail:    Sean A.R. Haynes

46 Udine St.

Arlington, MA 02174-5657

INTERNET:  PA_HAYNES@NUHUB.NORTHEASTERN.EDU

SHAYNES@LYNX.NORTHEASTERN.EDU

HAYNES@NUENG.COE.NORTHEASTERN.EDU

BITNET:    SHAYNES@NUHUB

BBS:       Xevious BBS   (617) 449-7322

Channel 1 BBS (617) 354-8873

VOICE:     (617) 648-8390

OTHER:     PC-Link    Killer68

Comments and suggestions are always welcome.....