NBridge.Doc for NIC Bridge Version 1.1i

Nov 12 1992

Copyright (c) Eighth Layer Systems 1990-1992.

All rights reserved.

Warning:  The possibility of file corruption exists when using any software.

Please take whatever steps are required to protect vital files.

This documentation is formatted with 23 line pages. It contains

IBM PC type block graphics characters.

����������Ŀ

� CONTENTS �

������������

RELEASE NOTES..................................................  4

INTRODUCTION............Definitions............................  5

Files Included.........................  6

NBridge Drivers........................ 10

SRTok Drivers.......................... 11

NBr Bridge Utility..................... 14

NicSetup Setup Utility................. 15

CONSIDERATIONS..........Ethernet Transparent Bridging.......... 16

Token Ring Source Routing Bridging..... 21

SETUP/MAIN CONFIGURATION....................................... 31

OPERATION...................................................... 36

NBRIDGE PARAMETERS............................................. 37

SRTOK PARAMETERS............................................... 43

ACKNOWLEDGEMENTS............................................... 48

���������������Ŀ

� RELEASE NOTES �

�����������������

- The Install.Exe utility can be used to upgrade recent releases

of the software by installing over a previous version.

- The NBridge CUSTOMFILTER parameter is now documented.

- The SRTok SRBC parameter is now documented.

��������������Ŀ

� INTRODUCTION �

����������������

�������������

Definitions

�������������

The term "LanRoot" denotes the directory under which all network

software is installed. For MS Lan Manager this is \LANMAN.DOS for DOS or

\LANMAN for OS/2. For IBM Lan Server this is \IBMLAN for OS/2.

NIC is a common network industry acronym for Network Interface Card.

This is the Ethernet, Token Ring or other adapter which connects PCs to a

network. NIC Async allows an asynchronous serial port to be used as a NIC.

MAC is an acronym for Medium Access Control. Network adapter drivers

are often referred to as MAC drivers.

NDIS is an acronym for the Microsoft/3Com Network Driver Interface

Specification. This is a software interface designed to allow multiple network

protocols to control network adapter drivers. NIC Async adheres to the NDIS

version 2.01 specifications.

����������������

Files Included

����������������

The following files are included on the Install Disk:

\Readme  .Doc       NIC Bridge/Async introductory document

\NBridge .Doc       This NIC Bridge document

\Tune    .Doc       Tuning document

\Readme  .Bat       DOS batch file for NicDoc.Exe

\Readme  .Cmd       OS/2 command file for NicDoc.Exe

\NicDoc  .Exe       NIC Bridge/Async DOS & OS/2 Documentation Viewer

\Install .Exe       NIC Bridge/Async DOS & OS/2 Install Utility

\NicSetup.Exe       NIC Bridge/Async DOS & OS/2 Setup Utility

\NicCfg  .Exe       NIC Bridge/Async DOS & OS/2 Configuration Utility

\NBr     .Exe       NIC Bridge DOS & OS/2 Full Screen Interface

\NBridge .DOS       DOS  NIC Bridge Driver for 386 & up

\NBridge .D86       DOS  NIC Bridge Driver for 8086 & up

\NBridge .OS2       OS/2 NIC Bridge Driver for 386 & up on OS/2 1.2 & up

\NBridge .O86       OS/2 NIC Bridge Driver for 286 & up or OS/2 1.1

\SRTok   .DOS       DOS  Src Route Driver for 386 & up

\SRTok   .D86       DOS  Src Route Driver for 8086 & up

\SRTok   .OS2       OS/2 Src Route Driver for 386 & up on OS/2 1.2 & up

\SRTok   .O86       OS/2 Src Route Driver for 286 & up or OS/2 1.1

\Ethern  .DOS       DOS  Dummy Ethernet Driver

\Ethern  .OS2       OS/2 Dummy Ethernet Driver

\Protman\Protman .DOS       DOS  Protocol Manager Driver

\Protman\Protman .OS2       OS/2 Protocol Manager Driver

\Protman\Protman .Exe       DOS  Protocol Manager Transient Portion

\Protman\Pro     .Msg       Protocol Manager Message File

\Protman\Proh    .Msg       Protocol Manager Help Message File

\Protman\NetbindD.Exe       DOS  Protocol Manager Netbind Utility

\Protman\NetbindO.Exe       OS/2 Protocol Manager Netbind Utility

\MSLM\Protocol.BD           DOS  Protocol.Ini for NIC Bridge

\MSLM\Protocol.BO           OS/2 Protocol.Ini for NIC Bridge

\MSLM\NBridge .XD           DOS   386 NBridge.Xif

\MSLM\NBridge .XD8          DOS  8086 NBridge.Xif

\MSLM\NBridge .XO           OS/2  386 NBridge.Xif

\MSLM\NBridge .XO8          OS/2 8086 NBridge.Xif

\MSLM\Protocol.SD           DOS  Protocol.Ini for Src Route Token driver

\MSLM\Protocol.SO           OS/2 Protocol.Ini for Src Route Token driver

\MSLM\SRTok   .ND           DOS   386 SRTok.Nif

\MSLM\SRTok   .ND8          DOS  8086 SRTok.Nif

\MSLM\SRTok   .NO           OS/2  386 SRTok.Nif

\MSLM\SRTok   .NO8          OS/2 8086 SRTok.Nif

�����������������

NBridge Drivers

�����������������

The DOS and OS/2 drivers NBridge.DOS and NBridge.OS2 are the "engine"

of NIC Bridge and the vast majority of work is done within them. They are NDIS

protocol level drivers and can be bound to multiple MAC level drivers to

provide bridging between them. They do NOT provide the NetBIOS interface that

protocol drivers such as MS/IBM NetBEUI or 3Com NBP do.

The NBridge drivers can be bound to a maximum of 9 MAC level drivers.

These MAC level drivers may be NAsync serial port drivers, SRTok Token Ring

drivers or NDIS ethernet adapter drivers.

If you need support for PCs based on 80286 or lesser CPUs you will

have to use the slower drivers: NBridge.D86 for DOS or NBridge.O86 for OS/2.

The slower OS/2 driver is also required on OS/2 versions prior to version 1.2.

���������������

SRTok Drivers

���������������

The DOS and OS/2 drivers SRTok.DOS and SRTok.OS2 are required to use

NIC Bridge in Source Routing mode on a Token Ring Network. The NDIS specs

provide for adapter drivers to advertise themselves as providing Source

Routing Bridge capability but no such drivers are readily available. These

drivers provide this capability for IBM Token Ring adapters.

If you need support for PCs based on 80286 or lesser CPUs you will

have to use the slower drivers: SRTok.D86 for DOS or SRTok.O86 for OS/2.

The slower OS/2 driver is also required on OS/2 versions prior to version 1.2.

These drivers should work with all IBM Token Ring Adapters except for

the original "IBM Token-Ring Adapter" and the new Bus Mastering adapter.

Specifically the drivers should work with any of the following:

"IBM Token-Ring Network Adapter II"

"IBM Token-Ring Network Adapter II/A"

"IBM Token-Ring Network 16/4 Adapter"

"IBM Token-Ring Network 16/4 Adapter/A"

"IBM Token-Ring Network PS/2 Model P70 386 Adapter/A"

At this time the SRTok drivers have been tested and verified with the

"IBM Token-Ring Network Adapter II", "IBM Token-Ring Network 16/4 Adapter"

and "IBM Token-Ring Network 16/4 Adapter/A" cards.

The SRTok drivers are not currently supported for any use other than

with NIC Bridge. IE, no other NDIS protocol drivers may be bound to SRTok.

Thus, given that only 2 IBM Token Ring Adapters may be installed in a single

PC, a two port local bridge configuration with the SRTok drivers will NOT be

able to act as a network workstation or server unless it does so via a non-IBM

Token Ring adapter.

A workstation or server may act as a remote bridge with 2 IBM Token

Ring adapters: 1 for the workstation/server functions and 1 for the bridge

function.

The use of "RAM paging" is not currently supported by these drivers,

nor is the use of shared interrupts on MCA machines. It is generally best to

set the adapter for the maximum available "Shared RAM Size". This is usually

16 KBytes on 4 Megabit per second cards and 64 KBytes on 4/16 cards.

��������������������

NBr Bridge Utility

��������������������

The bridge utility NBr.Exe provides a full-screen interface to view

NIC Bridge statistics, NDIS statistics and other information provided by the

network card drivers. It runs under both DOS & OS/2.

In addition to the function keys shown at the bottom of the Nbr

display, shortcut keys to the menu are implemented. These include Alt-O for

the Options menu, Alt-M for the Mode menu and Alt-A for the Adapter menu.

������������������������

NicSetup Setup Utility

������������������������

The setup utility NicSetup.Exe can be used to setup NIC Bridge on

standalone or networked PCs. It runs under both DOS & OS/2.

Please refer to the Setup & Configuration section for further details

on use of the NicSetup utility.

����������������Ŀ

� CONSIDERATIONS �

������������������

�������������������������������

Ethernet Transparent Bridging

�������������������������������

NIC Bridge can provide MAC Level transparent bridging when used in

conjunction with virtually any NDIS Ethernet driver and adapters. Installed

on a PC with two Ethernet cards this can provide local bridging. With the

NIC Async drivers on a PC with one Ethernet card a multi-port remote bridge

can be configured. A remote bridge configuration may be used to link distant

networks or to provide remote access to the local network by standalone PCs.

Transparent bridging is so named because network protocol software

needs no knowledge of the bridge's existence. The bridge operates by putting

the Ethernet adapters into "promiscuous" mode where every network packet is

seen. By monitoring these packets the bridge learns where network stations are

by looking at the IEEE source addresses of the packets. When a packet must

cross the bridge to reach it's destination the bridge recognises this and

forwards the received packet to the proper adapter. Broadcast packets are

forwarded to all adapters except the source adapter.

On busy networks this constant monitoring of all packets can use all

of the PC's CPU power. For this reason it is recommended that a dedicated PC

be used for transparent bridging on busy networks. It is further recommended

that high quality adapters and drivers be used with NIC Bridge. The high data

rates that the bridge is capable of can expose rare bugs in drivers. A list of

which adapters and drivers are fastest and most compatible with NIC Bridge

will be compiled and maintained. Testing has been done mostly with 3Com 3C503

Etherlink II adapters. The 3C503 drivers have been found to be very stable.

The promiscuous mode necessary for transparent bridging is available

on most true Ethernet NDIS drivers. Many NDIS drivers which merely emulate an

Ethernet driver do not support promiscuous mode however. These drivers include

those for Arcnet and FDDI. Among the true Ethernet NDIS drivers included with

MS Lanman 2.1 only the Spider Systems Ethernet SC-100E driver appears to lack

promiscuous mode.

Some Ethernet NDIS drivers provide a feature known as "Loopback". This

causes an adapter driver to appear to receive packets which it has transmitted

and can reduce the performance of NIC Bridge. Although they may otherwise be

excellent, the drivers from Accton, DCA, Western Digital and Ungermann Bass

provide Loopback and may be less than ideal for bridging.

The address learning responsibility of a transparent bridge requires

the use of an address table. The current version of NBridge allows a maximum

of 256 network addresses to be stored. This should present no problems except

on very busy, very large networks.

It should be noted that the NAsync drivers on a server acting as a

remote bridge may need to be bound to server protocols as well as the NBridge

protocol. This is because the NBridge protocol provides remote access to the

external network, but not local protocols on the PC on which it is installed.

One exception to this occurs when using a second network adapter

exclusively for bridging, as the second adapter provides full access to all

protocols bound to the first adapter. Another potential exception occurs when

the adapter driver fully supports loopback in promiscuous mode.

No special parameters are required for NIC Async when used with NIC

Bridge in transparent bridge mode. The [NASYNC_NIF] parameter MAC_TYPE should

be set to it's default of "DIX+802.3".

The following Protocol.Ini parameters are relevant for a transparent

bridge configuration and are set in the [NBRIDGE_XIF] section of Protocol.Ini.

; Only allowed parameter for Drivername.

DRIVERNAME    = NBRIDGE$

; "TRANSPARENT" required for transparent bridging.

BRIDGEMODE    = "TRANSPARENT"

; 1 Binding specified for each port of the bridge.

; This example is for a combined local and remote bridge.

BINDINGS      = "ELNKII_NIF", "ELNKII_NIF2, "NASYNC_NIF", "NASYNC_NIF2"

; Enables Bridging for each NIC driver. Order identical to BINDINGS.

;  Allowed: "YES" or "NO".

;  Default:  "YES" for all.

BRIDGE_MAC      = YES, YES, YES, YES, YES, YES, YES, YES, YES

; Enables Packet Capture for each NIC driver. Order identical to BINDINGS.

;  Allowed: "YES" or "NO".

;  Default:  "YES" for all.

;  Recommend "NO" for high-performance production use.

CAPTURE_MAC     = YES, YES, YES, YES, YES, YES, YES, YES, YES

; Filters Broadcasts rcvd by each NIC driver. Order identical to BINDINGS.

;  Allowed: "YES" or "NO".

;  Default:  "NO" for all.

; Set "YES" ONLY on local NICS to eliminate broadcast traffic.

; Note: "YES" does not allow correct operation of MS TCP/IP.

FILTERBC_MAC  = NO, NO, NO, NO, NO, NO, NO, NO, NO

������������������������������������

Token Ring Source Routing Bridging

������������������������������������

NIC Bridge provides Source Routing bridging when used in conjunction

with the SRTok driver and an IBM Token Ring Adapter. Installed on a PC with

two IBM Token Ring Adapters this can provide local Source Routing bridging.

With the NIC Async drivers on a PC with one IBM card a multi-port remote

Source Routing bridge can be configured. A remote bridge configuration may be

used to link distant networks or to provide remote access to the local network

by standalone PCs. Note that an IBM adapter is only required on the bridge PC.

Other stations on the network can use virtually any Token Ring adapter.

Source Routing Bridging is so named because the source of a network

packet must explicity specify the route the packet will take through bridges.

Thus the network protocol software must explicitly support Source Routing.

Protocols such as MS/IBM NetBEUI support this but 3Com NBP does not.

The bridge operates by putting the Token Ring adapter into Source

Routing Bridge mode, where only packets needed to be forwarded by the bridge

are seen. The bridge then forwards received packets to the proper adapter.

Because Source Routing Bridge mode on the IBM Token Ring adapters

can be set to only cause packets which need forwarding to be received, busy

networks are less of a problem. It is still recommended, however, that a

dedicated PC be used for Source Routing bridging on busy networks.

The Source Routing architecture requires that each ring in the Token

Ring inter-network be assigned a ring number from 0 to decimal 4095. The IBM

scheme for Source Routing Bridging on IBM Token Ring adapters requires that

the adapter be programmed with the "Local Ring Number" for the ring that

adapter is connected to, and an "Other Ring Number" for the ring another

adapter in the same bridge PC is connected to.

This generally restricts IBM Token Ring adapter Source Routing bridges

to supporting only two adapters. As only two IBM adapters may be installed in

a single PC this is no problem for a local bridge based on IBM adapters. This

is however a problem for a multi-port remote bridge.

NIC Bridge tackles this problem by using the concept of an "Internal

Ring" in the bridge. This Internal Ring has no stations on it, only virtual

internal bridges. The diagram following shows this.

Note that all "bridges" shown are virtual bridges within NIC Bridge.

This configuration employs two IBM Token Ring adapters and three serial ports

for a total of five ports. The local rings, Ring 1 and Ring 2 are each served

by one of the IBM Token Ring adapters. Ring 3 is the internal ring and exists

in a logical sense only. The remote rings, Rings 4-6, are served by one of the

three serial ports.

This "Internal Ring" scheme allows the use of a common "Other Ring

Number" for all ports and thus allows multiple port configurations. The only

potential disadvantage to this scheme is that two "bridges" must be crossed

by a packet moving from one port to another. As a "Hop limit", or maximum

crossed bridged count, of 8 is used in Source Routing, this 2 bridge hop will

limit the number of bridges a packet may cross to 4, rather than 8. As a

remote bridge must exist at each end of a serial link, the actual limit to

the maximum number of serial links a packet may cross is 2.

{ LS = Local Station, RS = Remote Station }

LOCAL RINGS              SHARED  RING      REMOTE  RINGS

(Wired)                 (Internal)          (Async)

���Ŀ     ���Ŀ

���ĴLS1�����ĴLS2���Ŀ         ��������Ŀ         �������Ŀ

�   �����     ����� �������������Ŀ    �������������Ŀ   ���Ŀ

�     Ring 1        � Loc Bridge 1�    � Rem Bridge 1�   �RS1� Ring 4

�   ���Ŀ     ���Ŀ ���������������    ���������������   �����

���ĴLS3�����ĴLS4�����         �        �         ���������

�����     �����             � Ring 3 �         �������Ŀ

�      �������������Ŀ   ���Ŀ

�      � Rem Bridge 2�   �RS2� Ring 5

�      ���������������   �����

���Ŀ     ���Ŀ             �        �         ���������

���ĴLS5�����ĴLS6���Ŀ         �        �         �������Ŀ

�   �����     ����� �������������Ŀ    �������������Ŀ   ���Ŀ

�     Ring 2        � Loc Bridge 2�    � Rem Bridge 3�   �RS3� Ring 6

�   ���Ŀ     ���Ŀ ���������������    ���������������   �����

���ĴLS7�����ĴLS8�����         �        �         ���������

�����     �����             ����������

It should be noted that remote Source Routing bridging from one

network to another requires attention to ring numbering. A two port remote

bridge, for example, requires three ring numbers: a local ring number, an

internal ring number and a serial port ring number. All three of these must

be different from each other and all other rings accessible via bridges.

The asynchronous serial data path between two remote bridges, however,

acts as if it is a single ring. Thus, the serial port ring number of two

remote bridges which connect must be identical. This is illustrated below.

����������Ŀ ����������Ŀ ����������Ŀ

�  LOCAL   � � INTERNAL � �  SERIAL  �

�          �Ĵ          �Ĵ          �Ŀ

� Ring # 1 � � Ring # 2 � � Ring # 3 � � Serial Data Path

������������ ������������ ������������ �

����������������

� ����������Ŀ ����������Ŀ ����������Ŀ

� �  SERIAL  � � INTERNAL � �  LOCAL   �

�Ĵ          �Ĵ          �Ĵ          �

� Ring # 3 � � Ring # 4 � � Ring # 5 �

������������ ������������ ������������

NIC Async must be configured to appear as a Token Ring adapter when

used with NIC Bridge in Source Routing bridge mode. This applies to the NAsync

drivers loaded on the bridge as well as those loaded on remote PCs.

This can be done by changing the MAC_TYPE parameter in the

[NASYNC_NIF] section of Protocol.Ini from the default of "DIX+802.3" to

"802.5".

; NIC Async parameter in [NASYNC_NIF] section of Protocol.Ini

;  required for driver installed on Bridge PC AND remote PC.

; Ensure "802.5" exactly, default is "DIX+802.3" for Ethernet.

MAC_TYPE      = "802.5"

The following Protocol.Ini parameters are relevant for a Source

Routing bridge configuration. These are set in the [NBRIDGE_XIF] section of

Protocol.Ini.

; Only allowed parameter for Drivername.

DRIVERNAME    = NBRIDGE$

; "SOURCEROUTING" required for Source Routing bridging.

BRIDGEMODE    = "SOURCEROUTING"

; Internal Ring number for virtual ring connecting all internal bridges.

;  Allowed:  0x0 - 0x0FFF or 0 - 4095.

;  Use any valid unassigned Ring Number.

INTERNALRING  = 1110

; 1 Binding specified for each port of the bridge.

; This example is for a combined local and remote bridge.

BINDINGS      = "SRTOK_NIF", "SRTOK_NIF2, "NASYNC_NIF", "NASYNC_NIF2"

; Enables Bridging for each NIC driver. Order identical to BINDINGS.

;  Allowed: "YES" or "NO".

;  Default:  "YES" for all.

BRIDGE_MAC    = YES, YES, YES, YES, YES, YES, YES, YES, YES

; Enables Packet Capture for each NIC driver. Order identical to BINDINGS.

;  Allowed: "YES" or "NO".

;  Default:  "YES" for all.

;  Recommend "NO" for high-performance production use.

CAPTURE_MAC   = YES, YES, YES, YES, YES, YES, YES, YES, YES

; Ring number for each NIC driver bound. Order identical to BINDINGS.

;  Allowed:  0x0 - 0x0FFF or 0 - 4095.

;  If you already have SR Bridges assigned Ring Numbers MUST match these.

RING_MAC      = 1111, 1112, 1113, 1114, 1115, 1116, 1117, 1118, 1119

; Filters Broadcasts rcvd by each NIC driver. Order identical to BINDINGS.

;  Allowed: "YES" or "NO".

;  Default:  "NO" for all.

; Set to yes ONLY on local NICS to eliminate broadcast traffic.

; Note: "YES" does not allow correct operation of MS TCP/IP.

FILTERBC_MAC  = NO, NO, NO, NO, NO, NO, NO, NO, NO

The SRTok driver Protocol.Ini parameters are the same as those for

the IBM Token Ring NDIS driver. The following parameters are those that are

most important and are set in the [SRTOK_NIF] section of Protocol.Ini.

; SRTOK$ for first adapter, SRTOK2$ for second adapter.

DRIVERNAME    = SRTOK$

; Required to set non-default Ram address.

RAM           = 0xD800

; Either Primary or Alternate adapter, NOT BOTH ! Default = PRIMARY.

PRIMARY

;ALTERNATE

; Setting SRBC = NO allows single route broadcasts to be filtered.

SRBC = YES

��������������������������Ŀ

� SETUP/MAIN CONFIGURATION �

����������������������������

To setup and configure NIC Bridge:

1)      Install Lan Manager or Lan Server if needed on this PC. Ensure that

all local Ethernet and Token Ring adapters have been configured

properly.

2)      Install NIC Bridge/Async on the PC. With the NIC Bridge/Async install

disk inserted into the A: drive the command "A:\Install" will initiate

the installation procedure. When this is complete the PC should be

rebooted.

3)      If remote ethernet bridging will be provided and this PC is a server,

NIC Async must be setup and configured. This will provide remote

access to this server, while the bridge component provides access to

other servers. Token ring bridges should not have NIC Async installed.

as the second Token Ring adapter provides access to this server. Refer

to the NIC Async document for NIC Async installation instructions.

4)      If this will be an ethernet bridge, any device drivers required for

your ethernet adapter should be copied to the NICDIR directory,

normally C:\NIC11.

Run NicSetup.Exe and ensure that the General Configuration information

displayed is appropriate. Option 2 "Change General Configuration" may

be used to make any changes.

Select option 3 to "Setup NIC Bridge" and answer the prompts with

appropriate selections.

If setting up an Ethernet bridge:

If your network adapter is not listed as a selection, specify the

"Dummy Ethernet Driver". This will require Config.Sys lines of the

form "DEVICE = C:\NIC11\ETHERN...." be replaced with entries for

your ethernet adapter driver. The Protocol.Ini [ETHERN_NIF] sections

will also have to be modified to set the appropriate options for

that adapter driver.

5)      Before rebooting your PC, the Protocol.Ini file may need modification.

This can be done with the NicCfg.Exe utility. Within NicCfg select

"Config", "Bridge". When each screen of options is appropriate

press the <Enter> key.

The Ring Number variables are important only when the bridge

is setup as a source routing bridge.

Other Bridge options may be left at default or changed as appropriate

for your environment. Press <Enter> to save the options.

If Source Routing is being used, select "Config", "SR Token 1" to

configure the Source Routing Token Ring driver. Ensure that the Ram

Address selected is available on your PC.

Other SRTOK options may be left at default or changed as appropriate

for your environment. Press <Enter> to save the options.

If this is a remote bridge select "Config", "Async 1". When each

screen of options is appropriate press the <Enter> key. Repeat this

for "Async 2" and all other serial ports configured.

The MAC Type option should be set to Ethernet unless this is a Source

Routing bridge when it should be set to Token Ring.

The Network address value should be unique among all network addresses

in use on the network. Remember to set a DIFFERENT Network address on

the host and all clients.

Other options may be left at default or changed as appropriate for

your environment.

Exit NicCfg with the <F3> key.

Inspect and edit Protocol.Ini to ensure any Ethernet adapter drivers

are configured as desired. Note that many ethernet adapters use IRQ 3

by default and that this will interfere with COM2.

6)      All servers and workstations which will use remote resources or

be remotely accessed must be tuned for low speed operation.

Run NicSetup.Exe and select option 4 to tune NetBEUI and the LAN

software for low speed operation on all servers and workstations

which will communicate via a remote bridge. NicSetup.Exe may be

copied to and run from a floppy disk to perform this on servers.

7)      Reboot the PC and ensure that all drivers load properly.

NOTE:   Running the MS Lan Manager Setup.Exe or IBM Lan Server

LanInst.Exe utilities after NIC Bridge is setup and

configured by cause information in Config.Sys and

Protocol.Ini to be deleted. This will require that

the NIC Bridge setup and configuration be repeated.

�����������Ŀ

� OPERATION �

�������������

The NIC Bridge PC should start and run normally. The Device driver(s)

should load from Config.Sys and display ID messages. Note that should any of

the drivers cause problems booting a PC, installation of the driver may be

aborted by holding down the Alt-Key.

No errors should be produced when NetBind is run (or when the

NETWKSTA.SYS IFS is loaded). If you get any errors, inspect the Protocol.Ini

file for configuration errors and/or try changing the adapter settings. If the

Netbind has succeeded the bridge should now be operational.

Executing NBr.Exe will provide a screen showing the bridge statistics.

Note that when the NAsync drivers are in a "Closed" state, packets will still

appear to be forwarded to them. It is only when a client connects to the host

bridge that these packets are actually forwarded through the serial port.

��������������������Ŀ

� NBRIDGE PARAMETERS �

����������������������

The following pages detail Protocol.Ini parameters for the NBridge

driver. Defaults are shown. Parameter values shown in quotes are ASCII string

parameters. All other parameters are numeric. Numeric parameters preceded with

"0x" are hexadecimal values while all others are decimal values.

REQUIRED NDIS PARAMETERS:

; Required parameter

; Range: NBRIDGE$ only

DRIVERNAME      = "NBRIDGE$"

; Required parameter

; 1 Binding specified for each port of the bridge.

; Form: "Driver1_NIF", "Driver2_NIF", etc. up to 9 ports

BINDINGS        = No Default

GENERAL PARAMETERS:

; Configures Transparent Ethernet or Source Routing

;  Token Ring bridge mode.

; Range: "TRANSPARENT" or "SOURCEROUTING"

BRIDGEMODE      = "TRANSPARENT"

; Enables Bridging for each NIC driver.

; Order identical to BINDINGS.

; Range: "YES" or "NO"

BRIDGE_MAC      ="YES","YES","YES","YES","YES","YES","YES","YES","YES"

; Enables Packet Capture for each NIC driver.

; Order identical to BINDINGS.

; Range: "YES" or "NO"

CAPTURE_MAC     ="YES","YES","YES","YES","YES","YES","YES","YES","YES"

; Enables filtering broadcasts received by each NIC driver.

; Order identical to BINDINGS.

; Range: "YES" or "NO"

; Set "YES" ONLY on local NICS to eliminate broadcast traffic.

; Note: "YES" does not allow correct operation of MS TCP/IP.

FILTERBC_MAC    = "NO", "NO", "NO", "NO", "NO", "NO", "NO", "NO", "NO"

; Each CUSTOMFILTER parameter configures one condition whereby

;  packets which would normally be forwarded are filtered.

; Up to 16 individual CUSTOMFILTER lines may be configured.

; Each custom filter consists of a string of 8 - 58 characters

;  representing a hexadecimal string of 4 - 29 bytes.

; The first byte represents the offset in the packet used

;  for filtering criteria.

; The second and third bytes represent a mask to determine

;  which bridge ports the filter shall affect.

; The subsequent bytes represent the filtering criteria.

; See the following pages for examples.

CUSTOMFILTER   = No Default

; The example below filters all Ethernet packets with a

;  destination address of 02608C010203. The offset of 00

;  corresponds to the beginning of the destination address

;  in an Ethernet packet. The mask value of FFFF prevents

;  packets matching the filter criteria from being forwarded

;  on all bridge ports.

CUSTOMFILTER    = "00FFFF02608C010203"

00                   offset = 0 = dest address

FFFF               Filter on all ports

02608C010203   match value

; The example below filters Ethernet packets with a source

;  address of 02608C010203 from MAC 2, 1 and 0. MAC 1 and 2

;  are the ports corresponding to the first and second drivers

;  bound to NBridge. MAC 0 is the virtual packet capture port.

CUSTOMFILTER    = "06000702608C010203"

06                   offset = 6 = source address

0007               Filter on MAC 2, 1 and 0

02608C010203   match value

; The example below filters all Token Ring packets with a

;  destination address of 10005A010203. The offset of 02

;  corresponds to the beginning of the destination address

;  in a Token Ring packet. Note that the destination address

;  of a Token Ring packet begins at offset 02, after the start

;  delimiter and access control bytes of Token Ring.

CUSTOMFILTER    = "02FFFF10005A010203"

02                   offset = 2 = dest address

FFFF               Filter on all ports

10005A010203   match value

; The example below filters all source routed Token Ring

;  packets with a source address of 10005A010203. Note that

;  the highest bit of the source address is set to accomodate

;  the source routing bit of Token Ring.

CUSTOMFILTER    = "08FFFF90005A010203"

08                   offset = 8 = source address

FFFF               Filter on all ports

90005A010203   match value

SOURCE ROUTING PARAMETERS:

; Internal Ring number for virtual ring connecting internal

;  bridges in Source Routing Token Ring bridge mode.

; Range:  0 - 4095

; Use any valid unassigned Ring Number

INTERNALRING    = 1110

; Ring number for each NIC driver.

; Order identical to BINDINGS.

; Range:  0 - 4095

; If you already have Source Routing Bridges the assigned

;  Ring Numbers MUST match these on Local

RING_MAC        = 1111, 1112, 1113, 1114, 1115, 1116, 1117, 1118, 1119

������������������Ŀ

� SRTOK PARAMETERS �

��������������������

The following pages detail Protocol.Ini parameters for the SRTok

driver. Defaults are shown. Parameter values shown in quotes are ASCII string

parameters. All other parameters are numeric. Numeric parameters preceded with

"0x" are hexadecimal values while all others are decimal values.

REQUIRED NDIS PARAMETERS:

; First driver is "SRTOK$", second is "SRTOK2$", etc.

; Range: SRTOK$ - SRTOK2$

; Only required parameter

DRIVERNAME      = "SRTOK$"

GENERAL PARAMETERS:

; Sets Primary or Alternate adapter

; Do not set both ! (semicolon starts remark line)

; PRIMARY is default

PRIMARY

;ALTERNATE

; Sets adapter Ram address

; Range: 0x0000 - 0xFFFC where low 2 bits = 0

RAM             = 0xD800

; Maximum queued transmits

; Range: 1 - 32

MAXTRANSMITS    = 8

; Sets the adapter product identification

; Range: exactly 36 hex digits

PRODUCTID = "0110F0F0F0F0F0F0F0F0F0F0F0F0F0F0F0F0"

; Enables reception of Single Route Broadcasts

; Range: "YES" or "NO"

SRBC            = "YES"

; Enables Early Token Release feature on 16 Mbps rings

; Range: "YES" or "NO"

EARLYRELEASE = "NO"

IBM ISA TOKEN RING ADAPTER SWITCH SETTINGS:

Off = Up   = U

On  = Down = D

������������Ŀ

�123456789111�  Switch Block

�         012�

��������������

D    (12)       4  Mbps Ring Speed

U               16 Mbps Ring Speed

DD     (10,11)    8  KB Shared RAM size

UD                16 KB Shared RAM size

DU                32 KB Shared RAM size

UU                64 KB Shared RAM size

U       (9)        Primary Adapter

D                  Alternate Adapter

Off = Up   = U

On  = Down = D

������������Ŀ

�123456789111�  Switch Block

�         012�

��������������

DD        (7,8)      IRQ 2

DU                   IRQ 3

UD                   IRQ 6

UU                   IRQ 7

UDDDDD          (1-6)      ROM @ C000

UDDUDD                     ROM @ C800

UDUDDD                     ROM @ D000

UDUUDD                     ROM @ D800

UUDDDD                     ROM @ E000

UUDUDD                     ROM @ E800

������������������Ŀ

� ACKNOWLEDGEMENTS �

��������������������

All trademarks are the property of their owners.

This includes, but is not limited to, the trademarks

of 3Com, IBM, Microsoft, and Hayes.

;