{ PRINTER -- A replacement unit for the standard Turbo 4 PRINTER unit.

Version 1.0 -  1/24/1988 - First general release

Scott Bussinger

Professional Practice Systems

110 South 131st Street

Tacoma, WA  98444

(206)531-8944

Compuserve 72247,2671

This UNIT for Turbo Pascal version 4.0 replaces the standard PRINTER unit in

provided with the compiler and solves some problems related to sending printer

control strings and graphics data to printers using the LST file.

DOS supports two modes of access to files and devices.  These are either

RAW and COOKED modes (or BINARY and ASCII modes, depending on what reference

you are looking at) and control the way that data being written or read is

interpreted by DOS itself. For example in COOKED mode, all tabs that output

are converted into spaces and ^C will stop the program. In RAW mode, data is

passed directly to the file or device with no special interpretation.

The PRINTER unit supplied with Turbo Pascal version 4.0 simply opens a DOS

handle pointing to the printer device and lets DOS use the default COOKED

mode. The problem arises that if you try and send a ^Z (or #26 or chr(26)) in

this mode, all further output to the printer is lost. The reason is that DOS

is interpreting the ^Z as an end of file (EOF) marker and closes the output

device. The solution is simple -- just change the printer file to RAW mode.

Since there is no reason to interpret the codes being sent to the printer

anyway, this works fine.

This unit does just that and then restores the original value of the

COOKED/RAW flag just to be sure no other programs get confused. The easiest

way to use this unit is to compile it to disk and replace the standard PRINTER

unit in the TURBO.TPL file. If you don't at least remove the original PRINTER

unit from TURBO.TPL file, then this unit will never be used.

To replace the PRINTER unit, go to the directory containing all of your

Turbo files and first be sure you are not working with your master disks. ONLY

DO THESE CHANGES TO A COPY OF THE TURBO.TPL FILE! Put a compiled version of

the new PRINTER unit in the directory (that will be the PRINTER.TPU file).

Now run the library utility program as follows:

TPUMOVER TURBO /-PRINTER /+PRINTER

This deletes the original PRINTER unit and adds the new PRINTER unit to the

TURBO.TPL file.  You can also delete the PRINTER.TPU file now.

Having made these changes, you should be able to use the PRINTER unit

exactly as documented in the Turbo Pascal manual, except that it will now work

in all cases.

If you compile and run this DOC file, it will test that you have

successfully compiled and installed the new PRINTER unit. If you run this

program on a machine with a printer attached, you should see TWO output lines.

If run on an un-replaced TURBO.TPL, you'll see only one line. Since the ^Z is

being sent to your printer and this can have different effects on different

printers, you may instead see both lines of information stuck together or in a

different font or something. This is a fine indication and means the fix did

work, but I suggest you turn off your printer to make sure it hasn't been left

in a strange mode. }

program Test;

uses Printer;

begin

writeln(Lst,'This is the first line and will always show up.');

writeln(Lst,^Z'This line only shows up with the new PRINTER unit.')

end.