EMS Emulator Software

The EMS emulator software provides services for converting the

Phantom RAM on the system board to emulated EMS memory. The

software is available from your distributor or from the Wave Mate

bulletin board.

The Wave Mate EMS Memory Manager Emulator conforms to version

4.00 of the Lotus/Intel/Microsoft (LIM) expanded memory

specification. Version 4.00 of the LIM specification is upward

compatible with version 3.20 and will run programs written for

that version of the memory manager. Since the version 4.00 LIM

specification does not support multiple memory managers, this

software <MI>cannot<D> be used at the same time that another

memory manager is being used with an IO expansion bus EMS

adapter card.

Configuration:

There are two separate hardware environments for the EMS

emulator. The standard environment is the BULLET-286ii and

BULLET-286E as shipped from the factory. In the standard

environment the EMS page frame is located in system RAM. Thus, in

a standard EMS installation with a 64K EMS page frame, the user

would lose 64K of system RAM. In the enhanced environment, the

BULLET-286ii hardware is modified to create a 64K page frame at

segment E000, in the Phantom RAM, above the 640K of system

memory. The enhanced environment requires replacing a

Programmable Array Logic (PAL) device on the BULLET-286ii. The

enhanced environment is switch selectable on the BULLET-286XT+.

The enhanced environment is not available on the BULLET-286E.

The PAL is an extra cost option which is specific to the board

assembly number. It is sold only through the Tech Support service

to ensure that the correct PAL is shipped for the target board

assembly number. The PAL is always installed in location U48 on

the BULLET-286ii. Retain the old PAL in case you ever wish to

install an IO adapter which uses segment E000. In that case, you

will no longer be able to use the enhanced environment for the

EMS emulator.

In the standard environment the EMS emulator provides 384K of EMS

memory. In the enhanced environment the EMS emulator provides

320K of EMS memory. The trade-off occurs in the system RAM, since

in the standard environment the user loses 64K of system RAM.

Installation:

The EMS emulator is installed by placing a copy of the software

in the root directory of the boot disk and adding the following

line to CONFIG.SYS:

DEVICE=WMEMM.SYS

Limitations:

Since the EMS is emulated, certain functions, such as memory

aliasing, cannot be done. Since use of this feature is not

common, most software will run without problems. The following

software which uses EMS memory has been tested and found to work

with the EMS emulator on the BULLET-286:

Lotus 123 V2.01

Symphony V2.0

Windows/286 V2.00 (requires enhanced environment)

Publisher's Paintbrush V1.61

DOS V4.01