UARTMON.EXE

Version 1.2

Feb. 18, 1991

For monitoring installed serial ports.

Copyright 1991  Unitech Associates, Inc.

P.O. Box 566

Newark, Delaware 19715

AUTHOR: Dale S. Hoover (CIS 73300,3712)

DESCRIPTION:

This program is designed to monitor the UARTS associated with

serial ports ( COM1-COM4 ).  It is intended to be use simultaneous

with an application using the UART.  It should not interfere in the

monitoring mode although the menus allow the end user to change the

baud rate, data_bits, stop_bits, and parity.  Certain combinations

of data_bits, stop_bits, and parity are not valid and are inhibited

in the menus.  Because the intended use is to monitor the serial port

the serial ports are not opened and the interrupts of are not

modified, tested, or chained.  If no application is using the UART

then the monitor will not reflect valid information ( i.e. the UART

needs to be initialized by an application other than the monitor. )

PRINCIPLE:

In the BIOS DATA AREA of system memory there is a list of serial

port addresses in the 4 words beginning at 0000:0400.  Most systems

use the default 3F8 for COM1 and 2F8 for COM2.  There is no

standard for COM3 and COM4, so you need to make sure the BIOS

DATA AREA correctly reflects the right addresses for COM3 and

COM4 if your machine has these ports.  Windows uses default

addresses as listsed below but these can be altered using the

COMxBase=XXXh directive in the SYSTEM.INI file. ( see SYSINI2.TXT

in you {WINDOWS} directory).

Default Windows 3.0 :           Changed using :

COM1=3F8h                COM1Base = xxxh;

COM2=2F8h                COM2Base = xxxh;

COM3=2E8h                COM3Base = xxxh;

COM4=2E0h                COM4Base = xxxh;

If you encounter any problems with accessing serial ports it would

be a good idea to verify the addresses stored in the BIOS DATA AREA.

This can be accomplished using the "Ports" menu.  There is an option

IO_Address which shows the IO_Address as determined from WINDOWS

default setting or values that are read from the BIOS.  If a COMx

address shows "default" as the source then the BIOS does not know

the address.  It is still possible that software (including WINDOWS

APPS...) can function if it is written to bypass the BIOS DATA AREA

values for the UART IO address.  If a COMx address shows the source

as bios then the value for that port was determined from a read of

the BIOS DATA AREA.

Once the IO address is known for a given UART it is a simple task

to investigate the UART registers to determine the status of the

various RS232 lines.  The monitor gets a WM_TIMER message every

200 msecs and update the screen with any changes in the UART

status.

As a utility this can be very useful when first connecting

a new serial device.  Basically this is a WINDOWS "BREAKOUT Box".

HOW TO USE THE PROGRAM:

Just run it and read the results.

DISTRIBUTION:

UARTMON.EXE and this document are provided free by Unitech

Associates, Inc., Newark, Delaware.  They may be copied and

distributed provided no money is charged for them and they are

not modified.  Although the program has been extensively tested,

Unitech Associates, Inc. provides no warranty concerning its

usefulness on all systems and UA, Inc. accepts no responsibility

or liability for any mishap resulting from its use.

Questions or comments can be answered by :

Dale S. Hoover ( CompuServe ID #73300,3712 )

or at :

Dale S. Hoover

Vice President

Unitech Associates, Inc.

P.O. Box 566

Newark, DE   19715