# West Wind Web Connection For Visual FoxPro

Making Waves on the Web with Visual FoxPro!

 Rick Strahl, West Wind Technologies, 1996-98

Support: http://www.west-wind.com/wwThreads/

Welcome to West Wind Web Connection. This product can connect your Visual FoxPro applications to the World Wide Web in real time, allowing you to take full advantage of Visual FoxPro’s powerful data access functionality for providing dynamically generated Web Content. Hook into the power of Microsoft's Internet Information server with ISAPI and OLE Automation or use any ISAPI or Windows CGI capable Web server to provide blazing backends for your Web applications.

Web Connection provides a scalable Web platform for building fast, powerful, flexible applications with ease. Take your pick between code based HTML creation or template/HTML scripting to generate your output. The supplied classes and tools greatly simplify receiving Web server requests, using the provided information and generating the final dynamic HTML pages to return to the web server.

## What is Web Connection?

Web Connection consists of two major pieces of software that make up the overall product. A small C++ based connector application (ISAPI and CGI) that implements the communication interface between the Web Server and your Visual FoxPro application, and an extensive framework of Visual FoxPro classes that makes it easy to build application specific Web code.

The Visual FoxPro framework consists of classes that manage communication with the connector. They handle server management and administration,  retrieving server and browser form variables from the server making them easily available to your code and providing a powerful set HTML classes and support tools that make short work of interactively building the HTML required to display your application on the Web. Web Connection provides access to all advanced HTML based features including built in support for frame-management and displaying HTML pages containing FoxPro code and expressions from disk. Many advanced features like HTTP Authentication, HTTP Cookies, custom headers, request logging and even built in support for some ActiveX controls are built into the product. And you can use Web Connection with tools like FrontPage or Visual InterDev to design your pages and still use Web Connection to run your FoxPro code inside of them!

There’s ample of documentation (over 30 pages of How to’s, examples and a Step by Step Guide, plus 80 pages of class and tool references) and over a dozen examples that demonstrate a variety of the features available in Web Connection for you to check out and plug into your own code.

Web Connection has been field tested on a number of high volume sites. Web Connection has been run at 248,000+ requests per day at www.surplusdirect.com. This busy site is one of the top commercial sites on the Internet – to view real time performance data you can check out the Web Connection hit log  at www.surplusdirect.com/wwcgi.dll?wwmaint~showlog for yourself.

So read on and get ready to get Webbified! It’s never been easier to build Visual FoxPro Web Applications!

## …and what it isn’t

Web Connection makes it easy to build Web applications, but it will not take an existing application and run it on the Web as is. Work is required to convert the interface to an HTML based design and if your application mixes interface and database logic tightly, existing code might not be easily integrated into your new Web application. On the other hand if your business logic is separated clearly from the interface then reusing business rules and other rule based classes or procedures will be easily integrated into your Web applications. Keep in mind you will have access to almost all of Visual FoxPro’s wonderfully powerful features with limitations on the VFP's GUI features!

Web applications are server based and respond to requests made from a Web page. As such they are not event driven or even really interactive. Rather a request made on your server is typically treated as an entire transaction in and of itself and must be fully self-contained. In the stateless world of the Web transaction based programming is the norm and it’s rather different than typical Visual FoxPro based Windows application programming. In some ways it’s more work to build the interface, in others it’s strangely simple to build straight line processing code that grinds through data and spits out the results or a populated input form without having to worry about all the GUI logic (80% of most apps is spent on GUI issues?).

If you’re new to the Web take a deep breath and read on – there’s lots of info in this document that will clarify the way the Web works. Web Connection makes it easy to leverage your FoxPro skills. Once you understand the basic flow of information over the Web things will fall into place and allow you to be productive very quickly. You can crank out quality high performance applications without having to learn all of the Web technology up front and you'll learn about Web technology as you use Web Connection to build applications.

## What you need to check out Web Connection

- Web Server

West Wind Web Connection is designed to work with any Web Server that supports the Internet Server Application Programming Interface (ISAPI - supported by Commerce Builder, MS Internet Information Server, Purveyor) or CGI 1.2 or later (WebSite, Commerce Builder, Netscape) specification. You will need a copy of a compliant Web Server and have it operational in order to use Web Connection with the example forms provided here.

- Fast Pentium with at least 32 megs (preferrably running Windows NT)

A Web Server is fairly resource intensive so a reasonably fast machine (P75 or better) and at least 16 megs on Windows 95 and 32 megs on NT are highly recommended. Web Connection allows you to run multiple Visual FoxPro sessions in order to process simultaneous ISAPI/CGI requests and more memory will make this process much more responsive. A Pentium is required on the Web server. Windows NT 3.51 with Service Pack 4 or later or Windows NT 4.0 with Service Pack 2 or later is required for OLE Automation to work with NT.

- Web Browser

You also need a Web Browser (preferably the MS Internet Explorer 4) and a copy of Visual FoxPro to run the fully functional shareware version.  Several features of Web Connection take advantage of IE 4.0's Dynamic HTML functionality.

NOTE:  Web Connection is designed for and requires Visual FoxPro 5.0. By default the 5.0 files are installed. If you are running VFP 3.0 you have to copy the two compiled program files SETUP.EXE and WCONNECT.APP (shareware only) from the VFP3 directory into the Web Connection root. VFP 5.0 is highly recommended for better performance, compatibility and stability.Some features may not work fully with VFP 3.0.

- Some HTML knowledge recommended

Finally, it  will be helpful, but not required, to have a basic understanding on how a Web Server and the Internet Server API (ISAPI)/ Common Gateway Interface (CGI) are used to process interactive HTML pages. Keep in mind that these protocols are nothing more than interfaces and are abstracted for you by Web Connection via its easy to use class interface. CGI is a standard protocol that is used by web servers to return information about the server, the browser and form variables used to capture user input. Web Connection uses CGI and its variations (both CGI and ISAPI are variations of the CGI scheme) to deliver the Web server interface to Visual FoxPro.

# Table of Contents

West Wind Web Connection For Visual FoxPro	1

What is Web Connection?	1

…and what it isn’t	2

What you need to check out Web Connection	2

Table of Contents	3

Acknowledgements	5

Ordering and Pricing	6

Pricing	6

Ordering	7

Support	7

Warranty Disclaimer: No Warranty!	7

Web Connection Versions	8

Professional Edition	8

Enterprise Edition	8

Runtime Licenses	8

Shareware Version	8

Getting Started	9

What's provided	9

What to do	10

If things aren't working right	12

Netscape and other CGI Web Servers	12

Installing Updates	14

Upgrades from 2.x versions prior to 2.70	14

Same Version Updates	14

Version 1.x Updates	15

SetServerEnvironment() and SetServerProperties	15

Setting up your calling programs in Process()	15

Modifying your individual application PRGs	15

How Web Connection works	17

Web Connection Components	18

Setting up a Web Connection Server	19

OLE Automation (COM/DCOM)	19

File Based Messaging	20

Implementing the Server	21

Receiving a Web Request	23

Processing Incoming Requests with the wwProcess Class	24

Creating the HTML	26

Creating output quickly with ShowCursor	27

The ShowCursor Method	27

Scripting and Templates	28

Templates - .WC files	28

Scripts - .WCS files	30

Using the Dynamic HTML Rendering for IE 4.0	34

Rendering Forms	35

Updating Forms with data from the Web	36

A couple of things to consider	36

Keeping Track of users with with wwSession	37

Your First Web Request	39

Overview	39

Step 1 - Calling your custom program	39

Step 2 - Set up your custom program	40

Step 3 – Set up your HTML and Datafile paths	42

Step 4 - Add your custom method to the Process Class	42

Step 5 - Check it out from your Browser	43

Step 6 - Go to it!	43

The wc DLL and EXE files	44

How to use it	44

What does it do?	45

The wc.ini file	45

DLL Error Message Formatting	47

DLL Error Log	48

Web Connection Server Management	50

Mangement Functions built into the wwServer Form	50

Maintainence functions built into the ISAPI DLL	51

Online Code Updates	53

Functions provided in the wwMaint Class	53

Functions provided in the wwMaint Class	54

Web Connection Hit logging	54

Displaying the ISAPI DLL Error Log	54

Management of Multiple Sessions for File Based Messaging	54

Server Statistics	56

Debugging Web Connection Applications	57

WCONNECT.H	57

Error Trapping in HTML Expressions	57

Checking HTML output and the INI File input	58

Common Problems	58

Web Server Configuration	60

CGI Temp Files (doesn't apply to ISAPI servers)	60

Troubleshooting	61

Windows NT and TEMP Files	61

Clean up the temp directory	61

Setup for OLE Automation	63

WCONNECT.H Settings for Automation Servers	63

Using the OLESERVER flag	63

When to use OLE Automation	64

Building  your Automation server	64

Testing your server	65

Automation Server Configuration	65

Configuring your server and the DLL	66

When to use Automation	67

Running Web Connection over the Network	68

Dealing with Network Pathing	68

File based Messaging	68

Remote OLE Automation via DCOM	69

Configuring Web Connection Servers to AutoStart	71

Automation	71

If you're always logged on under NT	71

AutoLogon to NT	71

Running without an NT Logon	71

File Based	71

AutoLogon to NT and Startup Group entries	71

The StartFileInstances flag in wc.ini	71

## Acknowledgements

I'd like to thank the following people for their substantial contributions to making Web Connection a better product:

Michael Dyer and John Forsberg at Merisel for always sharing their excellent ideas and for going the extra mile by running the listserver for the Web Connection list. Calvin Hsia from Microsoft for his help on getting the Automation Pool Manager working even though it ain’t FoxISAPI. Bill Budney for his ideas and input when I didn't know what to think of the Internet not too long ago. Randy Pearson for providing his killer CodeBlck class for use in Web Connection. Steven Black for his advice on class design. Markus Egger, Jeff Fleitz, Costas Menico, Jan Dorresteijn  for bug reports, suggestions and generally interesting feedback to get a reality check.

# Ordering and Pricing

## Pricing

The following product and licensing options are available:

| ###### Product | Price |
| --- | --- |
| West Wind Web Connection, Professional Edition | $199.00 |
| West Wind Web Connection, Enterprise Edition | $399.00 |
| Professional to Enterprise Upgrade | $229.00 |
| Limited Runtime Distribution License (50 Server Installations) | $599.00 |
| Unlimited Runtime Distribution License | $1229.00 |
| West Wind HTML Class (including IE4 form class) | $149.00 |
| West Wind Web Monitor | $39.00 |
| ###### Shipping Options |  |
| Handling Charge (applies on every order) | $3.00 |
| Send Floppy Disk or Invoice | $5.00 |
| Send Floppy Disk or Invoice Foreign (outside of US or Canada) | $10.00 |
| Processing Fee for Purchase Orders | $15.00 |
| Email notification and FTP download | FREE |
| ###### Support Options |  |
| Online Support on Message Board (C:\wwapps\wc2\docs\wconnect.doc) All questions directly related to Web Connection are answered within 24 hours. Application and design questions are typically addressed, but officially fall into the priority support below. | FREE |
| Priority phone or email support (½ hour increments billed via credit card) | $120.00/hr |

Licensing for West Wind Web Connection is on a per physical server basis. With a developer  license you get the full Visual FoxPro source code for the supplied classes and utilities and a license to use Web Connection on a single physical Web Server - additional servers or sites require additional licenses. Non-source code distribution licenses for value added applications using Web Connection are available for 50 server and unlimited distribution. The runtime licenses do not include a developer license which must be purchased separately.

## Ordering

You can place an order on the West Wind Web site via secure link at http://www.west-wind.com/register.asp with Visa or Mastercard. Be sure to leave a valid email address which is used to confirm your registration. Instructions where to download the registered copy of the software via FTP will be emailed to you as soon as your credit card is verified. Optional Disk and/or invoice shipments are available. Note: All documentation is included in Electronic Form only, regardless of physical shipment.

You can also register with a credit card payment (VISA/Mastercard) by calling (808) 579-8342 or faxing order info to (808) 579-8547. To register by mail send a check for the order, plus shipping and handling. All payments must be made in US Dollars. Mail to:

Rick Strahl

West Wind Technologies

32 Kaiea Place

Paia, Hawaii 96779

Mail orders are returned by standard mail, but you may specify an email address to send the program to. Email must be an Internet address. Always provide an email address even when ordering disks so that we can send you free update notices!

Purchase orders are not encouraged, but if you must order by PO please provide billing, shipping and an email address. Please highlight the PO Number so it can appear on your invoice. PO's must contain order total and all shipping charges listed for a complete price. POs must be in US dollars. Note: There's a $15.00 surcharge for processing Purchase Orders due to past problems and overhead with PO vendors. This charge is in addition to any shipping and handling charges that may apply. Fax your POs to (808) 579-8547.

Dealer and resale pricing is available only when more than 5 or more copies of the software product have been sold by the reseller. Once 5 copies have been sold cumulatively reseller pricing applies. A 15% reseller discount is given for resale orders.

## Support

Primary support is provided on the West Wind Web Connection Support Forum on the West Wind Web site. You can get there by typing http://www.west-wind.com/wwthreads  into your browser's address line. A Frames capable browser is required for the message board.

URL:		http://www.west-wind.com/wwThreads/

Offline Reader: 	http://www.west-wind.com/wwReader.asp

In addition if you need special attention, if you have problems, questions, comments, suggestions, need custom programming, or application hosting services you can contact me at the following addresses.

Application support that is not directly related to Web Connection installation and operation will be billed at $120.00/hr payable by Credit Card in ½ hour increments. General Web Consulting is also available at these rates.

Phone:		(808) 579-8342

email:		rstrahl@west-wind.com

## Warranty Disclaimer: No Warranty!

IN NO EVENT SHALL THE AUTHOR, OR ANY OTHER PARTY WHO MAY MODIFY AND/OR REDISTRIBUTE THIS PROGRAM AND DOCUMENTATION, BE LIABLE FOR ANY COMMERCIAL, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OR INABILITY TO USE THE PROGRAM INCLUDING, BUT NOT LIMITED TO, LOSS OF DATA OR DATA BEING RENDERED INACCURATE OR LOSSES SUSTAINED BY YOU OR LOSSES SUSTAINED BY THIRD PARTIES OR A FAILURE OF THE PROGRAM TO OPERATE WITH ANY OTHER PROGRAMS, EVEN IF YOU OR OTHER PARTIES HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

# Web Connection Versions

Web Connection is available in several editions to suit different developer needs. A fully functional shareware version is available to allow testing the product prior to buying the software. For more detail on features of each version see Features.doc (Features.doc).

## Professional Edition

The Professional version of Web Connection is the base product that provides the Web server interface and most of the Visual FoxPro application framework functionality to Web Connection. This version features all the connectivity tools, the wwCGI and base wwHTML classes as well as most of the support classes required to build applications using File Based messaging to the Web server. This version is geared towards developers who are first starting to use Web Connection for small to medium sized applications.

## Enterprise Edition

The Enterprise Edition provides several additional features geared for large scale Web site and Intranet development focused on Internet Explorer 4.0. This version adds support for building your application as COM/Automation Servers,  and IE 4.0 VFP to Dynamic HTML Form rendering class which allows you to render VFP forms 'as-is', advanced remote server admin management tools and West Wind Web Monitor as part of the package. This package also includes source code to the West Wind Message Board and Offline reader applications provided as a working, full-sized sample application. This product rolls all Web related products from West Wind into one package.

## Runtime Licenses

If you plan to distribute applications built with Web Connection you may purchase a runtime license. Without a runtime license each new physical server installation requires a separate license. A 50 server and an unlimited server non-source code installation license are available to allow distribution of the Web Connection libraries at a discounted rate.

## Shareware Version

The shareware version of Web Connection is a fully functional version that provides access to all the features of the Web Connection framework and tools, plus the full documentation. It allows you to check out the product's operation to run the samples provided as well as the creation of your own applications to see whether it fits your needs. The shareware version includes several features of the Enterprise edition including the HTML form rendering tools so you can evaluate the advanced features firsthand.  However, due to the fact that this version does not ship with source code a number of limitations abound:

- You cannot build COM/Automation servers since the source code is required for building an EXE/DLL.

- The Web Connection server always operates with DEBUGMODE set to .T. This means any errors in your code will not be handled by the Web Connection error handlers. In the full version setting the flag to .F. allows all errors to be handled by error methods in the framework, automatically generating error messages and logging the errors to the log file.

- A nag shareware screen pops up after ½ hour of continuous server operation. You have to click OK to exit this screen. The nag screen also pops up whenever the WCONNECT.APP, which contains the Web Connection framework classes, is loaded.

- The Web Connection server can only run inside the Visual FoxPro Desktop window. With the full version you can run the server on the desktop as a SDI form.

- None of the configuration options in WCONNECT.H can be changed, since these settings are compiled into the WCONNECT.APP file. With source code all settings can be propagated through the framework.

# Getting Started

I know, I know. You want to get started quickly, so this section describes how to quickly set up Web Connection in a step by step manner.

## What's provided

- Unzip the wconnect.zip file with the -d switch to create the following directory tree:

- WCONNECT 		Main Web Connection Program Files

- This should be your startup directory to work from.

- \CLASSES  	Contains all of the Web Connection Framework class source code.

- Make sure you set PATH TO this directory to find these classes.

- This directory applies only to registered users.

- \ECLASSES	Enterprise Edition classes

- \HTML		HTML and image files used with the examples.

- \SCRIPTS	This directory contains the Web Connection ISAPI DLL and CGI EXE files.

- \TOOLS		Several Utility programs.

- \DOCS		This Word document and the Help file

- \WWDEMO	The sample application and data

- wc.dll or wc.exe

- The ISAPI DLL or CGI Exe connector scripts that are called from an HTML page. A typical URL calls one of  these scripts to send a request to Visual FoxPro for processing. The EXE version is called for CGI compliant Web servers while the DLL is called for ISAPI servers:

- URL address: \cgi-win\wc.exe?TestPage with CGI or \cgi-win\wc.dll?TestPage with ISAPI.

- Wconnect.app  (Shareware Version only)

This is the Visual FoxPro application that contains the Web Connection framework classes in compiled form. This program acts as a loader for all the class libraries. This APP is never run directly except to load the class libraries. The sample program simply calls this APP from wcMain.prg. If you want to use Web Connection classes in other programs you can DO WCONNECT to load the class libraries.

- wcMain.prg

wcMain.prg is the main program that starts a Web Connection server. For the examples wcMain is the mainline program that instantiates a Web Connection server object that handles Web requests. This program handles creating a custom instance of the server class for your particular application. Check out the code to see how to how the demo is set up. You can customize the server for your application.

- wwDemo.prg
wwDemo.prg contains the basic samples that come with Web Connection. WwDemo consists of a custom subclass of the WwProcess class that implements exactly one method for each request incoming from a Web page. The class serves as a request handler application and should handle at least at the highest entry point level all application logic. Check out the code in wwDemo.prg for a clearer idea on how this works.

- wConnect.h - 
A header file that contains all the #DEFINEs used by Web Connection internally. It contains several important status flags like DEBUGMODE, OLESERVER, LOCALSITE which affect overall system operation and error handling. #INCLUDE WCONNECT.H should be included on all of your programs that use any of the Web Connection classes.

Note Enterprise Edition Users:

For information on the Message Board Application located in your .\wwThreads and .\wwReader directories see wwThreads.doc.

## What to do

- Run the Setup.exe program

- In order to properly install Web Connection several options need to be configured. To do so run the Setup program and fill out all information on the Wizard tabs and press Install to complete the installation. The Wizard will update the sample files for the appropriate server type (for ISAPI or CGI), set the proper path information in the HTML, program and configuration files and then copy the samples to the selected directories.

Note Visual FoxPro 5.0 Users:

Web Connection installs with VFP 6.0 files. To update the installation for use with VFP 5.0 you have to start the VFP 5.0 development environment and type:
CD \<WebConnectInstallDir>
DO vfp5\fixVFP5
This program updates Setup.exe, recompiles all source files and runs the setup

- Setup can be run multiple times to update the path information in the example applications without affecting any of your code changes you might have made to the examples. It's recommended that you don't modify the Web Connection sample programs but rather copy them for modification for your own applications so future updates don't overwrite any changes you've made.

- To run the Setup program:

- CD \WCONNECT  (or wherever you installed Web Connection)

- DO SETUP.EXE

- Select a Web Server from the list of Web Servers

- Set the Temporary Files directory.
In order for the Web server to connect with Web Connection a temporary file is created to pass data from the Web server. This option sets where Web Connection expects these temporary files to go. The value specified by the install program represents the default for the selected Web server and should be changed only if the default installation does not work.  
This directory must have READ/WRITE access for the Web Server User Account (IUSR_MachineName for IIS).

- Set the HTML/Script directory
The HTML document directory is the path where the sample HTML documents should be copied to. On MS IIS Website and Commerce Builder Web Connection will automatically create the appropriate directories and virtual  directory mappings to the server. Web Connection will default to create a directory /wconnect off the Web Server's Web root directory. For example: c:\wwwroot\wconnect with a virtual of /wconnect.

This path also holds the Web Connection connector DLL/EXE – wc.dll or wc.exe. While these server scripts can live anywhere the examples run them in the same directory as the HTML pages. The path must be accessible via the Web server's HTTP path or via a virtual directory mapping in the Web server setup. In addition this directory must be set up to allow script execution and must be configured for the appropriate script protocol. 

It's recommended that you keep the default Virtual Path set at WCONNECT or else you will need to manually set the paths to the scripts and related pages to reflect the new virtual path you selected in the demo HTML page.

- Press the Install button to let 'er rip...
Files will be installed and the Web server will be configured. Make sure you stop and start the Web server to make any changes to the server take. Make sure you read the dialog popup when installation is complete.

- Start up the Web Server
Before attempting to run Web Connection make sure your Web server is up and running! Make sure you can get to your default home page by accessing the Web server using the Localhost or 127.0.0.1 address or the IP address that is assigned to your network card if you have one installed. For example: http://localhost/ or http://MyMachine/ or http://111.111.111.111/. 

DO NOT TRY TO ACCESS THE DEMO PAGES FROM YOUR HARDDISK (ie. C:\wwwroot\wconnect\default.htm or by double clicking on the page in Explorer) – This will not work as requests are not going through the Web server.

Note that you don't need to be connected to the Internet, nor do you need to run on a network. Once the server is installed you can run and access Web pages in local mode without an actual connection to the Internet. For example to access the Web Connect demo page you would type http://localhost/wconnect/default.htm into your web browser’s address window.

- Start Visual FoxPro
In order to run Web Connection correctly you need to set up the environment properly. Web Connection needs to be able to find the various support classes which are stored in the Classes directory off the Web Connection Root. The easiest way to do this is by using the include CONFIG.FPW file in your Web Connection root. You can 'execute' this setup by creating a shortcut on the Windows desktop  for Visual FoxPro (c:\vfp\vfp.exe for example) and setting the Start In path to your Web Connection root (c:\wconnect most likely). 
If you have to set up your environment manually you do this from Command Window:

- CD <WebConnectionRootPath>

- DO .\classes\wconnect    (In the shareware version just DO WCONNECT)

- DO PATH WITH ".\classes"

- If you have the Enterprise Edition DO PATH WITH ".\eclasses"

- DO wcMain

This operation starts up the Web Connection server and makes VFP ready to receive requests from the Web server.

Note:
Make sure you are running out of the Web Connection root directory! This directory includes a file WCONNECT.H that is required for properly compiling the demo and class programs. If you get error messages to the effect of  'Can't find class definition WWC_HTML' it means that one of the demo programs compiled without finding WCONNECT.H. If you get this error, CD \WCONNECT (or whereever your WC root is) followed by COMPILE .\WWDEMO\*.prg.

- Start up your Web browser
Move to your local Web site home page by typing http://localhost/ into the address area to make sure your server is operating properly. This should bring up the default home page for the server. Once you are there, load the Web Connection demo page (most likely http://localhost/wconnect/default.htm if you chose the default installation options) and start exploring the sample.

### If things aren't working right

- Setting up a link between the Web server and Web Connection is the most tricky part of using this product, so please take a moment to check the following if you're having problems:

- The Web server is not finding or does not have access to the connector script (wc.dll/exe). 
This problem is happening if you click on a link and you immediately get an error message back from the server stating that wc.dll or wc.exe was not found. To fix, access your Web server's management interface and make sure you have the directory mappings to the wc.dll/exe files set up properly and have EXECUTE rights and appropriate protocol (CGI or ISAPI) in the specified Web virtual directory.

- The Browser immediately returns an Invalid Response or Server Error dialog
Make sure that the your Web server account (IUSR_machine on IIS) account has Change or Full access in your Temp file directory as specified in wc.ini and in the Web Connection server status window!

- Web Connection cannot find the request file, because the Temporary paths are set up incorrectly. 
This is the case if the Web browser shows an hour glass for about 30 seconds or so after clicking a link without Web Connection receiving a request in its window. To fix, make sure that the paths in wc.ini (in the HTTP scripts directory) matches the Temp File setting in the Web Connection server status window. This is done automatically by the SETUP program, but you should double check if you have problems. Shut down and restart the Web server for the changes to take. 
To check this out start Web Connection and click on the Status button and check the Temp File path displayed there. This value should match the setting in the wc.ini file (for ISAPI).

- A request shows up in Web Connection but you get "Cannot create CGI Object"
Are you using the correct server protocol – ISAPI or CGI? Are you running a wc.exe request on an ISAPI based server (you can only run DLL based scripts with ISAPI servers) or are you trying to run wc.exe on a Web server that does not support CGI? Your Web server must support either ISAPI or CGI - Windows CGI is no longer supported by Web Connection! Note that some servers require setting up the protocols for specific directories. See the 'Web Server Setup' section.

- You get an error page back that say this request is not supported.
This is a Web Connection generated error message and means that you're getting to Web Connection's code. This error means that you're not set up to handle a specific request parameter. These parameters are passed (by the demo at least as: wc.dll?Project~Method~Parameters). The Project name is added in the wwDemoServer :: Process() method in the CASE statement. The second parameter is handled by your processing code in the wwProcess :: Process() method and needs to correspond to a method in the class that handles requests. If the class does not exist you get the error.
To fix make sure you have set up an entry in the Server's Process CASE statement and you have a matching method in your custom Process() class. For more info, see the section on Your First Web Request.

### Netscape and other CGI Web Servers

Starting with version 2.80 Web Connection also supports Netscape Web servers or any other Web Server that can operate via CGI. CGI configuration by the Setup program is only partial with only the files being copied, but no configuration settings made on the Web server itself. The following steps describe Netscape installation, but they apply equally for other CGI servers.

- Configure NetScape Server to use CGI and make sure you can run CGI scripts on your server prior to installing Web Connection. You will need to create a virtual directory that allows CGI execution manually and then use Setup to copy the Web Connection files to this directory.

- Run the Web Connection Setup program and select Netscape server.

- The setup program will ask you for a virtual directory to copy the Web Connection Executable and sample HTML files to. This directory must be a path that can Execute CGI scripts (as configured through the Netscape Web Administration interface). The Netscape Virtual Directory setting and Execute rights must be configured manually!!!

- The CGI file installed is NPH-WC.EXE with a configuration file of NPH-WC.INI. All script links you see in this documentation as WC.DLL will have to be adjusted to NPH-WC.EXE. The Setup program automatically changes all the code and sample page links automatically to NPH-WC.EXE, so the demos should run without change.

- CGI servers require a slightly different version of the wwCGI object that handles the differingCGI server variable names. Instead of wwCGI you need to use wwShellCGI (contained in wwPostRequest.prg). The following code change in the startup program is requried to make this happen

*** Load the Web Connection class libraries

DO WCONNECT

*** Load the server - wcDemoServer defined below

goWCServer=CREATE("wcDemoServer")

*** If running a CGI Web server uncomment the following line

goWCServer.SetCgiClass("wwShellCGI")  && Overwrite for Shell CGI Script

Operation with Netscape or any other CGI server does not allow operation as an Automation server since CGI executable load once and immediately unload after completion and thus can't keep an automation server alive.

# Installing Updates

Web Connection is frequently updated and in order to make the most of the product, it's a good idea to install the new updates each time as they come along. There are several update scenarios depending on which version you are using.

## Upgrades from 2.x versions prior to 2.70

Version 2.70 introduces a new messaging mechanism via a new POST method that avoids the traditional INI file, posting data instead in a single URLEncoded string. This feature improves speed of the framework and is implemented in addition to the traditional INI file messaging. If you're upgrading to 2.70 or later it's recommended you install all files including the full WCONNECT.h and wc.ini, wcdll files as well as all the framework classes.

If you decide to do it manually here are the changes that need to be made:

- A new #DEFINE POSTDATA flag in wconnect.h determines whether to use the INI file or URLEncoding. When .T. URLEncoding is used which is the default for a new installation.

- A new key in wc.ini called PostMethod  controls how the DLL creates the request. Use INI or URLEncoded to set the mode. If the key is omitted the INI file method is used.

- Here's how the two go together:

| wc.ini – PostMethod | wconnect.h |
| --- | --- |
| PostMethod = URLEncoded | #DEFINE POSTDATA .T. |
| PostMethod = INI | #DEFINE POSTDATA .F. |

- When running in URLEncoded mode wconnect.h must contain another #DEFINE value to determine the boundary between the POST data and the server variables. It's defined as:
#DEFINE POST_BOUNDARY    CHR(13)+CHR(10)+ "#@$ FORM VARIABLES $@#" + ;
                         CHR(13)+CHR(10)

- There's a new log file format!The new format is optional – it adds logging of the script name in addition the query string for those of you working extensively with script maps. You can turn on the #DEFINE LOGSCRIPNAME flag in WCONNECT.H to .T. to log. Either add the field to your CGILOG table or simply delete the CGILOG table altogether to update the structure automatically.

- There's a new Template mode supported for .WC pages. You can now specify <!-- WCS_TEMPLATE -->at the top of a .WC page (or any page loaded by ShowHTMLFile() or ,ShowMemoPage()) to force Web Connection to use Active Server Syntax (<% %> and <%= %>) instead of the ## syntax for expanded expressions and code. The new flag is optional and ## continues to be the default behavior. The phrase WCS_TEMPLTATE must appear within the first 80 characters of the document.

These changes apply in addition to the following section. If you're installing Web Connection in full the new default is to use URLEncoded messaging.

## Same Version Updates

To install maintainence updates of the same major version, you can pretty much overwrite all of the frame work classes the Web Connection DLL:

- All files in the CLASSES Directory can be overwritten with the new versions. If you subclass Web Connection Framework classes I recommend you create a new directory for your subclasses and store them there.

- Update your version of WC.DLL or WC.EXE in your Web directories.

- Any files in the DOCS, TOOLS, HTML and SCRIPTS directories can be overwritten.

Files that you want to be careful of are:

- wcMain.prg/ini – if you've made changes to this program and configuration file do not copy this file from the installation. If you do this copy the files somewhere else for reference.
Ideally you never want to use wcMain as your main program name. Rather you should always create a new version of wcMain by copying and renaming it to something specific to your application. That way you don't have to worry about overwriting any of your changes come update time.

- wconnect.h – This all important header file for Web Connection frequently changes and should be updated on every update. Unfortunately this also means you need to adjust the 5 or so 'personalized' defines in this file. The main ones to reconfigure are:

- LOCALSITE

- OLESERVER

- DEBUGMODE

- WWC_<Admin Email Settings>

- Any other keys that you manually changed

- If you customize wconnect.h heavily you should use a tool such as WinDiff to see the differences in the versions and add any new key settings manually. All new keys are documented and added with a time header.

## Version 1.x Updates

Updating from version 1.x is a little more difficult as the startup interface has migrated entirely into a class. Version 2 also adds OLE Automation interfaces that make the setup code a bit more complex. However, all of your application specific code should migrate with no or very minimal changes.

### SetServerEnvironment() and SetServerProperties

The first thing that needs to be done is converting the CGIMain program into a class interface as shown by wcMain.prg. Basically, all code that was previously called to configure the server after instantiating oCGIServer has migrated into two class interfaces: SetServerEnvironment() which fires at the beginning of the Server Init() and SetServerProperties() which fires at the end of it. The former should contain any environment settings including establishing full directory state of your application and configuring the location of the application config file (wcMain.ini for example). SetServerProperties() should be used to set any of the server's property values and to set any directory paths etc. When SetServerProperties() is called the server is guaranteed to have changed to its startup directory, so it's safe to add paths here. You should always call THIS.ReadSettings in this method as it reads all of the INI file settings.

I recommend rather than converting CGIMain you start with wcMain (copy and rename both wcMain.prg and wcMain.ini to something application specific) and adjust the code as appropriate with the settings you previously made in CGIMain. For most configurations the setup in wcMain will work without requiring any changes.

### Setting up your calling programs in Process()

Next you need to add your PRG files to the Process() method. This is the same process that was used in CGIMain, except that you need to pass an extra parameter for each call to your PRGs:

CASE lcParameter == "YOURPRG"

do YourPRG with THIS.oCGI, THIS

Each PRG you call needs to have an entry here.

### Modifying your individual application PRGs

Finally each of your application PRGs has to be adjusted so that they can work with OLE Automation. Here's a typical stub header as in wwDemo.prg:

PROCEDURE wwDemo

LPARAMETER loServer

#INCLUDE WCONNECT.H

*** Now create a process object.

#IF OLESERVER

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,,,[WWC_wwHTMLstring])

#ELSE

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,,,[WWC_wwHTML])

#ENDIF

IF TYPE("loProcess")#"O"

*** All we can do is return...

WAIT WINDOW NOWAIT "Unable to create Process object WebConnectDemo."

RETURN .F.

ENDIF

*** Call the Process Method that handles the request

loProcess.Process

RETURN

Note that you need to pass and receive two parameters: A wwCGI and wwServer object. These objects are then passed to your custom implementation of wwProcess:

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,,,[WWC_wwHTMLstring])

where WebConnectDemo is the name of your class.

Everything else in your process class should continue to work as it did before.

# How Web Connection works

- Using Web Connection with Visual FoxPro code is a snap. This product is designed to be easy to use and provides you with the tools to make connecting VFP to the Web as quick and painless as possible. The following examples are taken straight from the demonstration program which is supplied with this demo. The program files are wcMain.prg for the mainline and wwDemo.prg for the example application code.

- The essential purpose of Web Connection is to pass  requests from the Web server to your Visual FoxPro application and back. The following diagram shows how this task is accomplished:

- This diagram shows how Web requests reach Visual FoxPro and your user code. Once your code has control it's responsible for creating an HTML document that gets passed back to the Web Server to display. Remember, you can take full advantage of Visual FoxPro's language to access data, process logic and output HTML including using FoxPro code embedded in HTML files that can be created with HTML editors such as FrontPage. Note that ISAPI is the Web Connection Interface layer which supports messaging through either Automation or File based messaging.

- Web Connection supports two messaging mechanisms: OLE Automation and File based. The following sections describe how each of the mechanisms work. Automation and File based messaging can be used interchangeably. Generally, you’ll use File based for building your application and then deploy your application on the live Web server using Automation.

- Keep in mind that OLE AUTOMATION is not a requirement for deployment of your applications! Web Connection works fine in file based messaging mode and in many ways it’s even easier to administer file based applications without some of the config headaches associated with Automation based installation. Also note, that OLE Automation is supported only for ISAPI based Web Servers (IIS, Commerce Builder, Purveyor, Website etc.). OLE Automation has also not been tested reliably under Windows 95. File based messaging works with any server supported under any Windows operating system .

## Web Connection Components

This diagram shows the interrelation of the Web Connection classes in Visual FoxPro. The wwServer class manages the connection to the Web server, while the wwProcess object is responsible for handling individual requests. You subclass wwProcessto add your user code by creating one method for each separate request. The wwCGI object serves as the input from the Web server providing HTML form variables, the browser 'command line' (Query string) and server and browser data. To generate output the wwHTML object is used. The wwProcess class encapsulates both wwCGI and wwHTML to provide ready to use objects so when your code is called you're ready to use these objects without any further configuration

- Before we dive into the actual operation of Web Connection let’s do an overview of what Web Connection provides and how the pieces fit together:

- wc.dll/.exe – wc.dll is the intermediary 'script' – a small C++ dll/exe - that is called by your HTML forms and passes a request on the Web Connection server that is already running and waiting for requests. HTML forms never call your Visual FoxPro applications directly - every request goes through wc.dll/exe first which then routes the request to the Web Connection server for processing. Once your application has processed the request it returns an HTTP response typically in the form of an HTML document returned to the DLL which in turn sends the output back to the Web server for display on the browser.
In addition to handling messaging with the Web Connnection server the DLL also handles a number of maintainence and server management features especially related to Automation objects.

- wwServer / wwOLEServer Classes - The wwServer class is responsible for handling incoming requests from the Web server and creating a wwCGI object that is passed on to your user code. The server object handles all the messaging with the Web server and both File based messaging and OLE Automation have separate classes with the same interface that deal with differences of each mechanism. 
You will always create a subclass of this class and override at least three methods: SetServerEnvironment(), SetServerProperties() and Process(). The first two are for configuration of the server, while Process serves as an entry point for your code on each request – Process() gets called after the Web server has created a wwCGI object.

- wwCGI Class - This class encapsulates all the information made available by the Web server. This includes the contents of HTML form variables and status information about the Web server and the browser that called it. When the Web Connection calls the Process() Event described above this object has been set and can be accessed with THIS.oCGI. The Process() then goes on to route the request to an external program, passing the wwCGI object and a reference to the the Web Connection server (THIS) to this program. You'll access this object in your code as Request.

- wwProcess Class – A wwProcess  object is used to provide an easy interface object that you can drop your application specific code into. When Process() calls a standalone program this program creates an instance of wwProcess that you create by adding methods to it. Each method in this class handles a particular request from a Web hit. This class is useful, because it provides pre-configured objects for you ready to use – the class contains oCGI and oHTML members that are ready to be used upon entry into your code without any setup. The class also provides solid error handling and error logging, and a number of handy interface methods that make it easy to display generic output. For typical output however you’ll likely use the oHTML member class or the HTML scripting via script mapping.

- wwHTML Class – The HTML class is an extremely powerful tool for easily creating fast, flexible HTML output using a variety of mechanisms. The class provides a simple interface for file or string based HTML creation. The methods range from the low-level Send() method to the ShowCursor() method that displays an entire cursor with one line of code to the immensely powerful ShowMemoPage() and ShowHTMLFile() which can display HTML containing embedded FoxPro expressions or even entire CodeBlocks (courtesy of Randy Pearson’s CodeBlck) right inside of the HTML text. The HTML class provides over 50 methods and allows access to the HTTP header for very fine control over output. Support for HTTP Cookies, Authentication, Keep-Alive and Page Caching is all built in. You'll access this object in your code as Response.

## Setting up a Web Connection Server

When you create a Web Connection application you take a few simple steps: You always create your own wwServer or wwOLEServer subclass that overrides the Process() method of the base class. In addition you also need to override the SetServerEnvironment() method which is responsible for setting up the server's environment settings. To set properties on the server or to adjust paths you optionally can implement the SetServerProperties() methods. These three methods are typically very similar so you can simply cut and paste it from wcMain.prg into your own program or modify wcMain (rename it if you do so updates won't override this file) to fit your needs.

Following is a sample class that sets up a Web Connection server. While it looks like a lot of code keep in mind that most of it is comments and that you can simply cut and paste this class and change maybe two lines of code below to point at the appropriate path and setup file.

## OLE Automation (COM/DCOM)

Note: OLE Automation support is provided only in the Enterprise Edition.

OLE Automation provides a powerful interface for Visual FoxPro 5.0 by making it possible to make any class OLEPUBLIC and expose its interface and functionality to external applications. Web Connection takes advantage of this scenario by using a connector Internet Server API extension to instantiate and manage Visual FoxPro Automation servers. Web Connection applications expose an OLEPUBLIC OLE object to this DLL – namely your implementation of the Web Connection Server Object. The DLL always calls this server object’s ProcessHit() method with a single parameter that contains the name of the INI file with all the server information. File based messaging does the same thing, but ProcessHit() is called from a timer rather directly from an wc.dll Automation call.

With Automation the Web Connection DLL does the equivalent of the following Visual FoxPro code on the first hit:

LoServer=CREATE(“myOleServer”)

LoServer.Show()

LcHTMLResult=LoServer.ProcessHit(lcIniFile)

Once the Automation server is loaded in memory the DLL maintains a reference to it, so on subsequent hits the server is not reloaded and only the call to ProcessHit() occurs. The pool manager in the DLL can manage multiple simultaneous instances of this server so that requests can be processed simultaneously.

If this sounds complicated, don’t worry. Web Connection provides the ProcessHit method for you and handles the incoming request for you automatically. Once the request comes in via the ProcessHit method it springs to live and creates a wwCGI object. This object is essentially a wrapper around the information the Web server provides and is passed to an entry point method called Process().

As mentioned above you should always implement SetServerEnvironment() to configure your server. Automation servers typically start up in the directory they are called from, not in the directory they are located. Hence setting default path is extremely important to establish a base path. While SetServerEnvironment is used to set the server into a predictable state the SetServerProperties() method can be used to set any properties on the server object. You should use this method instead of SetServerEnvironment() to change paths and actual properties of the server as Web Connection internally sets various properties after calling SetServerEnvironment(). Think of the SetServerEnvironment() happening before WC gets control, and SetServerProperties after all initialization is complete.

## File Based Messaging

File based messaging might not be as ‘flashy’ as OLE Automation and not follow the Microsoft line exactly, but it gets the job done and in many ways is much easier to set up and get going than OLE Automation. For this reason I highly recommend you look at setting up and testing the Web Connection examples and your own applications using file based messaging first then switch to Automation (if you still choose to do so after running file based) when you're ready for deployment. If you run the Setup program it'll set up for file based by default. Switching to Automation requires only switching a single #DEFINE OLESERVER in WCONNECT.H and recompiling.

The File based mechanism works by using request files to communicate between the ISAPI wc.dll or CGI wc.exe file and the Web Connection application. When a request is fired by the Web server wc.dll grabs the request and creates a small request file that contains the path to the INI file that was created in the connector. The Web Connection server is running as a form that contains a timer that polls for this request file by looking on disk for a certain file template in a specificed directory that is agreed upon by the DLL and the server.

Once the request comes in via the ProcessHit method it springs to live and creates a wwCGI object. This object is essentially a wrapper around the information contained in the INI file that was pulled out of the request file passed by the DLL. The newly created object is then passed to an event method called Process()  that gets fired and marks your custom code entry point for a request.

With OLE Automation simply Web Connection simply instantiates the wwOLEServer subclass when a request comes in from the Web Server by making an Automation call. Since File based messaging uses a standalone application, you need to explicitly load the server with a mainline program. This simplified code does the trick:

************************************************************************
FUNCTION WCMAIN
******************
***  Function: Web Connect demonstration program.
***    Assume: called from WCONNECT.HTM form test.
***            Web Server is running and calling
***            \wconnect\wc.dll?Program~Method
************************************************************************
#INCLUDE FOXPRO.H

#INCLUDE WCONNECT.H

*** Load the Framework classes

DO WCONNECT

*** Starts up the server and gets it ready to poll
*** for requests. Call Process UDF() on a request
oServer=CREATE("wwServer","Process")
IF TYPE("oCGIServer")#"O"
   =MessageBox("Unable to load the Request Server",;
      MB_ICONEXCLAMATION,"Web Connection Error")
   RETURN
ENDIF

*** This actually puts the server into polling mode
*** Displays a modal window 
oServer.show()

READ EVENTS

*** Done
RETURN

This mainline code simply loads all the Web Connection framework classes into memory and the creates an instance of the wwServer object. Since the wwServer object is a Visual FoxPro a call to the Show() method puts the form onto the screen and starts the event loop that gets the timer ticking and checking for incoming request files.

## Implementing the Server

Ok enough theory – you now have a pretty good idea of what makes up the server. Let's look at what it takes to implement the server. Here's the code:

#INCLUDE WCONNECT.H

#IF OLESERVER

*************************************************************

DEFINE CLASS wcDemoServer AS wwOLEServer OLEPUBLIC

*************************************************************

#ELSE

*************************************************************

DEFINE CLASS wcDemoServer AS wwServer

*************************************************************

#ENDIF

************************************************************************

* wcDemoServer :: SetServerEnvironment

*************************************

*** This method sets up the environment

*** These settings are required! For a more detailed example

*** on how to handle multiple setups see wcMain.prg

************************************************************************

FUNCTION SetServerEnvironment

*** Location of the startup INI file – use THIS.cAppBasePath to point at EXE path

THIS.cSetupIniFile= ADDBS(THIS.cAppBasePath) + "wcmain.ini"

*** Add Application datapaths - any paths required here

DO PATH WITH "WWDEMO"

ENDFUNC

* SetServerEnvironment

************************************************************************

* wcDemoServer :: SetServerProperties

*************************************

***  Function: This Method should be used to set any server properties

***            This occurs just prior to showing the server window.

***            Optional – if you do call it it's recommended you

***            ReadSettingsFromIni().

***            This method call is optional and only required if you

***            want to override any settings or attach any config

***            code otherwise the baseclass handles the call to

***            ReadSettingsFromIni()

************************************************************************

FUNCTION SetServerProperties

THIS.ReadSettingsFromIni()   && Reads from THIS.cSetupIniFile

*** Any settings you want to make to the server

THIS.Caption = " Demo - Web Connection " + WWVERSION

*** You can override anything here, but I suggest you use

*** the INI file for most settings

* THIS.SetTimerInterval(150)

* THIS.SetCGITemplate("XX_")

ENDFUNC

* SetServerProperties

************************************************************************

* wcDemoServer :: Process

*************************

***  Function: This procedure's main purpose is to route incoming

***            requests to individual project PRGs/APPs.

************************************************************************

FUNCTION Process

THIS.oCGI.aCGIParms()   && Create Array of parameters

lcParameter=UPPER(THIS.oCGI.GetCGIParameter(1))  && Get first Parameter (Project)

*** Set up project types and call external processing programs:

*** Think of these as 'projects' that contain many Web request handlers each

DO CASE

CASE lcParameter == "WWDEMO"

DO wwDemo with THIS.oCGI,THIS

CASE lcParameter == "WWTHREADS"

DO WWTHREADS WITH THIS.oCGI, THIS

*** SYSTEM TASKS

CASE lcParameter == "WWMAINT"

DO wwMaint with THIS.oCGI, THIS

OTHERWISE

*** No matching method

Response=CREATE("wwHTMLString")

Response.HTMLError("Unhandled CGI Request",;

"The server is not setup to handle this request: "+lcParameter)

#IF OLESERVER

*** Simply assign to wwServer's output property

THIS.cOutput=Response.GetOutput()

#ELSE

*** File based: We need to create and output to file

Response2=CREATE("wwHTML",THIS.oCGI.GetOutFile())

IF TYPE("Response2")="O"

Response2.Write(Response.GetOutput())

ENDIF

#ENDIF

ENDCASE

RETURN

ENDDEFINE

Most of this code is boilerplate, which means you can simply cut and paste it for each full application server. For a more complete implementation that allows for multiple setups take a look at wcMain.prg and the main server class there.

The two things that will change for each fully self contained server are:

- The paths in wwYourServer::SetServerEnvironment()
You need to set up the startup paths for each server you create. Because Automation Servers typically don't start in the directory of the EXE file the server needs to explicitly do a SET DEFAULT TO <startpath> to find its state. It does this by figuring out the full path for the EXE file. This value is retrieved and the wwServer::cAppBasePath property is set. This value is then used to point at the startup INI file for the application – in this case wcmain.ini:

*** Location of the startup INI file – use THIS.cAppBasePath to point at EXE path

THIS.cSetupIniFile= ADDBS(THIS.cAppBasePath) + "wcmain.ini"

As you can see, if THIS.cAppBasePath points at the wrong directory you will not properly load the startup values, hence it's important to double check the start path in the server's Status form. You can override the default path for the INI file in SetServerEnvironment by modifying the code above as you see fit, but it's generally a good idea to stick the INI file in the same directory as the EXE file.

- The Programs to call in the wwYourServer::Process() CASE statement
Web Connection parses out 'parameters' from the URL command line (the Query String) where the first 'parameter' is the name of the PRG to call. A typical URL is:

/wconnect/wc.dll?MyPrg~MyMethod

MyPrg references the PRG that is to be run. You need to fill each of these programs explicitly into the Process() method's CASE statement. The reasoning behind this is to allow some control of what programs can be executed. Keep in mind each of the PRGs can contain many requests so you'll typically add only rarely to this project list.

- The HTML and Data paths in your Application INI
Each program file should contain at least a pair of entries in the application INI file to point at the HTML and data paths. The startup INI typically has the same name as the mainline EXE/PRG. For example, wcMain.ini contains the following:

[WebConnectDemo]

HTMLPagePath=c:\http\wc\

DataPath=C:\WWAPPS\WC2\WWDEMO\

where WebConnectionDemo is the name of the Class that is responsible for handling requests in the wwDemo program file. 
Servers need to know where datafiles and HTML pages sit in order to properly access these files, since paths cannot always be set up easily to be relative. The values of the INI file can be retrieved with wwProcess::GetAppIniVar().

We haven't gotten to application implementation yet, but keep this setting in mind when we talk about the wwProcess object next.
The DataPath is more or less optional. If you're accessing a DBC container or you explicitly set the path to the data directory then the Datapath won't be necessary. You can also DO PATH WITH "<yourpath>" in SetServerEnvironment to add any directories to the current Path. I find it handy to not use Path statements on applications that store data in a number of different directories and then explicitly use the path with each file. The Datapath key serves this purpose.

## Receiving a Web Request

Up to this point we've only discussed the server object, whose job it is to grab a Web request from the Web server and pass it on to you. To review the server does the following:

- Request hits the wwServer::ProcessHit() method

- A wwCGI Object is created

- The server invokes your wwServer::Process() method

- Your method contains a CASE statement with the Program to run in response to the parameter passed on the URL Query string (ie. wc.dll?MyProgram~MyMethod).

So, at this point we have the server object calling your custom program. Now what?

### Processing Incoming Requests with the wwProcess Class

When the server calls your custom program it passes you two parameters: A reference to the wwCGI object it created and a reference to itself. You can now take these two object references and create a new wwProcess object that will contain all your program logic.

The wwProcess class is mostly a container for other objects in order to simplify access to the other objects throughout the framework. As such only a few custom methods and properties are defined in this class.  The object references are used to create a new Process object which builds references to them and uses them internally making them available for your own routines. The setup code for the UDF and class creation looks like this:

* PROCEDURE wwDemo

LPARAMETER loServer

#INCLUDE WCONNECT.H

#IF OLESERVER

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,"~","+","wwhtmlstring")

#ELSE

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,"~","+","wwhtml")

#ENDIF

*** Call the Process Method that handles the request

loProcess.Process()

RETURN

The code above basically receives the two objects as parameters then creates a Process object and calls its Process() method which is its entry point for Web request processing.

The actual wwProcess class implementation needs to be subclassed by you in order to attach custom processing methods to the code. The minimal wwProcess subclass looks like this:

*************************************************************

DEFINE CLASS webConnectDemo AS wwProcess

*************************************************************

************************************************************************

* webConnectDemo :: HelloWorld

*  URL to arrive here: wc.dll?MyPrg~HelloWorld

************************************************************************

FUNCTION HelloWorld

THIS.StandardPage("Hello World","Hello from Visual FoxPro. "+;

"The current time is: <b>"+Time()+"</b>")

RETURN

*EOF TestPage

FUNCTION OtherRequest

…

ENDFUNC

ENDDEFINE

Each additional request you create with a link from an HTML page (link or Form button that calls wc.dll) needs to have a corresponding method in this class. Your subclass of wwProcess can contain as many methods as you see fit, although it's best to logically break up the size of classes into logical classes stored in separate program files or classes. In order to do so you'd pass a different first 'parameter' on the URL (in the example above MyPrg – MySecondPrg for example) to route to another PRG file that can handle another set of requests using another subclass of the wwProcess object. Remember each additional PRG file will need an entry in the wwYourServer::Process() method's CASE statement.

In this example we're letting the wwProcess base class handle routing the request to our HelloWorld method. However, as you've seen in the setup code above we're actually calling the Process() method of the class. The wwProcess class manages calling your code via its entry point method – the Process() method.

Typically you'll want to override the Process method if you need to do some logic tasks that needs to occur on each hit to your process object. Think of the Process method as the entry point hook that allows you to fine tune how to handle every incoming request before it hits your custom logic. The very basic Process method (which is pretty much what the base class does) looks like this:

************************************************************************

* webConnectDemo :: Process

***************************

***  Function: This is the callback program file that handles

***            processing a CGI request

************************************************************************

FUNCTION Process

LOCAL lcParameter, lcOutFile, lcIniFile, lcOldError

PRIVATE Request, Response

*** And retrieve the HTMLPagePath and cDataPath vars

THIS.cHTMLPAGEPATH = THIS.GetAppIniVar("HTMLPAGEPATH",THIS.Class)

THIS.cDATAPATH = THIS.GetAppIniVar("DATAPATH",THIS.Class)

Request=THIS.oCGI

Response=THIS.oHTML

*** Retrieve the second parameter PrgFile~Method

lcParameter=UPPER(Request.QueryString(2))

DO CASE

*** GENERIC ROUTING BY PARAMETER NAME

CASE !EMPTY(lcParameter) AND PEMSTATUS(THIS,lcParameter,5)

=EVALUATE("THIS."+lcParameter+"()")

OTHERWISE

*** Unhandled method - display error page

THIS.ErrorMsg("The server was unable to respond "+;

"to the CGI request",;

"Parameter Passed: <b>"+PROPER(lcParameter)+"</b><CR>"+;

"Error: <b>Unhandled Method</b>")

ENDCASE

RETURN

Some things you might want to do in your custom Process method is do password validation, checking for Authentication, checking for cookies or other UserIds or anything that basically happens on every or most hits that you don't want to handle on a specific request each time. For example, to check authorization you might use the following code snippet inside of the Process() method:

*** Check if user was previously Authenticated

lcUserName=Request.GetAuthenticatedUser()

IF EMPTY(lcUserName)

*** User hasn't logged in yet

*** create HTTP header to force dialog

Response.HTMLAuthenticate(Request.GetServername())

RETURN .T.

ENDIF

DO CASE

…

ENDCASE

RETURN

Note:

In order to use Authentication in this fashion on IIS you need to enable Basic Authentication in the IIS service manager. Basic Authentication enables the server to respond to a user password request via HTTP. Basic Authentication works only against NT user accounts that are local to the Web server (no domain accounts). On IIS 4 you can configure Authentication at the root level and at the virtual level. The virtual setting overrides the root setting if configured.

The wwProcess object subclass you've created has a built-in error method. The Error method captures all errors that are generated while processing wwProcess code. Since all of your processing code will be implemented using new methods in this subclass the Error method will catch the majority of coding errors. The error method simply returns an error message to the Web Server thus completing the request even though the actual process failed. The ErrorMsg method is a quick way to spit out an HTML error page, or any other quick message in response to unexpected events, either under program control or as a response to an error event. To turn off built-in Error handling you can toggle the #DEFINE DEBUGMODE switch in WCONNECT.H. When set .T. the Error method is not compiled into the code and errors stop in the VFP debugger for you to trace or fix the error. When .F. the Error method gets compiled and saves handles the error.

ENDFUNC

### Creating the HTML

Finally, the routines called from the routing routine need to actually create the HTML document. There are many ways to accomplish this task from manually coding the HTML strings, to using the report writer to output ASCII text files or merging documents with embedded HTML strings. Whichever method you use, you'll find the included wwHTML class handy. The class provides for an easy mechanism of creating and outputting HTML text to a file, by providing a variety of methods to simplify generation of common HTML tags under program control. All class methods can either send their output to a file or return the individual HTML strings as string return values.

The example code uses this HTML class in code to generate the output to illustrate the class' functionality, but you can use any method you see fit to create the document. In its simplest form you can simply use the Write, Send and SendLn methods to output text to the file:

Response=CREATE("wwHTML","TEST.HTM")

Response.HTMLHeader("Visual FoxPro CGI Hello","Page Title","Back.gif")

Response.HeaderText("H1",[Hello World Wide Web World!])
Response.Write([<HR>])
Response.Write([<h2>Local time is:]+TIME()+[</h2>])

Response.Write([<p>])
Response.Write("This page was dynamically generated by the CGI request that you "+;
              " see in your browser's 'Location' or 'Address' line.")
Response.HTMLFooter(PAGEFOOT)

To make it easier to create simple output pages like this, Web Connection includes wrapper.scx which allows you wrap text lines from the clipboard with a leading and trailing string. With this you can simply copy an existing page and generate code on immediately from it. The majority of forms only contain small portions of dynamic data, so this comes in quite handy.

There's much more functionality in the wwHTML class to create links, images, tables, lists and manange memo formatting, merge text with table data etc. The following is an example from the demo, which generates a customer list containing hotlinks to another Web request by querying the supplied TT_CUST customer table:

************************************************************************

* wwDemoProcess:: ClientList

****************************

***  Function: Shows List of clients and allows selection of

***            Client to back fill query form by running another

***            CGI request to redraw the original form.

*************************************************************************

*** Get all entries that have time entries (expense=.F.)

SELECT tt_cust.Company, MIN(expense) AS HasTimeEntries ;

FROM TT_CUST, TIMEBILL ;

WHERE tt_cust.custno=timebill.custno ;

GROUP BY 1 ;

INTO CURSOR TQuery

IF _TALLY<1

=THIS.Error("No Clients available.")

RETURN

ENDIF

*** Create the document header

Response.HTMLHeader("Complete Client Listing","Web Connection Client List","back.gif")

Response.Write("<i><b>*</b> --- Client has time entries to review.</i><p>"+CR)

Response.Write(Response.HREF("/cgi-win/wc.dll?FillQueryForm~","No Selection",.T.)+"<p>"+CR)

SCAN

*** Create a HLINK to another CGI script for each client

lcCGICompany=STRTRAN(TRIM(Company)," ","+")

Response.Write(Response.HREF("/wconnect/wc.dll?FillQueryForm~"+;

lcCGICompany,company,.T.)+;

IIF(!HasTimeEntries,"*","")+;

"<BR>"+CR)

ENDSCAN

Response.Write("<HR>")

Response.Write([<A HREF="/wconnect.htm">Return to the Web Connect Demo form...</A>])

Response.HTMLFooter()

RETURN

For more examples of how the wwServer, wwCGI and wwHTML classes are used in code take a look at the wcMain.prg program file which contains all the examples used for the demo. You can use the examples as templates to get your own applications off the ground.

## Creating output quickly with ShowCursor

To make creation of HTML quick and easy use a pair of wwHTML methods that greatly reduce the time it takes to put together forms.

### The ShowCursor Method

The ShowCursor method can create a table (HTML or <PRE> formatted based on your browser) simply by calling it when a VFP table or cursor is open in the current workarea. All fields and and values are automatically displayed with a single line of code, including column headers and the ability to sum all numeric columns. Using the ShowCursor method looks something like this:

SELECT SUM(hits) AS Hits, ;

TRIM(lower(location)) as Location ;

FROM webdetl ;

WHERE &lcFilter ;

GROUP BY 2 ;

&lcOrder ;

INTO CURSOR Tquery

Response.HTMLHeader("Web Hits for "+lcBasePath,,BACKIMG)

Response.ShowCursor()

Response.HTMLFooter(PAGEFOOT)

USE IN TQuery

USE IN WebDetl

RETURN

That’s it. Create a query, set up a page header then simply show the table with ShowCursor and finally put a footer to the HTML document. Simple stuff. Note that you can embed links simply by having your SQL statement embed the HREF or other codes directly into the query result! For more info on wwHTML::ShowCursor check out the class reference.

If your needs are a little more sophisticated you can use the wwShowCursor class which allows you to customize the way the data is displayed by setting properties on the object. In addition the object provides methods for displaying a single record either in an HTML table or ASCII view for display in your browser or in an Email message for the ASCII view. By properly filtering your data with a query you can often use wwShowCursor to display your data without creating any HTML explicitly!

## Scripting and Templates

When building HTML applications it's often important to use the tools that were designed for building HTML output. Not only are tools like FrontPage, Visual Interdev and HomePage specifically designed to create HTML pages, but if you are working with HTML Designers on an application those types of tools are what are typically used. In addition using scripting/templates makes it possible to change the look of a page and even some of the display logic without having to recompile your application or even change any code at all.

Scripting and templates in Web Connection allow you to use static HTML pages stored on disk (or in a memo field) and mix them with Visual FoxPro expressions, codeblocks and even entire programs. Pages usually contain plain HTML mixed with special tags that hold the FoxPro specific code. Pages may contain embedded references to field names, variables and object references that were previously declared (PRIVATE or PUBLIC), user defined function or class method calls as well as blocks of code that can be embedded into the HTML.

Web Connection gives you several options in how to access template pages either by loading them at the end of a code based routine using an explicit call to the wwHTML::ShowHTMLFile() or by using a script map to directly run the script or template without having to write any code in the framework directly.

### Templates - .WC files

A template is a file that page that contains both HTML and embedded FoxPro expressions and Codeblocks. The key is that a template is evaluated! What this means the file containing the template text is read in and then parsed for the expressions contained within it. Web Connection expands each of the expressions and then continues on with the next one thus building up a dynamic document. Web Connection supports two types of templates: Old style ## scripting or Active Server type scripting.

#### Active Server Style Templates (<%= %>)

ASP style templates are new to Web Connection with Version 2.55 and later and require a special header tag called WCS_TEMPLTAE at the top of the page. You can embed any FoxPro expression into a page with <%= Expression %>. The expression can contain any valid FoxPro expression, including field names, PRIVATE variables or object references, Function and Method calls. Expressions should preferrably evaluate to a character value for best speed, although Web Connection will convert any type and embed the character result. You can also embed a block of code with <% CodeBlock %>. The block of code can be a single FoxPro command or assignment or entire block of code that includes multiple statements. Here's an example of this looks like:

<!--- WCS_TEMPLATE --->

<html>

<body>
<!-- Some Fox expressions --->

FoxPro Version: <%= Version() %>

Time: <%= Time() %>

<!-- You can call UDFs() assuming they're available in the call stack

expressions should return a string which is embedded in the page

-->

<%= BuildMyCustomTable() %>

<%  *** A code block

PRIVATE lcOutput

lcOutput = ""

SELECT * FROM wwDemo\TT_CUST INTO CURSOR Tquery

SCAN

lcOutput = lcOutput + Tquery.Company + "<BR>"+CHR(13)

ENDSCAN

** You can return a string which is embedded

return lcOutput

%>

<hr>

</body>

</HTML>

#### ## Templates

When Web Connection was originally created it predated Active Server Pages and many other tools with its capability to expand code dynamically inside of script pages. The original scheme used relied on a a single set of delimiters for both beginning and ending a statement. With this syntax you use ## Character Expression ## to embed any FoxPro character expression, function result into a page. The expression must return a string or else an error message is embedded in its place. To embed a block of code you use ##FOXCODE Codeblock ##. FOXCODE is a keyword that must follow the ## followed by the actual block of code to run and evaluate. The block of code must return a character string or an error is embedded instead of the result.

Although currently Active Server type scripting requires the WCS_TEMPLATE tag at the top of the page, it's recommended you use this style of scripting. The ## syntax will gradually be faded out and is provided only for backwards compatibility.

In general templates work best when they're combined with a code based request. For example to display an inventory record you might do something like this in a code request:

* wwDemoProcess :: ShowItem

FUNCTION ShowItem

lcItemNo = Request.GetFormVar("ItemNo")

IF !USED("Inventory")

USE INVENTORY IN 0

ENDIF

SELE Inventory

LOCATE FOR itemno = lcItemNo

IF !FOUND()

THIS.ErrorMsg("Sorry, item not found")

RETURN

ENDIF

THIS.oHTML.ShowHTMLFile(THIS.cHTMLPagePath + "ShowItem.wc")

RETURN

The HTML page (ShowItem.wc) should then look like this:

<!--- WCS_TEMPLATE --->

<html>

<TITLE>Windance Item Profile</TITLE>

<body bgcolor="#ffffff" text="#000000">

<P>

<img align=LEFT HSPACE=10 src=itemgifs\<%=Inventory.itemcode%>.jpg></p>

<P>

<pre>

Item Code: <%= Inventory.itemcode%>

Year: <%=Inventory.year%>

Description: <%=TRIM(Inventory.Descrip)%>

Size: <%=Inventory.size%>

Condition: <%=Inventory.condtn%>

Price: <%=TRANSFORM(Inventory.askprice,"$$$,$$$.99")%>

Comments: <%=TRIM(Inventory.Comments)%>

</pre><p>

<FORM  ACTION="wc.dll?wwSales~Addtem"    METHOD="POST">

<INPUT TYPE="SUBMIT" NAME="btnSubmit"

VALUE="Add item to to Shopping Cart" SIZE="40">

</FORM>

<HR>

<br></BODY>

</HTML>

Note the use of the WCS_TEMPLATE tag at the top to support Active Server syntax!

### Scripts - .WCS files

Like a template a script is a page that can contain both HTML and embedded FoxPro expressions and Codeblocks. However, scripts are actually converted into Visual FoxPro code that is actually run as a program. This difference may not be all that obvious until you see the differences. In a script page you can have FoxPro structured expressions like SCAN, FOR, WHILE, IF and CASE and put HTML in between the iterations or conditions. Since this page is converted into a running programming these loops iterate and expand any text between the commands. For example you can do this:

<% SELECT * FROM wwDemo\TT_Cust INTO CURSOR TQuery

SCAN %>

Name: <%= Tquery.Name%><br>

Company: <%= Tquery.Company%><BR>

<%IF Sex = "M" %>

Sex: Male

<%ELSE%>

Sex: Female

<% ENDIF%>

<p>

<% ENDSCAN %>

Scripts are implemented in Web Connection via the wwVFPScript object which provides the mechanism to read a page from disk, parse into a program and actually run that program. Behind the scenes the code goes out and creates a TEXTMERGE document from the HTML/script tags which is then merged and sent back to the Web server. The wwVPFScript object can be called directly from your code, or preferably be activated directly through a script map.

The wwVFPScript object also implements basic imitations of the Active Server Request and Response objects. Thus you can use Request.ServerVariables("User-Agent") to retrieve the browser of a user and send strings to output with Response.Write().

Web Connection implements a .WCS (Web Connection Script) script map in IIS for running scripted pages. Anytime you load a .WCS file through the Web server the Web Connection DLL is called and asked to process this page. Web Connection complies and uses the code in wwScriptmaps.prg to figure out the name of the script and then proceeds to load it and run it. The script handler also recognizes .FXP files as compile Web Connection scripts! When running an FXP file directly via the URL all compilation is bypassed even in the development version.

Important Scripting Issues

- The current implementation of Web Connection implements scripting via compiled program files.

- When running in the development environment a page is converted every time and then run as an on the fly compiled program. Pages are 'reevaluated' on every hit – if you make a change in the script file the change shows immediately after the file is saved.

- In the runtime pages are run from the compiled .FXP file. This means that pages must either be run through the runtime once, or be precompiled. This means until a .WCS page is compiled a change to the page is not reflected online.

- The maintainence page contains a link that compiles either an individual script or all scripts at once. In order to compile pages a development version of VFP needs to be available (it uses the VisualFoxPro.Application COM object to compile).

- Since pages are compiled they run at native VFP speed! They're pretty fast especially in the runtime!

- wwScriptMaps contains a switch property called lUseCodeBlock. If set .T. you can run runtime pages as interpreted pages using CodeBlock. Since CodeBlock evaluates each line one by one this option is much slower, especially on iterative code. Not recommended for heavy usage.

- Templates are generally quick, scripts are generally slower. Templates only evaluate pages, while scripts run the page.

- Keep any codeblocks to a minimum. If you find yourself writing a lot of code in scripts think about creating UDF() or objects in VFP instead and call those from the scripted pages – it'll be much easier to maintain that way!

You can also use a visual editor such as FrontPage and directly embed expressions inside pages, fields and headers.

If you're using a visual editor such as Frontpage for page creation:

- Whenever possible use HTML Markup to enter any code. Frontpage will leave the markup formatted exactly as you create it.

- Don't use double quotes (") as a string delimiter, but rather use square brackets ([]) or single quotes('). Frontpage and other editors translate double quotes into an HTML translation that will break your code. Square brackets and single quotes can be used safely.

- Realize the FrontPage's WebBots don't work on files that do not have an HTM or HTML extension. This includes .WC pages which don't work. Thus, if you plan on using WebBot's with your .WC type pages don't name them with a .WC extension, but rather .HTM, .ASP, .HTX etc.. The .WC is not a requirement  for Web Connection unless you use script mapping to 'execute' .WC pages directly.

The code required to display this page looks like this:

***********************************************************************

* wwThreads :: ShowMsg

**********************

***  Function: Displays a message in a table view.

************************************************************************

FUNCTION ShowMsg

LOCAL lcMsgId, ldDate, loUser

lcMsgId=Request.GetCGIParameter(3)

*** Now locate the actual message

IF !USED("wwThreads")

USE (DATAPATH+"wwThreads") IN 0

ENDIF

SELE wwThreads

LOCATE FOR MsgId=lcMsgId

IF !FOUND()

THIS.ErrorMsg("Can't Find Specified Message",;

"The message that you specified is not available to view at this time...")

USE IN TQuery

RETURN

ENDIF

Response.ShowHTMLFile(HTMLPAGEPATH+"ShowMsg.wc")

USE IN TQuery

ENDFUNC

* ShowMsg

Again the code to display this page is minimal. Since the stored page contains all HTML code and the ShowHTMLFile() method automatically adds a content type header, a single VFP method call will create the page described above. All that happens is code is locating the record pointer on the valid record to display in the HTML page and the page evaluation handles filling in the expressions in the call to ShowHTMLFile(). Pretty cool, eh?

Script Mapping - Executing .WC and .WCS files without Visual FoxPro methods

Web Connection also supports a mechanism called script mapping that allows you to execute a .WC template pages and .WCS script pages generically without having to write any code in Visual FoxPro itself. Rather you can use code and expressions directly inside of your HTML pages to 'execute' the page directly.

A script map is nothing more than a server extension map that tells the server to go out an look for an executable instead of directly running the file it found. In the case of Web Connection the default installation installs  .WC and .WCS script maps that map to the /wconnect/wc.dll file. So the following links are identical:

/wconnect/wc.dll?12341~ShowItem

wc.wc?12341~ShowItem

Let's see why this is significant. Let's say you're running your application from the Web Root directory or "/". Typically when you call the an ISAPI or CGI request, the server path is changed to the directory in which the ISAPI extension lives - in this case /wconnect. Now if you need to reference an HTML page you need to use a relative or absolute path to reference any images or links to other pages by explicitly using something like "../images/free.gif" or "/localpage.htm ".

With the script map in use in the second example, none of this path changing takes place - the server stays in the same directory that the request was generated from, so you now reference images just like any other HTML page with "images/free.gif" or local pages as "localpage.htm". This way you can use a single copy of your wc.dll from any directory without having to worry about any relative pathing issues.

The cool thing about a script map is that it executes any .WC extension file by calling the ISAPI DLL. So, you can do something like this:

Localpage.wc

where this page page contains HTML containing the scripting syntax described above to tie FoxPro logic to the page. When this page is accessed from the browser the server executes /wconnect/wc.dll. One of the parameters it passes back to Web Connection via the wwCGI object is called Physical Path (retrievable with wwCGI::GetPhysicalPath()) which in this case will contain the actual path to Localpage.wc: C:\http\LocalPage.wc for example.

That's just what we need to be able to interpret this page. Web Connection can now generically 'execute' this HTML page with a tiny snippet of code in wcMain.prg which routes to wwScriptmaps.prg:

lcPhysicalPath=Request.GetPhysicalPath()

IF ATC(".WC",lcPhysicalPath) > 0

Response=CREATE("wwHTMLString")

poCGI=Request

poHTML=Response

Response.ShowHTMLFile(lcPhysicalPath)

ENDIF

#IF OLESERVER

*** Simply assign to output property

THIS.cOutput=Response.GetOutput()

#ELSE

*** Here we need to create and output to file

Response2=CREATE("wwHTML",Request.GetOutFile())

IF TYPE("Response2")="O"

Response2.Write(Response.GetOutput())

ENDIF

#ENDIF

This code is generic and already built into Web Connection's wcMain.prg Process() loop. Anytime you call a .WC page from the browser the page is evaluated and displayed. For an example run the Scripted HTML sample look at the NoCode.wc page demo on the samples page.

Scriptmap handling is implemented in wwScriptMaps.prg which is a generic handler for .WC and .WCS files. This subclass of wwProcess makes it possible to run the script map code inside of the regular wwProcess environment providing access to the standard error handler and all of the objects used in a typical process request – in other words template pages run very similar to the way you'd write a standalone Web Connection request method.

The following PRIVATE objects are available to use in any script or template page:

poProcess = THIS

poSession = THIS.oSession

poServer = THIS.oCGIServer

poHTML = THIS.oHTML

poCGI = Request

For example your scripted page could do:

Query String: <%= poCGI.QueryString()%>

Be aware that the HTML object should not be used to send output do the output source since scripting builds output separately prior to sending the entire block to the HTML output source. Any HTML methods called should set the llNoOutput parameter to .T. so the string result is returned. For example, to ShowCursor you have to do this:

<% SELECT * FROM TT_CUST INTO CURSOR Tquery %>

<%=   poHTML.ShowCursor(,,,,.t.) %>

### Using the Dynamic HTML Rendering for IE 4.0

Note: This feature is available in the Enterprise Edition only and requires IE 4.0!

Wouldn't it be nice if you could take an existing Visual FoxPro form and run it, pretty much as is over the Web including data access on the server? Well, you can with the IE 4.0 form rendering class. This mechanism can take a running Visual FoxPro form and render it as Dynamic HTML 'as is'. Most common interface elements are supported including images, pageframes and grids in read only mode. Note that IE4.0 is the only browser that can display these generated forms!

Hint: To quickly test a form to see how it will look in IE4 interactively without running it through the Web server simply run your form and then do:

DO FORM wwDemo\Guest NAME oGuest LINKED

DO WWFORM WITH oGuest

For detailed class information see the wwHTMLForm class section in Classes.doc. Following is an overview of what you can do with this class.

### Rendering Forms

This rendering engine is implemented as a set of objects that work with an wwHTML object. When you stick this type of code into a Web Connection request only a few more lines of code are required:

************************************************************************

* wwDemoProcess :: IEFormDemo

*********************************

***  Function: Demonstrates a simple implementation of HTML Form.

***            Shows off use of Page Frame and data bound listbox.

************************************************************************

*** All buttons are *ALWAYS* tied to btnSubmit

lcAction=UPPER(Request.GetFormVar("btnSubmit"))

lcListValue=Reqiest.Form("PageFrame1.Page2.lstCustomers")

DO FORM tt_cust.scx NAME oCust LINKED NOSHOW

Response.HTMLHeader(,"HTML VFP Form Rendering Demo")

Response.Write("<CENTER>")

oHTMLForm=CREATEOBJECT("wwHTMLForm",oCust,Response)

oHTMLForm.lShowAsFullHTML=.F.

oHTMLForm.lShowFormCaption=.T.

oHTMLForm.lAbsolutePosition=.F.

oHTMLForm.cFormAction="/wconnect/wc.dll?wwdemo~IEFormDemo"

*** Handle Grid Selection

IF !EMPTY(lcListValue)

oCust.Navigate("SEARCH",lcListValue)

ELSE

oCust.Navigate()

ENDIF

oHTMLForm.ShowContainer()

Response.Write([</center>])

Response.HTMLFooter(PAGEFOOT)

ENDFUNC

* IEFormDemo

The crux of this code is that it creates a form that contains data and displays it over a Web Connection request. The key to this request are  the following lines:

oHTMLForm=CREATEOBJECT("wwHTMLForm",oCust,THIS.oHTML)

The first line creates an HTMLForm object from the form that was created by DO FORM. A reference to this form is passed to the Init of the new object. You also pass in an HTML object so that output is automatically generated to Web Connection's default output object. At this point, you've only set up a reference to these objects – nothing has happened yet.

After the object is created you typically will customize the way the oHTMLForm object behaves by setting various properties.

Above I'm basically setting the renderer to display a form with its caption (lShowCaption=.T.), that is embedded inside of a larger HTML document (lShowAsFullHTML=.F.) displaying the form centered rather than in its absolute position (which may be useful if you want to render more than one form at once).

The other important thing here is that you need to navigate your form to the appropriate location, prior to rendering it. The example above allows you to select an item out of the list on the second page then renders that new updated record when you click the update button.

*** Handle Grid Selection

IF !EMPTY(lcListValue)

oCust.Navigate("SEARCH",lcListValue)

ELSE

oCust.Navigate()

ENDIF

Notice the oCust.Navigate() call which handles this by telling the form to go to the specified record. How you accomplish form state navigation and displaymode (Editmode, ViewMode etc) is left up to you, but a couple of examples are provided in the IEGuest and IECustInfo sample requests that demo this technology.

Finally, when your form is all set up properly you can force the HTML to be rendered to the HTML object you specified. The call to ShowContainer does this:

oHTMLForm.ShowContainer()

You can also render more than one thing at a time (be aware that this can get slow if your forms are big). Lets say you want to show three records in a row:

oHTMLForm.ShowContainer()

oCust.Navigate("NEXT")

oHTMLForm.ShowContainer()

oCust.Navigate("NEXT")

oHTMLForm.ShowContainer()

If Next goes to the next record in the list you'll see three forms in a row inside of this single document.

### Updating Forms with data from the Web

Rendering is fine, but what about updating data into a form as well? Yes, you can using the SetValues() method of the oHTMLForm object. The following example out of the IECustInfo demo shows how easy this is:

IF lcAction="SAVE"

oCust.Navigate("CUSTSEARCH",lcName)

oCust.EditMode(.T.)

*** Now update the form values

oHTMLForm.SetValues(Request)

*** And tell the form to save it

oCust.SaveEntry()

ENDIF

In this example, the code checks an HTML form variable value off a button for a SAVE request. In this case, the form is relocated to the particular record that's being edited and the form is set to EditMode (fields can be updated and are editable on the form). The you simply pass the wwCGI object to the SetValues() method and it goes through the object and matches the  retrieved HTML Form variables to those on the form updating the data. If the fields are databound the database is updated accordingly.

Of course, this example simplifies a bit. You'd need to have error handling on this code – most likely you'd have a form Save() method that save variables and returns appropriate error codes or collections that let you know exactly if and where a save operatoin violated a business rule.

### A couple of things to consider

This is very neat technology that's extremely powerful for quickly moving content to the Web. This applies especially to IntraNet applications where the IE 4.0 Browser issue can be forced. However, there are a couple of things you should consider carefully:

- Internet Explorer 4.0 Release version is required. This stuff will show scrambled objects on other browsers!

- The process of rendering is relatively slow since you have to bring up the form and usually perform several refreshes on the form, then walk through all the objects to render it. If speed is your goal, carefully test your forms and keep them simple!

- Grids are read only.

- Grids require an ActiveX control that must be downloaded. The control is safe and signed (and is included with VB source code), but requires the VB5 runtime, which is automatically download from Microsoft (1.5 megs). On the other hand IE 4.01 ships with the VB runtime.

- ActiveX controls on your VFP forms are not supported.

- Grids are rendered only based on Alias/View rowsources

I'm finding that I am implementing all of my maintenance routines using this technology these days. It allows me to build rich interfaces in GUI versions first in the time that it would usually take me just to design the form in FrontPage or other HTML editor. The other benefit is that can have a standalone version and a Web version with the interfaces supported transparently.

For mainline applications the rendering speed can be an issue. But even so, you can also use the class to render to a static file and then customize the static page to be loaded from disk. If nothing else you can use this tool as a WYSIWYG HTML form editor and not use the Web Connection backend to generate the form.

### Keeping Track of users with with wwSession

By their nature HTTP requests are stateless, which means that each hit knows about any of the ones that proceeded it. Hence it’s often difficult to keep state for a particular user on a site and some of the information that was previously provided. Web Connection includes a Session object that can manage a state table for you via HTTP Cookies. Don’t worry you don’t have to understand what these terms mean – they describe a mechanism for managing state between request via a token that is stored on the browser and a VFP table that contains information that you store about this user. A session is really a User context or a user session or visit. For example in an online store, you’d want to track a shopper through a site so that you can keep track of what was placed into a shopping basket. The session object makes this job easy. The session object also makes it possible to dynamically attach string values to the session object.

To initialize a Session object call the wwProcess object’s InitSession() method. It will automatically retrieve the client token from the browser and create a Session object that you can use to retrieve values from. For example, to retrieve the SessionId for a user, which you might use as a key for a Shopping Item List you’d use code like the following:

FUNCTION MyProcessMethod

lcItem = THIS.GetCGIParameter(3)  && ItemCode

lnQty = THIS.GetFormVar(“Quantity”)

*** Retrieve user’s Session Id

lcSessionId = THIS.InitSession()

*** Use as a key to identify user’s items in shopping basket

INSERT INTO ShopCart (SKU,Qty,UID) VALUES (lcItem,lnQty,lcSessionId)

*** Set User properties

THIS.oSession.SetSessionVar(“MyOwnVariable”,”Stored Value”)

*** Retrieve a value we previously stored when user entered his state

lcState = THIS.oSession.GetSessionVar(“State”)

THIS.StandardPage(“Thanks for your order”,”Your order will be shipped to “+lcState+;

“tomorrow”)

Once the call to InitSession has been made you can access the session object from anywhere. Where you make this call is up to you. If the session is required on each hit, it’s a good idea to use the wwProcess::Process() method to automatically initialize the session, so the object is available on each hit without the explicit call.

The session table is stored in wwSession.dbf, and can be extended with your own fields. You can retrieve any field value with the GetField() method and supplying the name of the field to retrieve. Use this to add your own usage statitistics for visitors. Fields make more sense if you need to report on the data captured as retrieving session variables is rather slow as a lot of string parsing has to occur to retrieve values. For example, if you were to track how many hits a user incurs on a site per visit you’d want to run a query against that at some point. Using a field is much easier to work with for a SQl statement.

Note:

Because wwSession works in conjunction with the wwProcess::oHTML object, it's important that you are careful when creating your own custom HTTP headers in order to make sure the cookie is sent/retrieved properly on each request. To do so you must follow the following rules:

- Make sure you use ContentTypeHeader(), HTMLHeader(), ShowHTMLFile/ShowMemoPage() or

- If you use an wwHTTPHeader object, make sure you pass a reference to the existing THIS.oHTML object  to the header to send output directly to the HTML object instead of returning a string with GetOutput():

loHeader=CREATE("wwHTTPHeader",THIS.oHTML)  && Forces output written now!
loHeader.DefaultHeader()
loHeader.AddCookie("SomeOtherCookie","Value")   && Add another cookie
loHeader.CompleteHeader()

# Your First Web Request

Once you've got the example application up and running, you're ready to start building your own customized Web request handlers. Web Connection makes it easy to get started by building on the provided examples. Make sure you can run the wcMain.prg/default.htm demos before following the steps below!

### Overview

Here's a diagram of how Visual FoxPro code flows within the Web Connection:

The basic idea is that the wwServer calls your code when a request is pending. In a nutshell you will be adding methods to a subclassed version of the wwProcess class for each request you want to respond to.

The wwServer object receives a request created by the wc.dll/exe script and calls the Process function in the WCMAIN program with a CGI object as a parameter. The Process function in turn routes the request to a PRG of your choice passing along the CGI object reference. This PRG should consist of two pieces: A subclassed version of the wwProcess class that contains your custom process methods and a small stub program at the top that creates this class and calls the Process method.

The easiest way to get started is to use one of the example programs as a starting point. I'm going to use CGIMAIN.PRG as my main program for the examples here, since this setup allows the most flexibility.

### Step 1 - Calling your custom program

Ok, let's get started. Open up WCMAIN.PRG and find the wcDemoServer::Process() method which follows the initial setup code for the server.  There you'll find a CASE statement that is responsible for calling other PRG files that contain a set of processing commands. The code looks like this:

Request.aCGIParms()   && Create Array of URL 'parameters'

lcParameter=UPPER(Request.GetCGIParameter(1))  && Get first Parameter (Project)

DO CASE

CASE lcParameter="WWDEMO"

DO wwDemo with Request, THIS

*** ADD YOUR PROGRAM NAME IN HERE ***

CASE lcParameter="MYCODE"

DO MyCode with THIS

ENDCASE

Add the code for the MYCODE as indicated above. By adding this code into the process routine you now have a hook that will call your own custom program from any URL that provides a first parameter of MyCode:

http://www.west-wind.com/wconnect/wc.dll?MyCode~MyFirstWebRequest~OtherParameter

The first 'parameter' on the query string following the ? indicates the program that we want to call. This directly corresponds to the lcParameter="MYCODE" CASE expressions which is responsible for calling your program.

How are we retrieving that first parameter from the query string? The GetCGIParameter() method of the CGI class accomplishes this with ease by parsing out the parameter strings for easy access. We'll use this method again to retieve the remaining parameters from the query string.

You only need to do this CASE insertion step once for each program module that you create. Each PRG file can contain many request handlers inside of your custom wwProcess class, so you only need to add to the CASE statement when adding additional PRG files with separate wwProcess classes.

### Step 2 - Set up your custom program

In the WWDEMO directory there is a template program provided, appropriately named MYCODE.PRG. This PRG contains a program header that sets up a wwProcess class and calls its Process method:

************************************************************************

*PROCEDURE MyProg

******************

***  Modified: 06/15/96

***  Function: Processing Template for new processing programs

***    Return: nothing

*************************************************************************

LPARAMETER loServer

#INCLUDE WCONNECT.H

*** Now create a process object. It's not necessary

*** to use an object here, but it makes error handling

*** document and CGI handling much easier!

#IF !OLESERVER

loProcess=CREATE("MyCodeClass",loServer.oCGI,loServer,,,"wwHTML")

#ELSE

loProcess=CREATE("MyCodeClass",loServer.oCGI,loServer,,,"wwHTMLString")

#ENDIF

IF TYPE("loProcess")#"O"

*** All we can do is return...

WAIT WINDOW NOWAIT "Unable to create CGI Processing object..."

RETURN .F.

ENDIF

*** Call the Process Method that handles the request

loProcess.Process()

RETURN

The code above is responsible for taking the incoming request and creating a wwProcess object. The wwProcess object is the container for all of your custom routines that you will create to handle various Web requests.

MyCode.prg also provides an empty shell class for the wwProcess class:

*************************************************************

DEFINE CLASS MyCodeClass AS wwProcess

*************************************************************

***    Author: Rick Strahl

***  Modified: 02/11/97

***  Function: Empty Template to add custom methods to

***

***            Subclassed from a generic wwProcess class

***            handler which provides error handling and

***            HTML and CGI object setup.

*************************************************************

*** Properties defined by wwProcess Parent Class

*** -----------------------------------------------

*** oCGI=.NULL.

*** oHTML=.NULL.

*** Methods defined by wwProcess Parent Class

*** --------------------------------------------

*** Init(oCGI)     && Initializes and checks HTML and CGI objects

*** Process        && Virtual Method that is always overridden

*** Error          && Handles errors that occur in the Process code

*** ErrorMsg(cErrorHeader,cMessage)  && Quick Message Display

************************************************************************

* MyCodeClass :: Process

***************************

***  Function: This is the callback program file that handles

***            processing a CGI request

***    Return: .T. to erase Temp File .F. to keep it

************************************************************************

FUNCTION Process

LOCAL lcParameter, lcOutFile, lcIniFile, lcOldError

Request = THIS.oCGI

Response = THIS.oHTML

*** And retrieve the HTMLPagePath and cDataPath vars – [MyCodeClass] section

THIS.cHTMLPAGEPATH = THIS.GetAppIniVar("HTMLPAGEPATH")

THIS.cDATAPATH = THIS.GetAppIniVar("DATAPATH")

*** Retrieve the second parameter which specifies the Method to call

lcParameter=UPPER(THIS.oCGI.GetCGIParameter(2))

DO CASE

CASE lcParameter = "DISPLAYFILE" OR lcParameter="SHOWPAGE"

=THIS.DisplayFile(THIS.oCGI.GetCGIParameter(3),HTMLPAGEPATH)

*** NOTE:    GENERIC ROUTING BY PARAMETER NAME

CASE !EMPTY(lcParameter) AND PEMSTATUS(THIS,lcParameter,5)

=EVALUATE("THIS."+lcParameter+"()")

OTHERWISE

*** Unhandled method - display error page

THIS.ErrorMsg("The server was unable to respond "+;

"to the CGI request",;

"Parameter Passed: <b>"+PROPER(lcParameter)+"</b><CR>"+;

"Error: <b>Unhandled Method</b>")

ENDCASE

RETURN .T.

ENDDEFINE  && MyCodeClass

### Step 3 – Set up your HTML and Datafile paths

You have two options here. The easiest way to do this is to store the paths in the startup INI file (in this case wcmain.ini). Add a section with the name of the new class you created to the file like this:

[MyCodeClass]

HTMLPagePath=c:\wwwroot\wconnect\

DataPath=C:\wconnect\wwdemo\

You can retrieve these variables with the following code at the top of the Process method as the sample does:

*** And retrieve the HTMLPagePath and cDataPath vars

THIS.cHTMLPAGEPATH = THIS.GetAppIniVar("HTMLPAGEPATH")

THIS.cDATAPATH = THIS.GetAppIniVar("DATAPATH")

The above requires access to the INI file on each hit, which can be a slight performance hit that you might not want to take.

Another good approach is to add a set of #DEFINE's at the top that identify where your code can find the HTML files that might be read from scripted pages and your datafiles. To go this route you'd add:

*** Development machine

#IF LOCALSITE=1

#DEFINE HTMLPAGEPATH "c:\http\wconnect\"

#DEFINE DATAPATH "c:\wconnect\wwDemo\"

#ENDIF

*** Online

#IF LOCALSITE=2

#DEFINE HTMLPAGEPATH "c:\wwwroot\westwind\wconnect\"

#DEFINE DATAPATH "c:\wconnect\wwDemo\"

#ENDIF

The #DEFINEs get directly compiled into the code which makes this very efficient, but in order to switch between sites (such as your local machine, online and your notebook) you need to recompile. The LOCALSITE switch can be set in your WCONNECT.H.

Whenever you reference pages on a scripted page using ShowHTMLFile() or ShowMemoPage() you'd then use or THIS.cHTMLPagePath or HTMLPAGEPATH, and THIS.cDatapath or DATAPATH depending on whether you chose to use the INI file keys or the #DEFINE to set the path.

### Step 4 - Add your custom method to the Process Class

The next step is where your work begins. Up to this point the code is really generic and the same for each application you'd create. The Process method of the class above checks to see if the method the URL is calling for exists and if found executes it. So now your job is to add a method that handles the specified request.

Add a new method to the MyCodeClass above before the ENDDEFINE statement:

FUNCTION MyFirstWebRequest

*** WwProcess class contains oHTML and oCGI objects

*** Make it easier to reference these

*** Create a page header - HTML object is already created at this point

Response.HTMLHeader("My First Web Request")

*** Select some records from a database

SELECT client ;

FROM (THIS.cDataPath + "TT_CUST") ;

INTO CURSOR Tquery ;

ORDER BY Client

*** Now display the results in an HTML table

Response.ShowCursor()

*** For fun let's put some CGI information on the page as well

Response.SendPar()

Response.Write("Browser used: "+Request.GetBrowser()+"<BR>")

Response.Write("Query String: "+Request.GetCommandLine()+"<BR>")

Response.Write("3rd Parameter: "+Request.GetCGIParameter(3))

Response.HTMLFooter(PAGEFOOT)

ENDFUNC

Save that and DO wcMain from the command window, which restarts the Web Connection server.

### Step 5 - Check it out from your Browser

In the previous steps you've set up a new program that will handle Web requests and added a method in the WwProcess class to respond a request. The next step is to call this request handler from the browser. Switch to your browser and type in the following into the location line or create an HTML with this link information:

http://localhost/wconnect/wc.dll?MyCode~MyFirstWebRequest~Parameter1

or  from a link on an HTML page:

<A HREF="http://localhost/wconnect/wc.dll?MyCode~MyFistWebRequest~Parameter1">

My First Web Request</a>

Note how the request is routed to your code: The URL references the MyCode.prg file, which in turn creates a wwProcess object which contains a method called MyFirstWebRequest. The final parameter or parameters if you like can be easily retrieved with Request.GetCGIParameter(nParameter).

What happens if these programs or methods don't exist? Try it by mistyping them. You'll get an error page generated by Web Connection that tells you that this request type is not supported.

### Step 6 - Go to it!

Alright, now you have some working code to work with. Adding additional request handlers is as easy as adding another method to the Process class. For more examples on how to create pages and handlers check out the examples provided in the wwDemo program file.

# The wc DLL and EXE files

The Web Connection Connector application is the ISAPI or Windows CGI script file that is called from your HTML pages and forms. It comes in two varieties: As an EXE for CGI compliant Web Servers (Commerce Builder, WebSite, FolkWeb)  and as a DLL for ISAPI compliant Web Servers (MS IIS, Commerce Builder, Purveyor, WebSite). The intermediary program approach serves three purposes:

- It does away with the overhead involved to load Visual FoxPro, instead letting a Visual FoxPro session sit in ‘server’ mode waiting for an incoming Web requests. Without the load time, request parsing overhead is somewhere just below .15  second (on a P133 – 64megs) plus the actual time it takes to perform the query and generate the HTML document.

- wc handles time-outs and VFP program errors by using a time-out value contained in an INI file to determine when to consider a request as failed. Since the Web Server will wait indefinitely for a request to succeed, it’s possible to hopelessly tie up a machine with never ending requests. By using a time-out the intermediary program can terminate the process if VFP has not returned a result on time, which frequently occurs while debugging HTML page code in response to requests.

- The intermediary program allows remote execution of the processing program, namely our VFP code. Since the messaging approach used by Web Connection and wc.dll is using either OLE Automation or File based messaging, the request file can be accessed across the network and be processed accordingly using either DCOM or UNC file naming . Since only the result document travels over the network, network overhead is minimal, yet provides maximum flexibility in terms of CPU load by seperating the Web service from the data service.

Note:   The following examples all use wc.dll in the HTML page URLs. If you are using an CGI based server replace all DLL extensions with a EXE extension instead. Otherwise the functionality for CGI is identical.

## How to use it

The wc program acts as an intermediary between the web server and Visual FoxPro. It handles the communication aspect of sending server information and form variables to your FoxPro backend and returns the HTML (HTTP compliant output to be exact) to the Web server.

A typical request from a simple HTML HREF link might look like this:

<A HREF="/wconnect/wc.dll?wwDemo~TestPage">Simple Web Test</A>

When the user of the HTML document clicks on the hot link the request specified in this HREF reference is executed. Note the ‘?’ to specify additional parameters. This additional parameter is a very important tool to pass information to the VFP server process. In my examples I always use the first parameter as a program key – the program that is to be run. The second parameter tells the VFP program which request is incoming and which method in the wwProcess class to run. For the sample page the methods are: TestPage, ClientList, ShowHours etc.

Of course, you can also generate CGI requests as part of HTML input forms:

<FORM  ACTION="/cgi-win/wc.dll?ShowHours" METHOD="POST">

<A HREF="/wconnect/wc.dll?wwDemo~ClientList"><B>Client Name:</B></A><br>

<INPUT NAME="Client" TYPE="TEXT" ROWS=1 SIZE="40">

<p>

<PRE>

<B>From: </B><INPUT NAME="StartDate" TYPE="TEXT" ROWS=1 SIZE="10">

<B>  To: </B><INPUT NAME="EndDate" TYPE="TEXT" ROWS=1 SIZE="10">

</PRE>

Again, note the ‘?’ and the following ‘ShowHours’ method parameter.

Note that spaces are not allowed in the parameter portion of requests, so you commonly have to translate strings that include spaces to use another character. In my examples I use the plus sign (‘+’) to indicate spaces as part of a parameter and a tilde (‘~’) to indicate a separator between individual parameters. Individual parameters? Sure, sometimes you want to supply a method and additional information as part of a request.  For example, each entry on the client list that is dynamically generated by clicking on the client name button needs to create a ShowClient request and also pass along the name of the client to lookup. The generated URL looks like this for each client in the list:

<A HREF="/wconnect/wc.dll?wwDemo~ShowClient~Bank+of+America">Bank of America</A><BR>

## What does it do?

The messaging mechanism determines how wc.dll handles request processing. With OLE Automation the server essentially creates a persistent OLE Automation server and then repeatedly calls its public ProcessHit() method for each request. With file based messaging the DLL creates a 'messaging' file that contains the path to the INI file with the server variables. The Web Connection server on the Visual FoxPro side is waiting for the incoming request by polling for a specific file template in a specified directory. When a properly formatted file is found the server calls its ProcessHit() method which is almost identical to the one in the Automation server. ProcessHit() goes on to call Process(), which in turn calls your own PRG file which can then handle the incoming request. Typically, you'll create a wwProcess object and use it to create the output back to the server.

The Web Connection DLL then waits for the FoxPro code to complete its operation. With Automation the server simply returns control when the ProcessHit() method ends, simply returning the HTML output as a string value of that method. With File based messaging the ProcessHit() method creates a small result file that the DLL is expecting as completion signal. When it finds the file, the DLL reads the HTML output from an output file that was created by your code (handled transparently by the wwHTML class). The output is sent back to the Web server and the result is displayed in the browser.

## The wc.ini file

This file contains several runtime options that are used by the program to determine how to operate. A typical wc.ini file looks like this:

[wwcgi]

Path=c:\temp\

;*** File options

Timeout=50

PollTime=200

Template=wc_

Mechanism=File

PostMethod=URLEncoded

AdminAccount=rstrahl

AdminPage=c:\http\wconnect\admin.asp

ExeFile=c:\wwapps\wc2\wwdemo\wwdemoole.exe

UpdateFile=c:\ftp\uploads\wwdemoole.exe

FileStartInstances=0

[Automation Servers]

Server1=wwDemoOle.wwDemoServer

Server2=wwDemoOle.wwDemoServer

;Server3=wwDemoOle.wwDemoServer,\\RemoteServer

| INI File Setting | What it does | Automation | File |
| --- | --- | --- | --- |
| Path | Determines where the DLL sends the server request file that contains the content of a particular request. Typically this will be your system Temp directory |  |  |
| Timeout | Determines how long a request can take before it is timed out and an error page is returned to the Web server. If you plan on requests taking more than 60 seconds each you’ll need to bump this value up. Default: 60 (seconds) |  |  |
| PollTime | Applies to file based only and determines how often the DLL polls for a return result. Default: 200 (millisecs) |  |  |
| Template | Applies to file based only and determines the 3 letter prefix that is used for the messaging files. Default: WC_ |  |  |
| Mechanism | Determines whether file based or Automation based messaging is used. Values are: Automation or File |  |  |
| PostMethod | Determines how the Request data is encoded. Older versions of Web Connection used an INI file newer version pass URLEncoded strings. If using the default of URLEncoded wconnect.h must have #DEFINE POSTDATA .T. Values are: URLEncoded or INI. |  |  |
| AdminAccount | Allows setting of the Administrative account that is allowed to access the DLL’s internal maintainence functions that can start and stop servers. If an account is specified only that account will be allowed access. If not logged into the Web server a login will be prompted. Values: AccountName – The account to check for Any – Any account as long as the user is logged in Or leave the key blank to disable account checking. |  |  |
| AdminPage | Full path to the admin page you use. This is used to provide a back link from the various maintainence functions built into the DLL. Example: c:\http\wconnect\admin.asp. |  |  |
| ExeFile UpdateFile | These two entries allow you to update EXE files online while the server is running. You can set up the EXE file of the server and provide a name for another file that serves as an update file. The Maintain?UpdateExe allows you to upload a file to a server and ‘hot’ swap server EXEs without stopping the Web service. For file based messaging you need to make sure all servers are shut down first or else the update will fail. You can also use StartEXE to restart a stalled server or to restart after an update. ExeFile=c:\wwapps\wwdemo\wwdemoole.exe UpdateFile=c:\ftp\uploads\wwdemoole.exe |  |  |
| FileStartInstances | This flag allows you to automatically launch Web Connection servers when the Web Connection DLL is first loaded. This key uses the value in ExeFile to determine which EXE to launch. Make sure you test operation of this feature first by using the wc.dll/maintain?StartExe function to load your EXE, since this function returns error information. If FileStartInstances cannot load your servers no indication is given of the failure. |  |  |
| [Automation Servers] | This section of the INI file determines which servers are loaded on requests. Note the ability to access remote servers with the last example. [Automation Servers] Serverloading=0 Server1=wwDemoOle.wwDemoServer Server2=wwDemoOle.wwDemoServer ;Server3=wwDemoOle.wwDemoServer,\\MachineName |  |  |
| ServerLoading | Used only in the Automation Server section this option determines how requests cycle through the loaded servers. 0 - Load Based Server only gets called when the previous one is busy. Typically the first server is worked heaviest and the last server the least. 1 - Round Robin Request always cycle through servers sequentially. All servers are loaded equally. |  |  |
| KeepAlive | This flag allows getting around a DCOM bug that causes COM objects loaded by IIS threads to unload after 8 minutes of idle time. KeepAlive forces an extra reference on the server making COM keep the server locked and making it not unloadable. 0 – Normal Servers can unload. Use this if your servers are lightly loaded and run with short requests and you want the system to unload these servers to preserve resources. 1 – Keep Alive Keeps an extra COM reference to force servers to stay alive at all times. Servers unload only when IIS unloads or when you use Web Connection's own unload links. |  |  |

### DLL Error Message Formatting

The Web Connection ISAPI extension can throw several errors internally. By default these errors generate error messages that are displayed as HTML generated to a simple template inside of the DLL. Since these errors never hit the Web Connection Visual FoxPro server, this means you can't change the error message directly.

In some instances this may not be appropriate. While errors are rare and usually point to a problem in the Visual FoxPro server code, it's sometimes necessary to provide an error message that is suitable for end users rather than the technical message that the DLL pops up. So, to do this you can provide an override form for each error message via settings inside of wc.ini.

The following entries and error messages are available for customization:

| Ini Entry [HTML PAGES] | Conditions |
| --- | --- |
| Exception | An exception occurred during Web Connection processing. This is a wc.dll internal error and should hopefully never occur <s>. |
| NoOutput | The Web Connection server returned no output to the ISAPI extension. This means your Web Connection VFP code failed to create any output, which should never happen. |
| Maintenance | Your server is set into Maintenance mode and the HoldRequest flag is set to 1, which doesn't allow requests to reach your VFP server. Use this for maintenance while the server is running. |
| NoLoad | The Web Connection servers couldn't be loaded (Automation only) |
| Busy | All server instances are busy (Automation only) |
| OleError | An exception occurred while calling the Automation server. |

If  entries are left blank the default error messages are brought up generated by the DLL. You can assign files to each of these keys that identify the page that you would like to load. Pages are loaded from the specified file – pages should not exceed 16k in size.

[HTML PAGES]

Execption= C:\westwind\wconnect\error.htm

Maintainance=C:\westwind\wconnect\error.htm

NoLoad= C:\westwind\wconnect\error.htm

Busy= C:\westwind\wconnect\error.htm

NoOutput=

OleError=

You can also embed two special keys into the page to match the error message that the Web Connection request creates.

Embed the %s string into the page and the first encountered will expand to the error header, the second to the error message:

<HTML>

<h2>%s</h2>

West Wind Technologies regrets that the following error occurred:<br>

%s

</HTML>

### DLL Error Log

Web Connection is an ISAPI DLL that runs inside of IIS. Since it's a system level component errors don't show on the console, so all errors are logged into a file that you can examine. The file is called wwCGIErr.txt and goes into the Web Connection temporary file path. Note IUSR_ must have Full Access in this directory in order to create the log and write to it.

The maintenance page contains a Show DLL Errors link you can use to view the log.

There are a number of errors that are generated in this log that are not critical – they're reported for your information and debugging purposes. Harmless errors include:

- All servers are busy
If your server pool is too busy to take further requests in the allocated timeout period you'll see this error message. This is not really an error, but merely a notification that your server is maxed out.

- Forced Release of Server
If you're running Automation servers are unloaded by releasing the COM references which release the object when the reference count goes to 0. In some instances related to the multithreaded nature that Web Connection objects are called from the reference counts alone do not allow the objects to unload. In these cases the Web Connection DLL forces the objects to unload by terminating the process explicitly. This is normal for servers that have been running for long periods of time, especially if you use the KeepAlive flag on your server. This information is logged merely for informational purposes.

- Web Connection Request timed out
This means that a request took longer than the allotted timeout period as specified in wc.ini. This either means you had a very slow request or possibly your server hung on an error or some unexpected UI operation. If the Web server is busy the offending server was probably unloaded automatically and reloaded. This error is also common when working with file based operation and testing development installations. Anytime you hit a WC link and don't have the server running one of these messages will eventually be generated when the request times out.

- GetIDsOfNames or QueryInterface Failed due to thread Time Out
Web Connection Server has been unloaded (RPC Server Unavailable)
These errors relate to crashed or killed Web Connection servers that unloaded while they were idle. When Web Connection tries to access these server these errors occur. These errors are actually trapped and the server are automatically reloaded at that time so that the client never sees an error in most cases.

The errors above are all benign and are nothing to worry about even if you see a lot of them. There are also permission errors related to maintenance tasks which also are not crucial. Errors to watch out for are:

- Exception Errors
Unhandled exception errors are caused by system errors (like heap corruption, corrupted memory) and believe it or not code errors <s>. If you have isolated exception errors that occur very rarely it's nothing to be worried about – IIS has been known to get unstable and memory/COM subsystem errors are something that will occur from time to time. In my experience these errors are very few indeed and most don't manifest themselves as serious to require even a Web server shutdown/restart.

If you run into consistent exception errors make a note of the error message (the nature of exceptions makes this necessarily vague but some will report a little info) and try to get as much context as you can to report the issue. What is the server doing, what types of requests are you running when the problems occurred etc.

# Web Connection Server Management

Web Connection provides a number of built-in, useful utility routines that help you manage your Web Connection applications. There are two kinds of services provided: Those that are internal to the DLL and manage server loading/unloading and those that are provided as part of the wwMaint Process object that's provided as a Web Connection application class. The wwMaint class can be plugged into any application and can be extended for additional maintainance tasks.

You can find the code in wwMaint.prg. Note that wwMaint.prg must be explicitly called in the mainline CASE statement that controls all PRGs that can be handled by the application. Check out WCMAIN.PRG and the Process procedure to see how this is accomplished.

Web Connection also provides a maintainence page: Admin.htm which contains all of the links discussed below.

## Mangement Functions built into the wwServer Form

The Web Connection server is implemented as a Visual FoxPro form and provides an interactive interface for setting some server properties on the fly as well as providing access to the log and some additional debugging features. The server form by itself displays the hits that are currently processing on the server.

The server form shows the last 8 requests in its main window and the red light on the lower left corner indicates whether the server is busy. The status display can be turned off with various options available on the server object. The wwServer::ShowStatus() method toggles the display on or off. Turning off the display status results in slightly faster operation, but it's often nice to have some visual feedback on the servers online operation.

The Status button provides access to some of the most important status information of the server. These properties correspond to the server's actual properties, and if set here are not permanently set. To set properties permanently you can edit the wcmain.ini file physically off the button link. Here's what the fields mean:

- Startup Path
The startup path is the path where the EXE file lives. While this is fairly obvious when you're running a standalone EXE, this setting is crucial for OLE Automation Servers which typically start out in \Winnt\system32. This setting causes a SET DEFAULT TO (THIS.cAppBasePath) to be executed in the Init of the wwServer to set the context. This setting is maintained in the registry under /HKEY_LOCAL_MACHINE/SOFTWARE/West Wind Technologies/Web Connection/Servers/ClassName/AppBasePath. The value to this key is set the first time the server is run based on the current startup directory. The value can be changed in the Status form and updated by clicking on the Update Settings button.

- Temp File Directory
This is the actual directory that the server is using for retrieving the message and content files used in communication with the server.

- Tasking Priority
This option shows the value of the server object's nTaskingPriority setting which determines Process priority for the server. Higher numbers mean faster performance, but more strain on system resources. Under NT this value should be 1 if running a single server, or 0 if running more than one server. Under Win95 this value can be set to 2 or 1 respectively for the same configurations.

- Timer Interval
Applies only to file based operation and will be dimmed under OLE Automation. This option determines how often the server fires a timer event to look for incoming Web request files. Corresponds to the wwServer::SetTimerInterval() method.

- File Template
Applies only to file based operation and will be dimmed under OLE Automation. This option determines what file template the server looks for when checking for request files.

- Show Status
This checkbox determines whether each request is shown in the Server's list of the last requests. When set each hit is shown on the form and the form is UI active with the buttons going disabled while the request processes. When off no UI updates occur and clicking on a button might result in a slight delay while VFP catches up with the UI.

- Log To File
Determines whether each hit is logged into CGILog.dbf. Logging can be turned off for slightly faster operation.

- Save Request Files
This handy feature allow you to capture the incoming INI file and the outgoing HTML file and save the content to TEMP.INI and TEMP.HTM in your TEMP directory. This feature is very useful while debugging to review what came in and how your code responded. To view the files right click over this text box and select to view either the HTML or INI file output of the last captured request.

- Update Settings
To update the INI file settings and the registry from the values set above click this button. If you make changes to the fields and don't click this button the changes affect only the current server session.

- Browse Log
This option brings up the Log browser. You can view individual hits or you can view errors. If you view errors you can expand the columns to see the error information in full window view. Handy for checking errors, but keep in mind this is a non-optimized view, so error viewing might be slow.

- Clear Log
Allows you to clear out the log file. The file is ZAPPED.

- wcMain.ini – Edit the Application startup INI file
Clicking this button will bring up an editing window that contains the startup INI file as set by the server's cStartupIniFile property. Changes made there do not take until the server is restarted.

- wc.ini – Edit the DLL startup INI file
Allows editing the startup ini file. The name of this file is passed by a Web request, so until you hit the server with a request at least once this button is disabled. Once edited the DLL must reload the INI file with the wc.dll/maintain?ReadSetupIni call.

## Maintainence functions built into the ISAPI DLL

The Web Connection wc.dll provides a number of built in functions for managing Web Connection servers. These maintainence functions are available only through the Web interface of an application and follow a very specific format.

All of the maintainence requests are accessed by using a special syntax with the ISAPI DLL. For example to release all servers you would access:

http://localhost/wconnect/wc.dll/Maintain?Release

You use the 'extra path' following the name of the DLL by specifying /Maintain. Follow this by a ? to enter the actual parameters on the query string. In this example the Release method is called which is responsible for unloading all Automation servers.

Access to all maintainence functions of the DLL is controlled via NT Authentication on the Web server. A special key in wc.ini AdminAccount determines which account has access to maintainence functions. This key can be blank to not check for Authentication, Any to allow any logged in account (ie. anything but the anonymous IUSR_ account) or a valid NT user account name. For more details see the wc.ini section under AdminAccount.

The following table lists all of the maintainence features available inside of the DLL:

| ISAPI DLL Command | What it does | Automation | File |
| --- | --- | --- | --- |
| ShowStatus | Displays information of the current settings of the DLL. For Automation servers this display also shows which servers are loaded and if they are currently busy. This link summarizes the most important settings in wc.ini and you should use it to make sure you have all expected settings correctly set. |  |  |
| Load | Loads all the specified Automation servers into memory from the DLL startup INI file. |  |  |
| Release | Releases all Automation servers from memory. |  |  |
| StartEXE | Starts an EXE specified in ExeFile in the DLL Ini file for file based messaging. The EXE is started in the System context so it will run invisibly when started from a Service. You can make the session visible by allowing the service to 'Interact with Desktop' in the Service manager. Use this only if you've crashed the server, or if you've switched from Automation to File based and you need to get one server started to manage additionals. You can also use the FileStartInstances key in wc.ini to force instances to start up when the DLL first loads simulating behavior of an NT service and OLE Automation with file based operation. |  |  |
| UpdateExe | Updates the EXE file as specified by the INI file EXEFile and UpdateFile keys. With Automation the process is automatic: Servers are unload, the Hold RequestFlag is set and the EXE file is copied. The servers are then reloaded. With file based messaging you are responsible for unloading all running sessions first using the Session links (see next section). Once unloaded the update operation is identical. |  |  |
| MaintMode | Releases all but one of the Automation servers from memory. This is very useful for doing maintainence tasks that require exclusive access to tables when running more than 1 server instance for a particular application. |  |  |
| HoldRequests | Forces the DLL to return a Message page that states that the server is busy. Requests are not passed on to the Web Connection servers. This can be used to update files on the server including Automation Server executables. |  |  |
| MaintHoldRequests | This is a combination of the two above commands that sets up a 'Server is Busy' message while allowing the AdminAccount user to perform any site tasks. One Automation server is automatically loaded. When through use HoldRequests to undo the HoldRequest flag. |  |  |
| RecoverDeadLock | Manually overrides the HoldRequests flag in case you set it and can't access the maintainence page when not logged in. |  |  |
| ReadSetupIni | Re-reads the settings from the DLL startup INI file. |  |  |
| SetFileMechanism | Switch the DLL from Automation to File based message processing. |  |  |
| SetAutoMechanism | Switch the DLL from File based to Automation message processing. |  |  |
| KeepAlive | This request fires all of the Automation servers currently loaded. You can use this link to keep alive servers from Web Monitor by hitting it every 2 minutes. IIS unloads low usage threads after approximately 8 minutes and this allows keeping servers running. Servers are hit only if idle for more than 1 minute. |  |  |

### Online Code Updates

Updating code online without shutting down the Web server is possible with Web Connection. The idea is that you upload your new executable to the server into a temporary location and then use an update link to actually copy the new file over the existing version.  It's not quite so easy however, as you have to make sure all other sessions of the EXE are terminated before updating the file.

The first thing you have to do is set up the wc.ini keys EXEFile and UpdateExe. The EXEfile should point at your executable file, while the UpdateExe should point at the new EXE file. It's very important that the Admin account that you use when you click the link (and is set up as the AdminAccount key in wc.ini) has access rights to read the file from the source directory and write access in the target.

To actually update the files:

OLE Automation

With Automation the process is actually automated. Assuming you have the files in the correct location all you have to do is click Update Code on the maintainence page and all servers are shut down for you and the files copied. Once complete the servers are restarted for you.

File Based

File based is more difficult as the ISAPI DLL has no control over the server instances. Instead you have to first kill all server instances by using  wc.dll?wwMaint~Sessions~KillUnconditional until ALL sessions have been killed from the server. You can then click the Update EXE link. Once the files have been updated you now have no sessions running. In order to start a session you have run the Emergency Restart Exe file link on the maintainence page. This will start up the EXE specified in the EXEFile key of wc.ini. Note that this server will run invisibly on the NT desktop – it will be visible only in task manager (or if you have your Web server set up in the Service Manager to 'Interact with Desktop').

## Functions provided in the wwMaint Class

COM/Automation operation provides much easier and more complete server maintenance functionality. Since the ISAPI DLL controls your COM servers it has full control over them with the ability to load and unload them as needed.

## Functions provided in the wwMaint Class

### Web Connection Hit logging

By default the Web Connection server automatically logs each request that hits the server. The wwServer's LogEntry() method automatically logs the following into CGILOG.DBF located in the Web Connection startup directory:

| Field | Function |
| --- | --- |
| Time | The date time entry when the request occurred |
| Request | The full Query String (anything following the ?) of the URL that caused the request to occur. |
| Script | Name of the DLL/EXE/ISA file or script mapped page. .WC and .WCS pages show up here. |
| RemoteAddr | The IP Address of the user that incurred the request. This is the Referrer CGI variable pulled from the Web server. In addition the end of the string contains a numeric value that reflects VFP's memory usage (SYS(1016)) to help pinpoint memory usage problems. |
| Duration | The time the request took. This value contains the time the user VFP code took to execute - it does not include the request messaging process or the time it takes to get the actual response back to the user over the Internet. Use this value to see how efficient your code runs and to pinpoint any server slowdowns based on memory usage etc. |
| Error | Logical value set to .t. when an error is logged. Used for simplified error retrieval. |

To show the contents of the Log from an HTML page do:

wc.dll?wwMaint~ShowLog

Note that you'll only see the last 250 records. This is done to keep the request short and not overload the browser's HTML table display. If you're using Internet Explorer 3.0 you'll also see an ActiveX chart that shows the activity since midnight.

wc.dll?wwMaint~LogSummary

You can also view a summary of hits tallied request. This works only for numbered parameters (wwDemo~TestPage for example). You can group by each parameter number. So, application would be 1, request would be 2 etc. Currently there's no support for  scripts – script pages will not be counted.

wc.dll?wwMaint~ClearLog~NOBACKUP

In order to clear the log only one session can be running as exclusive access to the log file is required. When the log is cleared only the data up to the current day is cleared – today's data always stays in the file. When clearing the data in the log is appended to LOGBACK.DBF for archive purposes. Ideally, you want to run a daily log and keep the archive for reporting purposes. This file can get big, and clearing can get slow because of the APPENDs to the LOGBACK file. If you don't want a backup use the NOBACKUP parameter.

### Displaying the ISAPI DLL Error Log

In addition to viewing the Web Connection Server server generated DBF file log you can also view the ISAPI DLL log of errors by using the Show DLL Errors link. DLL show any problems that may be occurring with the ISAPI DLL from minor notifications to serious exceptions. For description of common errors and what they mean see the previous section on wc.dll settings and commands. You can also clear the log from this link.

### Management of Multiple Sessions for File Based Messaging

Since Visual FoxPro is a single threaded application (actually, it isn't - several threads run, but only one thread is available to application code) the only way to provide support for simultaneous requests is to run multiple instances of Visual FoxPro and Web Connection. Running multiple sessions can provide your application a better feel although the overall response time is probably not improved much by running multiple sessions. Rather multiple sessions should be used to balance request loads so that longer requests are not hogging the CPU and tying up the pipe while short requests have to wait for long requests to complete. On a single processor box two simultaneous sessions provide good performance without noticable slowdown - more sessions will see requests slowing down considerably when running simultaneously. On a dual processor box the cutoff point comes at 4-5 sessions.  If you need more sessions the next step is to move off to different machines and run them across the network (see previous section).

Running multiple sessions can provide the scalability needed to run simultaneous requests, but it also causes some maintenance headaches. If you're running multiple sessions it becomes crucial that you can shut down sessions so you can perform maintenance tasks that might require exclusive access to your data files. How often this occurs depends on your site, but if you are running a site that's running offline from another database application with data being shuffled back and forth maintenance tasks occur frequently.

Session management for Automation servers is handled via the DLL Maintain functions described in the previous section. For file based messaging the DLL has no control over the standalone VFP EXE files and thus can't manage them. The wwMaint module handles some of the chores by using the RUN and QUIT commands to manage running sessions using its Sessions method.

Note: In order to start a new session of Visual FoxPro development you need to set the following key in the wcmain.ini file in the Web Connection root:
   [wwMaint]

##### RestartExePath=c:\vfp\vfp.exe -t wcmain.prg
   or
   ; RestartExePath=c:\wconnect\wcdemo.exe

The URL syntax for the available functionality is:

wc.dll?wwMaint~Sessions~START

Starts a new session. Note that at least one session must already be running for this to work since Web Connection implements this mechanism. Web Connection actually issues a RUN command to start another session.

wc.dll?wwMaint~Sessions~KILL   wc.dll?wwMaint~Sessions~KILLUNCONDITIONAL

Kills a session that's running. Note that you can't kill the last session. Sessions are tracked in a special table in the CGITempFile directory (wwSessions.dbf). KILLUNCONDITIONAL will not check the sessions table for the last session to kill, but will simply shut down the session that receives the request.

wc.dll?wwMaint~Sessions~RESTART

Recycles an existing session, RUNning a new session then QUITing the current session. Why? Visual FoxPro does some minor memory leak problems in extremely busy transaction situations as the 'garbage collection' routines never have a chance to clean up memory released. Over time the VFP session can end up bloated which slows things down. RESTART will kill the session and start a new one. Note that you can't tell which session to restart so it's usually best to hit restart for as many session as are running.

wc.dll?wwMaint~Sessions~SHOWSESSIONS

Shows all running sessions. This is simply a display of the wwSessions.dbf file which is stored in the temp file directory and contains a list of servers. When Web Connection is launched  the server logs itself into the table and logs out of it when the server is released.

wc.dll?wwMaint~Sessions~CLEARSESSIONS

In order to clear a corrupted log file use this function to clear the entire log file. After you've done this, hit RESTART a few times for each server running to have them reregister themselves in the table.

Note: If the Web Connection server crashes it's possible for the session log table to become out of synch with the actual servers running as the server never actually logs out of the sessions file. Use the CLEARSESSIONS options to clear all the entries in the table, then follow up with a number of quick RESTARTs for the number of servers that is actually running. When a server restarts it logs itself back into the session table.

### Server Statistics

There's also a simple link to display basic statistics about the Web server itself. Check out the following link:

wc.dll?wwMaint~ServerStatus

wc.dll?wwMaint~ShowStatus

These pages report settings for the server. The first requires IE 4.0 and allows editing the same server settings you can interactively change on the server's status form. The latter only displays stats about the running application.

Warning!  It's highly recommended that you add Authentication to the requests discussed here or else risk the consequences of people messing with your site. If you check out the wwMaint.prg file Process method you'll find some commented out code that enables security based on a password defined in a #DEFINE that you set up. In addition it's a good idea to set security on the Maintain.htm page that contains all the links to these requests.

wc.dll?wwMaint~EditConfig

It's also possible to edit the Web Connection Configuration files remotely. This link displays a page with the server's config files for the DLL Ini file (wc.ini) or the Web Connection Server (wcMain.ini or whatever your main program is). The files are displayed in textboxes and can be edited and updated via a edit box and button.

Note: Updates work only if the Web Connection server resides on the same box as the Web Server. This request will not work if the INI files reside on remote machines due to the paths that are returned by the Web Server for the DLL and INI files. You can however modify the code in wwMaint.prg to hardcode the appropriate server path.

The actual update links for the INI files are:

wc.dll?wwMaint~EditConfig~SaveDLLIni

wc.dll?wwMaint~EditConfig~SaveWCIni

If you're running COM/Automation you can use the [Automation Servers] section to add and remove servers from the pool.

# Debugging Web Connection Applications

Web Connection provides several built in facilities for error handling via a set of class Error methods and ON ERROR handlers. In order to efficiently debug Web Connection applications a debug mode is also provided that bypasses the error handlers and allows you to stop code operation at the source of an error.

## WCONNECT.H

The 'debug mode' is implemented via a setting in the WCONNECT.H header file that is included in all source files provided with Web Connection:

#DEFINE DEBUGMODE .T.   && or .F.

When DEBUGMODE is set to .T. all error methods and ON ERROR handlers are disabled. The exception to this rule is the wwEval class which evaluates embedded expressions in HTML pages. This object by default uses an Error method to trap errors, but in DEBUGMODE uses an ON ERROR handler that can be disabled by any code that runs inside of an evaluated expression.

When DEBUGMODE is set to .F. a global ON ERROR handler is set (as a safeguard), and Error methods are active for the wwServer and wwProcess. Note that the support classes do not have error methods in order to propagate errors back to wwProcess, so the error can be displayed with an error message and logged – errors in these modules are unlikely though. The wwProcess class receives all errors generated by your user code and creates an entry in the CGILOG and outputs an Error HTML page in response. Since error handling runs up the calling chain, this Error method receives most errors that occur in your code. If an error is handled execution is returned to the ProcessHit method of the wwServer object – this means all objects including the wwProcess object are deleted and control returns to the server for handling the next request. Check out the wwProcess::Error() method to see exactly how errors are handled.

## Error Trapping in HTML Expressions

When debugging embedded FoxPro expressions in HTML code things get a little more tricky. If you are running with DEBUGMODE .T.,  an ON ERROR handler is active while HTML expressions are evaluated. If you are a calling a UDF() or embedded FoxPro code block you can disable the error handler by inserting a ON ERROR without additional statements into the UDF() code, which essentially disables the error handler and stops code when it encounters an error. Don't forget to unset this ON ERROR statement when you're done debugging code.

When DEBUGMODE is set to .F. the Evaluate object's Error method handles all HTML expression errors by setting an error flag and ignoring the error. The error flag is then used to insert an error expression into the HTML output:

< %= Error: <your expression> % >

However, no indication is given as to what caused the error at this point. This method works well for catching errors without doing damage to the system even when evaluating unknown code. If you're in DEBUGMODE you can debug any UDF() code that gets embedded inside of the expression by sticking an ON ERROR command into the code. With DEBUGMODE .T. the ON ERROR will override the existing error handler and stop the code at the error and display the appropriate error message.

When error handling is enabled it's next to impossible to crash the wwServer process. There are a few exceptions though: Any operation that brings up a dialog will hang the server. The most annoying one can be a File Open dialog if a file is not found! To avoid File Open dialogs use UNCs rather than drive mappings that might disappear, check for files before opening them where applicable.

Important:  Changing the #DEFINE DEBUGMODE setting in WCONNECT.H requires that you recompile all program files! You can do this by rebuilding a project that contains all the class and support files, making sure to use the Rebuild All option. Alternately you can delete all the Web Connection related FXP files to force these programs to be recompiled when run or open COMPALL.PRG and 'Execute Selection' of the right click menu.

The #DEFINE DEBUGMODE setting has no effect in the Shareware version of Web Connection, since the class libraries have been pre-compiled into an APP file. The shareware version has DEBUGMODE set to .T. to make it easier to debug your sample applications.

## Checking HTML output and the INI File input

An extremely useful mechanism for determining problems is to use the Status Form's Save Request file option, which saves the last request run. When this checkbox is checked the last HTML result and the request input are saved into your temp path specified in the Server setup window as TEMP.HTM and TEMP.INI respectively. You can view this output by clicking on the Display button. The pop up menu lets you view the last request as well as the last HTML output.

Use this feature to check output and make sure that you are returning exactly what you're expecting to return. One important thing to look for is the HTTP header of the resulting HTML document. The INI file input can be very useful in seeing whether all the form variables from a form are retrieved. This lets you see exactly what the server is sending you on each request and lets you access some of the keys that are not exposed directly by the wwCGI class methods.

## Common Problems

My browser is asking to download a call to wc.dll rather than displaying the data I sent

The most likely problem here is the lack of a Content-type header with your response. Every response should include an HTTP header which as part of it includes the Content-type that tells the browser what to do with the request. A minimal content type header looks like this:

HTTP/1.0 200 Ok

Content-type: text/html

The more common wwHTML class methods automatically add this header for you, so usually you don't have to worry about it. The problem shows up when you don't use one of the following methods:

- HTMLHeader

- ShowHTMLFile

- ShowMemoPage

- ContentTypeHeader

If you build your page manually with wwHTML::Write() calls, make sure you first call wwHTML::ContentTypeHeader() or use the wwHTTPHeader object to add a header to the document.

Web Connection 2.70 and later automatically adds an HTTP Header if you don't provide one. However, it's good form to always provide a header in the client code rather than letting the server create it.

When I click on a Web Connection wc.dll link I see data from an old request

You're likely using Internet Explorer and are experiencing a cached request. Rather than performing the request the browser decides to reload the same request it previously ran from cache. For some pages that are more or less static during a session (an inventory list for example) that might be Ok, but it would be bad for a link displaying an invoice that continues to add items.

The solution is to use an HTTP header that tells the page to expire from the cache immediately. You can use the Force Reload option to add an expiration to the request. To do so pass "FORCE RELOAD" to any of the methods listed above as the cContentType or use the wwHTTPHeader::AddForceReload() method for the same task.

I cretaed a new application for my own requests, but I get an error from WC: "Request Not Supported"

When you create a new application (a new PRG or class file) you have to always add a new entry into the CASE statement in your wwServer::Process() method that routes the call to your own PRG file.

# Web Server Configuration

Web Connection has been tested with the following ISAPI/CGI compliant Web servers. If your server supports ISAPI it is highly recommended you use it rather than CGI for better performance.

- Microsoft Internet Information Server 1.0/2.0/3.0/4.0 (http://www.microsoft.com/iis/)
Microsoft's free offering into the Web server market. This high performance server runs only under Windows NT and uses ISAPI to interface with Web Connection so you need to use wc.dll. It's extremely fast, and my recommendation for running under NT. Same versions of Personal Web Server for NT Workstation also applies here. Automatic Setup is provided.

- Microsoft Personal Web Server Win95 1.0/4.0 (http://www.microsoft.com/ie/)
This is Microsoft's Windows 95 Web server that aims to duplicate IIS functionality on Win95. Before installing Web Connection, please make sure the Web server works correctly – proper configuration and access via proper IP addresses is crucial with this server. Automatic Setup is provided.

- WebSite  1.x/2.x by O’Reilly Associates (http://website.ora.com/)
Version 1.1e and later supports ISAPI (please check the WebSite site and docs for info on using ISAPI) and this is the default that's used by the setup program. If you plan on running ISAPI you must have version 1.1h or later for Web Connection to work properly – prior versions had an unstable ISAPI subsystem. Version 2.x highly recommended with this server. Automatic Setup is provided.

- Apache for Windows
With version 2.95 Web Connection introduces set up support for Apache under Windows. The setup program configures Apache for operation with Web Connection using ISAPI. The setup program modifies conf/srm.conf and leaves a marked Web Connection block at the bottom of the config file.

- Netscape Servers
As of version 2.80 Web Connection supports Netscape Web servers through CGI. Only partial Automatic Setup is provided.

- Commerce Builder 1.5/2.0 by The Internet Factory (http://www.aristosoft.com/)
Commerce Builder is a high performance and very solid secure Web server that supports ISAPI. Automatic Setup is provided.

- Purveyor for Windows NT  (http://www.process.com/)

- The folks at Process Software are the originators of the ISAPI spec and Purveyor provides a solid Web server with a fair number of tools available. Note: Only the NT version supports ISAPI - the Personal Server does not work with Web Connection. Automatic Setup is not provided

- Generic ISAPI and CGI Web Server Installations
Web Connection also provides basic installation for any ISAPI or CGI compatible Web server. However, because servers are unknown, automatic setup is not available. The setup program will only copy the files into the Web directories and configure the startup environment for ISAPI or CGI operation.

### CGI Temp Files (doesn't apply to ISAPI servers)

- The CGI protocol generates temporary files that contain the CGI information generated by HTML forms. Since Web Connection needs to access these files to retrieve the CGI information and return the HTML output, it needs to know where these temporary files are created by the Web server.

- Important:  Most CGI Web Servers store CGI variables in the System Temp directory. Windows NT returns \WINNT\SYSTEM32 as the TEMP directory unless the TEMP and TMP system variables are defined in the System Environment. However, by default these are defined in the User environment - move them to the System Environment! Go to Control Panel, System and add TEMP and TMP variables to the System Environment to get temp files generated in the correct directory. For more info see Troubleshooting below…

- By default Web Connection looks for the CGI temporary files in one of the following directories (in this order):

- The location specified by the SetCGIFilePath method. This parameter should match the setting in the wc.ini file’s Path entry. If none is specified the system's temp directory is used.

- The location specified by the WebSite registry key (not relevant when using the ISAPI DLL) HKEY_LOCAL_MACHINE\SOFTWARE\Denny\WebServer\Currentversion\Tempdir. Obviously this will only work if you are running WebSite on your machine. Be aware that you need to override this setting if you have WebSite installed, but are running another CGI server that possibly sticks temporary files into directory other than the one specified by the registry key.

- The TEMP directory that is returned by the GetTempDirectory() API call.

- If the CGI temp files are going to another location, or you want to access these files across the network you have to override these defaults in two places:

- Using the oCGIServer.SetCGIFilePath() method or the Path key in wcmain.ini.

- This method tells the VFP CGIServer portion of Web Connection where to look for the temp files.

- OCGIServer.SetCGIFilePath("C:\NONTEMPPATH\")

- In wc.ini.

- The Path setting in wc.ini tells the cgi translation program where to find the temp files generated by the Web server. Remember this file is created only after you run wc.dll/dll at least once!

- Path=C:\NONTEMPPATH\

- To be absolutely sure where temporary files are going I suggest you set both of the above settings explicitly, even though the default directories might seem the right place to go. That way the only variable element is the Web Server and its placement of temp files.

## Troubleshooting

- There are a few gotchas to watch out for when working with these alternate file paths.

### Windows NT and TEMP Files

- Make sure that your Web Server Account has READ and WRITE (CHANGE) access in the TEMP directory – the ISAPI or CGI connectors need to write temporary files here!

### Clean up the temp directory

- It's important to keep the temporary path used for Web Connections message files clean. The less files the quicker file lookups will run. Under normal operation Web servers and Web Connection will clean up all their temporary files. However, while testing and failing on requests both Web Connection will occasionally fail to erase all temporary files created as part of a request. Therefore do an occasional DEL *.* on the temp directory or the use of a maintainence routine that's part of the VFP server to do it for you.

- In the tools directory there's a utility called FileCheck.exe that can be set up to clean up your temp directory at certain intervals. FileCheck is configured with an INI file and runs in the background.

# Setup for OLE Automation

OLE Automation is supported transparently by Web Connection. Automation is implemented via a special subclass of the the wwServer class called wwOLEServer. This subclass handles the logic for communication with the Web Connection DLL. Besides this specialized class there are few other internals placed inside of the Web Connection framework that handle slight differences in the way OLE Automation requests need to be handled as opposed to file based messages.

### WCONNECT.H Settings for Automation Servers

Web Connection determines whether your server is to be an Automation server or a plain application based on a setting in the WCONNECT.H file. Changing your application between filebased and Automation is as easy as setting the #DEFINE OLESERVER flag to .T. or .F. and recompiling your application as a single use EXE Automation server (which is VFP's Automation server default).

The following 3 WCONNECT.H directives should be set after you have built and thoroughly tested your Web Connection application as a file based server:

#DEFINE OLESERVER .T.
#DEFINE DEBUGMODE .F.

### Using the OLESERVER flag

The OLEServer flag is the most important. This flag is used internally by Web Connection to properly handle specialized areas of code that differ between the two messaging approaches. It also affects one important part of your code – namely your Server class definition. You might remember the following code from the discussion of the Server operation:

#IF OLESERVER

*************************************************************

DEFINE CLASS wcDemoServer AS wwOleServer OLEPUBLIC

*************************************************************

#ELSE

*************************************************************

DEFINE CLASS wcDemoServer AS wwServer

*************************************************************

#ENDIF

This code is necessary so that each instance of your Server class is subclassed from the proper server class.

The other spot where your user code needs to specify the OLESERVER flag is when you instantiate your own subclass of the wwProcess class. Here the difference is the use of the appropriate HTML subclass to cause the output to be directed to the proper output source for the messaging mechanism. Here's the definition from wwDemo.prg (ie. your application specific stub).

#IF OLESERVER

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,"~","+","wwhtmlstring")

#ELSE

loProcess=CREATE("webConnectDemo",loServer.oCGI,loServer,"~","+","wwhtml")

#ENDIF

From your perspective the only difference in operation between OLE Automation and file messaging is simply the way that the output is generated. File based messaging sends output to a pre-determined output file that is provided by the INI file. Automation on the other hand sends its output back as string. The return text value is stored in the wwOLEServer::cOutput property, which is set automatically from the HTML string output generated when the wwProcess object is destroyed. Because of this you typically don't have to worry about how the output is returned to the server, unless you decide to create output without using a subclass of the wwProcess object.

To demonstrate the logic that goes into this look at this code from the wwServer::Process() method that handles script map processing without going through a wwProcess object instance:

*** Check for .WC file only passed and do the URL mapping

lcPhysicalPath=THIS.oCGI.GetPhysicalPath()

IF ATC(".WC",lcPhysicalPath) > 0

Response=CREATE("wwHTMLString")

poCGI=THIS.oCGI

poHTML=Response

Response.ShowHTMLFile(lcPhysicalPath)

ELSE

*** No matching method

Response=CREATE("wwHTMLString")

Response.HTMLError("Unhandled Request",;

"The server is not setup to handle this type of Request: "+;

"+lcParameter)

ENDIF

#IF OLESERVER

*** Simply assign to output property

THIS.cOutput=Response.GetOutput()

#ELSE

*** Here we need to create and output to file

Response2=CREATE("wwHTML",THIS.oCGI.GetOutFile())

IF TYPE("Response2")="O"

Response2.Write(Response.GetOutput())

ENDIF

#ENDIF

This code creates a temporary HTML object and then assigns the output from this temporary HTML object to the appropriate output object. For Automation this means simply retrieving the output and assigning it to the cOutput property of the server. For File based it means actually creating another HTML object that sends to file and simply sending the previous HTML object output there.

These places are the only ones that you need to worry about the differences between OLE Automation servers and plain VFP applications running file based messaging.

### When to use OLE Automation

I've mentioned a few times that OLE Automation can be difficult to configure and it is. You need to carefully follow the steps described below for configuring the COM security settings and for making sure that your paths are set up properly. However, operationally the wwOLEServer class is only a slightly modified subclass of the the wwServer class and provides all the logic to handle incoming Web requests. Once you get to your application specific code there's absolutely no difference between the two messaging mechanisms.

### Building  your Automation server

Web Connection makes it easy to build your server interactively using file based messaging and an interactive Visual FoxPro session which allows you to thouroughly debug and test your code. That gets you 99% of the way. Once the code works with file based messaging follow these steps:

- Before getting started, save a set of request files with a sample request so you can test your server offline.

- Start the server with DO WCMAIN.PRG or run your standalone EXE (make sure the WCONNECT.H flag is not set to OLESERVER .T.)

- Click on Status.

- Check the Save Request Files

- Run a request from the Web browser that exercises the server. The request files are saved as c:\temp\temp.ini and c:\temp\temp.htm.

- Save the INI file under different name such as c:\temp\temp2.ini by renaming the file from Explorer.

- Exit the server.

Now you're ready to start setting up your Automation server.

- Set the WCONNECT.H #DEFINE OLESERVER .T.

- Compile your main program such as wcmain.prg (don't ask – sometimes building an EXE will not properly find the OLEPUBLIC class so better safe than sorry).

- Create a project. Make sure you choose a sensitive name and that the project does not have the same name as the OLEPUBLIC class! The project name is the OLE Server name that is used as part of the ClassId. The full class ID for your server will be: YourProject.YourwwServerClass. For example, the demo uses wcdemo.wcdemoserver.

- Add the main program to it. Also add WWUTILS.PRG and all of the BMP files from the classes directory.

- Rebuild the project and make sure you can build without errors.

- Now exit the project manager and type:
DO TOOLS\BLDOLE WITH "wcdemo","wcdemo.wcdemoserver","Interactive User"
Actually the above parameters are the default and you can modify the PRG to use your EXE and Server name so you don't have to type the parms each time.

- The above builds your project as a Single Use EXE server and sets the user Impersonation to the Interactive User. NOTE: I recommend you do not change these settings unless you know exactly what you are doing!!!

### Testing your server

You now have an Automation server. Let's make sure it works before trying to let Web Connection at it. We're going to load the server and run the captured request that you saved earlier against it.

- Go to the Command Window and type the following code one line at a time

- o=CREATE("wcdemo.wcdemoserver")

- * ? o.cErrorString    && Show Error message if an error occurs

- o.show                && Make it visible so we can see the request

- ? o.ProcessHit("c:\temp\temp2.ini",.t.) && If your using PostMethod = INI skip .t.

- *** Or you can also do this
? o.ProcessHit("&Query+String=wwDemo~TestPage")   && Match your Url

- If all is well you should now see some HTML text printed to the screen.

- Bring up the Status window and make sure that the Startup and Temp path are correct. The startup path should point at the EXE file's path – if it doesn't change it and click the update button. Release the server and reload. You should now see the proper settings in the server.

- OLE Automation servers cannot kill themselves because an external reference determines their lifetime. So, when you're ready to kill the server type o=.F. or RELEASE o. Do not click on the Exit button which expects that the server runs loaded of the Web Connection DLL. If you do click on it the server will not unload and potentially hang your VFP session for several minutes.

### Automation Server Configuration

Ok, now you've built your server, you've tested it interactively and you're ready to go. But you're not done yet! Get ready to meet the worst behaved piece of software you've ever seen: Windows NT's DCOMCnfg utility, which is responsible for setting access rights on your Automation server. We already set up the impersonation level for the server when we ran the BLDOLE program to build our server. Now we have to configure the COM subsystem in general to allow the IUSR_ account to launch the Automation servers.

You should understand basics of creating an Automation server with Visual FoxPro. If you’re running under NT 4.0 you need to have Service Pack 2 or later installed! You can download the latest NT Service Pack from www.microsoft.com/ntserver.

- You have to set access rights for your Automation server using the DCOMCnfg utility. Use the NT Run box to type DCOMCnfg.

- Set the Default access rights. Add IUSR_machinename to Default Launch Permissions on the Default Security tab. I also suggest you add IUSR_machinename to the Default Access Permissions. This should be the default, but it's a good idea to make sure!

- It's necessary that the server is configured for an Impersonation level. This was set previously with BLDOLE, but you should check out how to set this setting manually in DCOMCNFG.

- On the Applications Tab find your server’s class (ie. WcDemoServer) and double click. Go to the Identity Tab and select Interactive User. Click Ok, OK to exit.

- Default access must be set once. Server specific access must be set whenever the server OLE Ids are rebuilt in the registry. This means when you build the project in VFP you need to re-rerun DCOMCnfg and select your server and click OK. You don’t need to actually reselect the Interactive User setting unless it’s changed. BLDOLE saves you from this DCOMCNFG roundtrip each time.

- You can run the Automation server without DCOMCnfg, but the server will not be visible. Also, if the server hangs you will not be able to kill the process from task manager or otherwise! I recommend running BLDOLE or DCOMCnfg everytime you rebuild your server!

- If you think you MUST use a DLL server you have to take some extra steps: Bring up the NT Service manager and select your Web server service. Click on startup and check the Interact with Desktop checkbox. This allows the server to run visible on the desktop. Keep the following in mind with DLL servers: You can only have 1 instance of a VFP Automation object that is an In Process DLL. Once loaded DLL servers cannot be unloaded unless the Web server is shut down. I don’t recommend using DLL servers unless you have an extremely stable server and you only need one instance. They are faster, but much less flexible in terms of management!

### Configuring your server and the DLL

You're ready to try it via the Web server now. For multiple instances of your server you can specify the server instances in the Automation Servers section of the wc.ini file.

- Set up your wc.ini file. Add the Automation Server to the list:

- [wwcgi]

- …

- Mechanism=Automation

- [Automation Servers]

- Server1=wwDemoOle.wcDemoServer

- Server2=wwDemoOle.wcDemoServer

- You can find the Server Administration links on the Admin.htm page. You can load and unload your servers from this page. Note that you don't have to explicitly load servers – they'll load automatically when you click your first WC link after you've switched to Automation.

- You can also unload your Automation server loaded by Web Connection by clicking on the Exit button on the server form. In order for this to work the wwServer.cReleaseURL variable needs to be set to the link that releases the server. The default is localhost/wconnect/wc.dll/maintain?RELEASE which should be fine if  you installed WC in the default directories. You can change this setting in the SetServerEnvironment() method of your server.

- Fire up your browser and run http://localhost/wconnect/wc.dll/maintain?SetAutoMechanism to switch the Web Connection DLL into serving OLE Automation server requests.

- Click on any link and your server should now fire up.

Some tips:

- Be very careful when building your servers to a specific Class IDs. If you move the EXE file to a new location then recompile it you will end up with two or more DCOMCnfg references to the server. This doesn't hurt operation, but it gets difficult to find the right server in the DCOMCnfg manager.

- Name your servers so that they show up at the top or bottom of the DCOMCnfg Server list – that way you can quickly find it and change the settings.

- Use BLDOLE to compile and build your servers. It will do away with DCOMCNFG except for the first time configuration of the default access settings.

- Automation servers will unload after about 8 minutes of inactivity. The Operating System simply kills the servers and removes them from memory. Web Connection checks for this condition and reloads the server as needed – be aware that the reload will incur a short delay. This is a weird 'feature' but it's actually quite handy for servers hosting multiple sites that are not very busy. When unloading all resources associated with that server are released and can be used by other system services.

- Servers do not run round robin but rather on a stack basis. This means that if a site is not very busy only the first of x number of servers might get hit frequently. The other servers will be idle and maybe even release because of 8 minutes of inactivity. This feature allows you to easily see what load your server is incurring. If 4 sessions are fully loaded and running you can add a fifth and sixth and so on.

- While you can run multiple instances of servers the optimal configuration of performance and response time seems to be two instances per CPU. If you have longer requests more sessions can help responsiveness of your site.

- When running under IIS 4.0 stopping the Web service from the Management Console will not unload the Web Connection DLL nor your Automation Servers. To unload the DLL and the servers correctly use the Services Manager or use NET STOP W3SVC from the DOS command line.

### When to use Automation

I've mentioned in several places here that Automation might not always be optimal, because of the setup hassles. But once configured correctly OLE Automation servers are easier to administer. Here are some pros and cons:

PRO:

- Much more powerful maintainence control from within the DLL. Servers start automatically and can be managed and controlled via commands from HTML pages.

- Automation servers are slightly faster than file based servers and CPU load is a lighter as the polling timer and file lookups are taken out of the process. On busy servers these slight improvements can have a big effect.

- Better load balancing. Because Automation servers are run on an overflow basis you can easily see how load is spread among servers. With file based polling access is a free for all and makes load monitoring more tedious.

- Server statistics and built-in process management and even error recovery for hung servers (due to a code problem or even GPFs) can fire off new instances when requests time out.

CON:

- DCOM configuration is a hassle.

- Compiling servers has to be done carefully in order to not disturb the DCOM entries. Always build your server to the same exact EXE name and don't regenerate OLE Ids or you'll end up with multiple DCOM entries and possibly a bad entry that cannot be configured with DCOMCNFG.

- Remote OLE Automation for multi-machine configurations is difficult to set up. Special user rights are required for the IUSR_ account and servers need to be built with different class names on a different machine in order to run both on the local machine and the remote.

The bottom line is that Automation server setup is difficult for the first time, but once a server is up and running under DCOM, server updates are easy. Servers can be hot swapped using the /maintain?UpdateExe option which does not require running through the DCOM setup again.

# Running Web Connection over the Network

It's possible to run Web Connection over a network to move the CPU load of processing CGI requests off the machine that is running the HTTP Web server. By offloading the database processing off the Web server CPU overhead on the Web server is minimal allowing the server to process more simultaneous requests without loss of response. Using this approach it's possible to scale your VFP processing server's to any CGI request load simply by adding VFP sessions and/or machines on the network, without adversely affecting Web server performance.

## Dealing with Network Pathing

When building a server that will run on multiple machines simultaneously it's important to set up your pathing correctly so that a single EXE file can be run on every one of these machines and still point at common data accessed on a network path. The following tips are suggestions only – you may configure your servers differently to be able to share data, but the following steps have worked well for me here:

- Always use Network UNC paths and relative paths
In order to make access to data or any file based resources transparent to the Web Connection application always reference everything with a network relative path using UNCs or relative paths. Use SET DEFAULT TO <UNC> and from then on in use a relative path such .\data or .\classes to reference other files and Paths. UNCs are much slower than drive maps but once you use relative paths things work quickly again.

- SET DEFAULT TO <\\MachineName\Share\Path>  or wwServer::cAppBasePath
The first step should be to SET DEFAULT to your network startup directory. This simple step puts every application into the same directory regardless from which machine it is running. You can actually set this with the server's startup form by setting the Startup Path setting or by setting the value explicitly in wwServer::SetServerProperties to THIS.cAppBasePath (cAppBasePath causes SET DEFAULT TO be called with the path specified).

- Use relative paths whenever possible
If you used SET DEFAULT TO to establish a net startup path you can follow this up with relative paths that don't point at a particular drive to allow. Use relative paths for explicit file paths or for SET PATH TO statements that add specific data or other paths to the server.

- Make sure your TEMP path points at the Web Server's TEMP path
The temporary files that Web Connection creates are always sent to the Web Server's temp path, so your remote server must pick up the files from there. You can change this setting in wcmain.ini or on the Status form of the server. You can set this on each server's setup form. Ideally you'd point all servers at the same wcmain.ini and use a UNC path to reference the Temp path: Path=\\Server\tempshare\

The above suggestions apply to any network servers built regardless of whether you use Automation or File Based messaging.

## File based Messaging

Setting up Web Connection to work over a network link involves setting it up so that it looks for the temporary message files generated by requests across the network and mapping the filenames properly to locally addressable paths, which is handled internally by Web Connection. Your setup is mainly concerned with identifying the remote path that Web Connection will scan for incoming request message files.

This value is set on the Status form's Temp Path field or in wcmain.ini with the PATH= key. In code you can use the SetCGIFilePath() method to set this path.

No special setup is required on the Web server or with wc.dll/exe/ini. The server operates as before placing the files onto its local drive. The VFP Web Connection server on the other hand is now looking across the network, polling for incoming  requests on the Web server's drive. All filenames returned by the wwCGI class methods are automatically mapped to this network path (the actual filenames will be Web server pathed - the translation is necessary to properly translate the path into the network path), so that no code changes are required internally when moving a CGI process off the local machine onto the network.

## Remote OLE Automation via DCOM

Running Automation servers across the network is a bit trickier. Please note that these complications are not an issue with Web Connection as much as they are NT's DCOM and security issues that apply to porting any Automation objects for simultaneous use on multiple machines.

As with file based messaging make sure all your application paths are pointing across the network at the appropriate remote files if data is shared. Establish a Network startup path.

In order to run Automation remotely you run Distributed COM (DCOM) across the network. DCOM is available as a built in component of Windows NT 4.0 and as an add-in download for Windows '95 (if you have IE 4.0 installed DCOM95 will always be there). I don't recommend using DCOM on Windows 95 – it's a hassle to setup and unreliable when it runs. For serious remoting always use Windows NT.

The steps to remote a server are are:

- In this setup the Web server is the Client (Web Connection ISAPI DLL manages multiple client instances of your server) and the remote machine is the Server. The Web server makes requests on the remote server and receives results from it.

- Build your server and test it as a local server to the Web server with local Automation. Make sure you have your pathing properly corrected to adjust for access across the network using UNC pathing. For example, if your data sits on the local C: drive build your data access so it points at: \\YourServer\CROOT\DATA\MyFile rather than accessing the drive letter (unless you can guarantee drive maps – drive maps are significantly faster than UNC names).

- Copy the server to the remote machine. Copy the .EXE, .TLB and .VBR files. Register the server with:

YourServer.exe /regserver

You'll also want to copy your application INI file like wcMain.ini.

- Test the server locally on the remote before attempting to run it over DCOM (if you have VFP installed that is). Make sure you apply the DCOMCNFG.EXE (from the Run Box) settings described in the previous section (Interactive User, IUSR_ Access and Launch rights). To see the server come up on the remote try code like the following from VFP:

o=CREATEOBJECT("wcServer.wcClass")

o.GetProcesId (1)

Bring up the Status window and make sure all of the settings are correct. You'll likely have to fix the start up path.

- Still on the remote, run DCOMCNFG and add the IUSR_ account of the Web server to the Default Access and Launch permissions on the Default Security tab. If you don't use a Domain Server you'll need to explicitly create new account/password and match it to the Web Server's account and password. With IIS the IUSR_ password is a random value so you'll have to change it both in User Manager and in the IIS configuration utility. Note: You have to set up the IUSR_ account in this fashion – other accounts (even EveryOne) appear to not work for accessing the remote server!

- At this point you should test the server from the Web server machine. First off run DCOMCNFG and bring up your server's specific setting. Bring up the Location tab and select 'Run application on the following computer' and enter the name of the remote machine.  Make sure 'Run application on this computer' is unchecked. This is temporary for testing only – we'll remove the remote reference later.

- Fire up VFP and see if you can instantiate the remote object on the remote machine. Use the CREATEOBJECT code from the previous step.If the server comes up on the remote box your server is installed correctly for operation under DCOM. If you have this working, run DCOMCNFG again and reset the Location to 'Run application on this computer' to allow a local copy of the server to run on the local machine.

- You should now be ready to load the server through Web Connection. Add the remote servers to wc.ini:

[Automation Servers]

Server1=wwDemoOle.wcDemoServer

Server2=wwDemoOle.wcDemoServer,\\RemoteServer

Server3=wcRemote.wcRemoteServer,\\OtherRemoteServer

This setting will load one local server, one on the RemoteServer machine and one on OtherRemoteServer. Actually, for testing it's a good idea to use a single remote server. but in a live environment you'll probably want it to look like the entries above.

- If all is well, the server will come up on the remote box as expected. Note that load time for remote server is significantly slower than local servers as is the time it takes to release the server. Operation of method calls however, is swift. If the server fails to load, you'll see Servers are busy message in which case you need to review the steps here.

- If you run into difficulties with 'Access Denied' errors I suggest  you physically log on to your Web Server box as your IUSR_ account then fire up VFP to try and create the remote object. This allows you to interactively change the DCOM settings plus get the COM error messages properly returned to you by VFP. It's much easier to handle and respond to the errors in VPFs interactive environment than trying to handle the Web server and its error messages.

- If you get 'Class not registered' error for CoCreateInstanceEx this means that the server is either not registered on the remote or that the classids have changed and no longer match. In this case recopy the .VBR file and rerun the server registration routine.

# Configuring Web Connection Servers to AutoStart

One topic that frequently comes up is how to set up Web Connection so that servers automatically start when the system boots up. Depending on whether you run Automation or File based there are several mechanisms available:

## Automation

When running Automation, Web Connection brings up Automation servers automatically when a link that requires a WC server is hit. The trick is to properly configure the Automation server for the environment it's running under. The following suggestions only apply to NT – under Windows 95 there's no security on Automation servers and they will always start automatically.

### If you're always logged on under NT

If you're logged on under NT you should configure your Automation server to the Impersonate the Interactive User using the DCOMCnfg options (as described in OLE Automation Setup). Using this account allows the best operation of WC Automation servers that live visibly on the NT Desktop.

### AutoLogon to NT

A simple way to allow proper operation of  WC is to use the above settings and force NT to automatically log in at boot up. You can use the NT Resource Kit to set this up or set the following registry keys:

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows NT\CurrentVersion\Winlogon

AutoAdminLogon	REG_SZ		1 – AutoLogon   0 – Manual Logon

DefaultPassword	REG_SZ		The accout password

DefaultUsername 	REG_SZ		The account name

### Running without an NT Logon

Automation servers can run without having an NT logon. In order to so you need to configure the server to impersonate a specific user account via settings in DCOMCNFG. Select your specific server within the DCOMCnfg and click on the Identity tab. Choose This User: and enter a valid account and password. I recommend you use an Admin account so you have full access to the machine/network via the Automation server. You can also use the Launching User account which would be IUSR_. However, this account has limited rights and may not be able to access some resources (files, registry or network) that might be required by your application. Note: You cannot use the Interactive User as recommended in the OLE Server configuration. The Interactive User is not available prior to logon. You must use a specific account and make sure that account has rights in the TEMP and any of your application paths with Read and Write access.

Note: When your server is configured using a specific account or Launching user the Web Connection server runs invisibly.

## File Based

When running file based servers do not start up automatically as the Web Connection servers are standalone Visual FoxPro applications.

### AutoLogon to NT and Startup Group entries

The easiest way to force WC servers to start up is to use Autologon (as described above) and server entries in the startup group. When the machine boots the startup group fires off servers automatically.

### The StartFileInstances flag in wc.ini

You can automatically start Web Connection server instances the first time a call to wc.dll is made by specifying the following two keys in wc.ini:

ExeFile=c:\wwapps\wc2\wcdemo.exe

FileStartInstances=2

Servers started in this fashion can be auto-started before NT logon, but the servers will run invisibly (you can see them in Task Manager). In order for FileStartInstances to work you need to make sure you have the following permissions set: Your TEMP directory must have Read and Change (or Full) rights for the IUSR_ and SYSTEM accounts. Any data and your application directory needs to have Read and Change rights set for the SYSTEM account.