FAX480 480 LINE FACSIMILE PROGRAM

Dr. Ralph E. Taggart WB8DQT

602 S. Jefferson

Mason, MI  48854

COPYRIGHT NOTICE

The  FAX480 program is copyrighted by Dr. Ralph E.  Taggart.

You are free to make copies of this software for your own use and

to distribute copies to others, provided that no charge is levied

for such distribution. ALL OTHER RIGHTS ARE RESERVED. You may not

incorporate  any  elements of this program  into  other  programs

offered  for  commercial sale without written permission  of  the

author.

INTRODUCTION

Since  the  publication of the original article on  the  480

line  fax standard in the February 1993 issue of QST, there  have

been additional enhancements to the release version of the FAX480

program that are worth noting:

(1)  The three image conversion utilities (BINFAX,  FAXSSTV,

and BINSSTV) have now been incorporated into the FAX480  program,

providing more convenient use from within the FAX480 environment.

(2)  Two new 480 line display routines  (one  crystal-locked

and one line-triggered) have been added to permit display of  fax

transmissions where you might have missed the start tone.

(3) The synchronous AVT 94 video mode has been added to  the

SSTV receive options.

(4)  A synchronous 36 second SSTV receive routine  has  been

added  for the display of pictures transmitted by  other  FAX480-

equipped  stations.  This  mode, which  depends  on  the  extreme

accuracy  of the transmission rate from a FAX480-driven  ViewPort

interface,  provides  the  same noise  immunity  enjoyed  by  the

crystal-locked  480 line display mode while operating in the  240

line SSTV module.

DISTRIBUTION FILE

The  FAX480.ZIP  distribution file contains a total  of  six

archived files:

FAX480.DOC

FAX480.EXE

HOUSE.A

HOUSE.B

JEN.STV

K4OVV.STV

The  FAX480.DOC  file  is this  documentation  file  on  the

operation of the FAX480 program.

The  FAX480.EXE  file is the complete software  package  for

operation  with  the 480 line fax mode, using  the  modified  A&A

ViewPort VGA SSTV interface.

The HOUSE.A and HOUSE.B files represent a sample 480 line fax

image while JEN.STV and K4OVV.STV are sample SSTV image files.

HARDWARE REQUIREMENTS

The  following hardware requirements must be  satisfied  for

proper operation of the FAX480 program:

(1)  An  IBM PC/AT/PS1/PS2 or compatible with a  10  MHz  or

faster clock speed and a minimum of 512K of RAM.

(2)  A VGA display adapter that is bios-compatible  with

the  IBM VGA standard. Virtually all VGA adapters  meet

this requirement.

(3) A monochrome or color analog VGA monitor.

(4)  A minimum of two 360K floppy disk drives, or  a  single

1.2  meg 5.25  or 1.4 meg 3.5 inch floppy drive,  or  a

hard  drive. The choice of drive options has no  effect

on  the  operation of the program but does  impact  the

number of fax or SSTV images that can be stored as well

as  the speed with which images can be saved or  loaded

from  disk. All things being equal, you will enjoy  the

program more if you have a hard-drive (a statement true

for  most software), but, if you lack one, there  won't

be any limitations on what you can do.

(5) An optional Hewlett-Packard DeskJet or LaserJet  printer

for  generating hard-copy printouts of fax images.  The

Laser printer must have sufficient memory to handle 300

dpi graphics.

(6) An A&A Engineering ViewPort VGA SSTV interface, modified

by  the addition of a 4 MHz clock circuit as  described

in the February, 1993 issue of QST.

(7) An available parallel printer port.

INSTALLATION

Hardware

Installation  is a simple matter of connecting the  parallel

port cable between the interface and your computer and making the

needed  audio  connections.  A shielded  audio  cable  should  be

connected between the receiver audio output and the SSTV IN  jack

on  the rear of the interface. A microphone is connected  to  the

MICROPHONE IN connector on the front panel while a cable connects

between  the MICROPHONE OUT connector on the rear apron and  your

transceiver's  microphone  input connector.  The  two  microphone

connectors on the interface have the the following pin-outs:

Pin       Function

1        Microphone audio

2        PTT "hot"

3        PTT return/ground

4        Microphone ground

A  connector  compatible with your transceiver  input  should  be

installed  at  the  transceiver  end of  the  cable  between  the

interface  and  the  rig.  Consult  the  documentation  for  your

transceiver  for  the  equivalent  pins  on  your   transceiver's

microphone  connector. You can test the connections by  operating

your rig with the interface either on or off. In either case, the

rig  should  key  with normal microphone  audio.  The  microphone

patches  through the interface at all times unless the  interface

is actually transmitting an image.

The cable used to connect the parallel port of the  computer

with the DB-25 connector on the rear apron of the interface is  a

double-male  and  is  usually sold as a  "parallel  cable".  Such

cables  will  have all connector  pins  wired.  Similar-appearing

cable, usually marked "serial cable", will not be fully-wired and

will NOT function with the interface.

Software

If you have a hard drive, the best results will be  obtained

by  operating  the software from the hard disk.  From  your  root

directory, create a FAX480 subdirectory by typing:

md\fax480  <ENTER>

Switch to the new directory:

cd\fax480  <ENTER>

Now  copy  ALL  the distribution files into  this  directory.  In

operation, you would enter the FAX480 subdirectory to execute the

program  and  all  image  files  would  be  stored  in  the  same

subdirectory.  Image file capacity is limited only  by  available

hard-disk storage.

The  FAX480  program  can be operated from  a  floppy  drive

without any penalty other than the number of file images that can

be stored on a single floppy disk. Copy the programs noted  above

on to a formatted disk for the drive in question. Operating  from

a  floppy drive would normally involve booting the software  from

your  program disk, after which you can remove the  program  disk

and insert formatted disks for image file storage. Each fax image

file  requires  128K of available disk storage while  SSTV  image

files use 64K, so you will have the following image capacity  for

various floppy drive configurations:

Drive     Fax Images     SSTV Images

360K           2             5

720K           5            11

1.2 meg         9            18

1.44 meg       11            22

These figures assume that no other files are stored on the  disks

in question.

OPERATION

Booting the Program

The SSTV portion of the FAX480 program inserts a small block

in the upper left of all transmitted images that contains  either

<FAX480> or your call (six characters maximum), depending on  how

you   boot  the  software.  Assuming  you  are  in   the   FAX480

subdirectory, or you are operating from the floppy drive with the

program  disk installed, to enable the call-sign  option,  simply

type:

fax480 urcall  <ENTER>

The call letters will appear in upper-case letters,  irrespective

of  whether you type then in in upper-case or lower-case. If  you

don't  care  about using your call-letters, boot the  program  by

typing:

fax480 <ENTER>

To make booting the software even move convenient in the  future,

you might consider using your word processor (in the non-document

or  ASCI  mode)  to create a batch file (FAX.BAT)  in  your  root

directory. This file would have the following format:

cd\fax480

fax480 urcall

cd\

with the batch file installed, simply typing:

fax <ENTER>

from the root director will move you automatically to the  FAX480

subdirectoruy and boot the software with your call. When you exit

the program, you will be back in the root directory.

As the program boots, a "CHECKING PARALLEL PORTS..." message

will be posted as the computer checks the three possible parallel

port  addresses  looking for the interface.  The  interface  unit

should be connected and turned on PRIOR to booting the  software.

If the program doesn't find the interface unit, an error  message

will be posted, outlining the possible problems. You can continue

at  this point (type <C>) or exit the program (type <Q>). If  you

continue,  all image file functions will be available to you  but

the  program will not let you use the receive modes  or  transmit

modes, since,  lacking a connection to the interface, the program

would "hang up" and require a keyboard or hard reset to get  back

on  line. If the problem was a lack of power to the interface  or

failure  to  connect or switch the parallel  port  cable,  simply

hit  the <T> key to repeat the port test. If you don't intend  to

use the interface for a particular operating session, hit <C>  to

continue.  Almost all program functions will be available to  you

in  such a case, except that the system will not let you  display

or transmit images. Typing <Q> will cause you to exit the program

if your problem requires more in the way of thought or action.

MAIN MENU

This  menu  is  the crossroads of  the  FAX480  program  and

provides access to all three of the primary program modules:

<1> This will cause you to jump to the 480 line fax module

<2> will route you to the SSTV module

<3> will  take  you  to  the  package  of  file  conversion

utilities

<Q> will  permit you to exit the FAX480 program,  at  which

point you will be returned to your operating system.

This  menu  illustrates  a  characteristic  of  all  the  primary

functions of the FAX480 program. With the exception of file names

that  must be entered to load, save, or convert images, all  menu

commands  are  initiated by hitting a single  alpha-numeric  key.

There  is  no need to use the <ENTER> key -  one  keystroke  will

accomplish  each designated menu option. Alphabetical  characters

may  be  entered in either upper or lower case, so  the  keyboard

shift status is irrelevant.

Each of the three functional units will be described in  the

sections which follow.

480 LINE FAX OPTIONS

Fax  functions  are  organized into two  columns  -  RECEIVE

FUNCTIONS  and  OTHER  FUNCTIONS.  The  former  deals  will   the

different  options  for receiving and displaying a 480  line  fax

image  while  the latter includes a variety of  other  functions.

Note  that  the display also includes a real-time  clock  display

that updates every minute. This is included as an aid in  logging

contacts. The display will be in local or UTC time, depending  on

how  you have set your computer with the TIME function from  DOS.

For maximum utility, you should periodically reset the time  from

an accurate source.

RECEIVE FUNCTIONS

<1> Live Autostart

This is the preferred receiving mode since image loading  is

initiated  by the start tone and controlled by the crystal  clock

circuit,  providing the maximum in noise immunity. This  mode  is

used to display transmissions from other stations where the image

is  transmitted directly from the other computer  ("live").  When

you  key  <1>  the image display screen will appear  and  the  **

ACTIVE ** message will be posted to let you know that the  system

is attempting to load an image. You must enter this mode prior to

the start of a transmission from the other station or just as the

start  tone  is heard, as the system must receive a  valid  start

tone to begin image display.

If  you entered the mode by accident, you missed  the  start

tone,  or  there  was a problem with  the  transmitting  station,

simply hit the <Q> key to exit. The ** ACTIVE ** message will  be

replaced  by  ** COMMAND **, at which point you can  key  <Q>  to

return to the Main Menu. See <3> and <4> below for how to display

an image when you have missed the image start tone.

Assuming image transmission proceeds normally, the  received

picture will be displayed, line by line, in the 512 x 480 display

window.  Reception can be aborted at any time by hitting the  <Q>

key, at which point the ** ACTIVE ** message will be replaced  by

** COMMAND ** and you can use <Q> to exit back to the Main Menu.

Assuming  the transmission proceeds to completion, when  the

display  window  is  filled with image data,  the  **  ACTIVE  **

message  will  be replaced by ** COMMAND **. At this  point,  the

received  image  will be retained and you have four  options,  as

indicated by the menu on the left side of the screen:

<S>ave Image

<T>ransmit

<P>rint Image

<Q>uit

See the FAX POST-DISPLAY options for the use of these functions.

<2> Recorded Autostart

This  option functions like <1>, except that it is used  for

transmissions that have been recorded on audio tape. This  option

does  not  have the noise-immunity of the  clocked  display  mode

(noise  or  interference can disrupt  line-triggering)  but  does

provide a useful tape-recording option.

All display and post-display functions are similar to  those

described above.

<3> Live Manual

This  option  provides crystal-locked display  if  you  have

missed  the  image  start  tone. In all  other  respects,  it  is

identical to the preferred Live Autostart option.

<4> Recorded Manual

This  option  provides for recorded display when  the  image

start  tone has been missed. In other respects, this  routine  is

identical to the Recorded Autostart option.

FAX POST-DISPLAY OPTIONS

Any  of  the  fax display options  (1-4)  will  provide  the

following options when image display is complete:

<S>ave Image

<T>ransmit

<P>rint Image

<Q>uit

<S>ave Image

Keying <S> will switch you to a screen prompting you for the

name  to be used for the image save. This name can be  any  valid

DOS  filename (8 characters max.) and should not incorporate  any

extensions. If you are using a floppy drive, be sure a disk is in

the  drive with enough capacity to hold the 128K image.  Normally

the file will be created in the default drive/directory. You  can

save to another drive/directory by including the appropriate path

data  in the file name you provide. NOTE: Files save  to  another

drive/directory  will NOT appear on the listing when you  attempt

to load images from the fax menu. If you want access to the image

you have saved, the file must be transferred at a later time. See

the IMAGE FILES section before attempting to do so. When the save

is complete, you will be returned to the Main Menu. The  received

image will NOT be lost as it is retained in a RAM buffer and  can

be recalled at any time using the DISPLAY LAST IMAGE  option from

the Main Menu or by using the LOAD IMAGE FROM DISK option.

<T>ransmit

Hitting the <T> key will post the transmit menu display  and

send  the image in memory. The system will key the interface  and

begin image transmission. When you use this option for the  first

time,  you  will  need to adjust the MIKE  GAIN  control  on  the

interface  for at RF output reading on your transceiver  that  is

between  1/2 and 2/3 of the PEAK reading normally  observed  with

voice  transmission.  Keeping the audio level  down  will  reduce

distortion  and the possibility of over-driving the  transmitter.

It is also important since fax transmission (like SSTV)  involves

a  100%  duty-cycle and you do not want to  overheat  your  final

amplifier  stage(s).  Using  a  dummy  load  to  make  the   gain

adjustment  is highly recommended. The setting of the  MIKE  GAIN

control  has  no  effect on normal  microphone  audio,  which  is

normally  set by a level or gain control on your transceiver.  If

you  change this control, readjustment of the MIKE  GAIN  control

will probably be required.

Transmission  can be aborted at any time by hitting the  <Q>

key,  at which point the interface will drop out of the  transmit

mode and you will be returned to the fax menu. When  transmission

is  completed  (normally  about  2 minutes  and  8  seconds,  the

interface will drop out of the transmit mode and you will  return

to the Main Menu.

<P>rint Image

Hitting  the  <P>  key lets you print a  hard  copy  of  the

displayed image provided you have a compatible printer. Refer  to

the PRINTING IMAGES section prior to using this function for  the

first time.

OTHER FUNCTIONS

<R>ecall Last Image

If you have previously received an image or loaded one  from

disk,  that image will be scrolled into the active window of  the

display screen. When scrolling is complete (** ACTIVE ** switches

to ** COMMAND **), you have the same options available to you  as

described under the receive modes.

If  you  have  not loaded or received  an  image  since  the

program  was booted, a black screen will be displayed.  IMPORTANT

NOTE:  Although  unlikely, if you do something  that  causes  the

program  to abort, you can usually save the image by  immediately

rebooting  the  program  and calling up the  DISPLAY  LAST  IMAGE

option.  This is due to the fact that DOS does not  normally  use

the RAM area allocated for the image buffer. If you  reboot/reset

the computer, however, the image will be lost since a reset  will

cause the buffer area to be over-written as part of the  power-on

system test (POST). While such image recovery is valid for system

operating  under DOS, I cannot tell you what the results will  be

using  WINDOWS  or any other GUI operating system. You  can  test

this option by exiting the program and then rebooting the  FAX480

program. If the image was corrupted, you may want to operate  the

program from the DOS environment.

<L>oad Image From Disk

When  you key <L> to invoke this function, the names of  all

FAX480  image  files  on  the  default  directory/drive  will  be

displayed.  Each  of these will have an ".A" extension  (see  the

section  on FAX IMAGE FILES). Enter the name of the file to  load

(less  the .A extension) and the image file will be  loaded,  the

system  will  switch to the image display screen, and  the  image

will be transferred to the screen. Any of the secondary functions

(<S>ave,  <T>ransmit, <P>rint, or <Q>uit can be used if  desired.

Note  that you can save the image to the default  directory/drive

under  a  different  file  name  or  you  can  save  to   another

directory/drive  with  the save or a different  file  name.  This

provides a convenient way to copy interesting images to a  floppy

for  backup  purposes or to another disk to  provide  to  another

fax/SSTV operator.

<G>rayscale Display

Keying  <G>  will  fill the entire  screen  with  a  16-step

grayscale display that is ideal for adjusting the brightness  and

contrast  of the VGA display or for setting camera exposures  for

taking  photographs (see PHOTOGRAPHING THE DISPLAY). Hitting  any

key will return you to the Main Menu.

<A>dd Label to Image

This  option  lets  you  add  a  single  line  of  text  (60

characters  max.)  to  the  bottom of  the  image.  Any  text  or

numerical  character is acceptable but some punctuation  (period,

for example) or characters (" quotation marks) are not. You  will

get  a  RE-DO FROM START message if your text entry  contains  an

illegal  character.  Also,  although the  480  line  format  will

resolve lower-case characters, you may want to use upper-case  to

retain maximum readability under less-than-optimum conditions. If

you  enter Q instead of a text line, the routine will return  you

to the Main Menu.

Assuming you have entered a text line, the routine will jump

to the image display screen and show you the image with the label

overlaid  on the bottom. Note that the text, no matter  how  long

the  line, is centered with respect to the image. This  label  is

now in the video RAM buffer so will go out with the image if  you

transmit  it. Also, if you Save the image at this point, it  will

be saved along with the image. As a general rule, you may want to

Save  received images before experimenting with labels so an  un-

labeled version can be recovered if desired. If the image is save

using  the original file name, the labeled version  will  replace

the unlabeled one.

If  you don't like your label, it can be replaced by  keying

<Q> to exit the display screen, followed by an <A> from the  Main

Menu  to do another label. Any new label will completely  overlay

any  previous  one.  Labels on previously  saved  images  can  be

replaced  or edited using this feature. If you want to  eliminate

any  label, reload the image in question, provided it  was  saved

without a label..

The  label  feature can perform several functions.  You  may

wish   to  label  an  image  just  prior  to  transmission   with

information specific to that contact:

WB8DQT TO W1AW  20M  1830Z  20 JULY 1992

You  may also wish to label received images in a similar  manner,

thus permitting your image files to serve as an image log. Labels

can  also be used as captions to "explain" images that are  less-

than-obvious in terms of the subject.

<S>STV Options

Keying  this  option  will  route you to  the  SSTV  menu  to  be

described in a later section.

<C>alibration Tone

This routine causes your interface to transmit one of  three

possible calibration tones (1200 Hz Sync, 1500 Hz Black, or  2300

Hz  White) to assist the other station in precision  tuning  your

signal. You select the tone, depending on the needs of the  other

station.  For communicating with other FAX480 stations,  use  the

1500  Hz  (Black)  option.  The calibration  tone  will  be  sent

continuously until you press the <Q> key. Generally, 5-10 seconds

will  be  adequate  to allow the other  station  to  make  tuning

adjustments.

<T>une Display

Proper  grayscale  rendition  in both  fax  and  SSTV  modes

requires  accurate  tuning/carrier insertion  at  your  receiver.

Usually you can achieve sufficient accuracy by tuning for natural

voice  quality,  but  this can be ambiguous at  times.  The  tune

display  provides  a  graphic  aid  for  precision  tuning.   Two

vertical, parallel lines define the range from 1200 Hz (Sync)  at

the  top of the display to 2300 Hz (White) at the bottom. If  you

are uncertain about the accuracy of your tuning, request that the

transmitting station transmit a 1500 Hz (Black) calibration  tone

prior  to sending an image. Key <T> for the tone display and  you

will  note  a  horizontal line between the  two  vertical  lines.

Carefully adjust the receiver tuning (use the RIT control in  the

case of a transceiver) until the horizontal line is lined up with

the two arrows. These arrows are positioned at the 1500 Hz  point

on the frequency display. When the line is properly aligned  with

the arrows, your are precisely tuned for image reception.

To use the tune display for SSTV, you have several  options.

If  the other station can transmit a short calibration tone  (all

stations  using  the  FAX480 software can do  so),  ask  them  to

transmit a black (1500 Hz) tone and tune as noted above for  fax.

Stations  can also be tuned while transmitting SSTV. If  you  can

note  the bar at the top of the display that corresponds to  sync

(1200 Hz), tune to the point where this is positioned JUST at the

top. Multiple bars should also be noted in the range between  the

arrows  (black) and the bottom of the display (white). The  black

threshold can usually be noted because of the clustering of bars.

If you can see the black threshold, carefully adjust tuning to it

aligns with the two arrows. The rapidly shifting frequency of  an

SSTV signal results in the bars, corresponding to the  individual

frequencies, being fairly faint. They may not be readily  visible

on  a  laptop computer with an LCD screen because of  the  slower

response  time  of  such  a  display.  LCD  displays  will   work

well with the single-tone tuning aid.

No  matter what your tuning mode, hitting the <Q>  key  will

return you to the fax menu.

<Q>uit 480 Line Fax

This  keystroke  will cause you to exit the fax  module  and

return to the Main Menu.

FAX IMAGE FILES

Although  I  have referred to image files in  the  singular,

each  image is actually contained in two 64K binary  files.  This

strategy  allows for fast loads and save of the image  data.  The

file name is made up of the name you selected plus the  extension

A for the upper half of the image and B for the lower half of the

image:

filename.A

filename.B

Within the FAX480 program this two file structure is  transparent

to  the  user but it must be considered if you wish  to  copy  or

delete  files  from the DOS environment. The wildcard  option  is

useful in both cases. Use

delete filename.* <ENTER>

to remove a specific image file or

copy filename.* <ENTER>

to copy an image file.

PRINTING FAX IMAGES

The  <P>rint Image option from the display menu allows  very

high-quality printing from displayed images. The routine supports

LaserJet  compatible printers, including the H-P  DeskJet.  Laser

printers must be equipped with sufficient memory to handle a full

page  of  300 dpi graphics (typically 1  megabyte).  The  DeskJet

requires no ancillary memory for printing.

The  printer  must  be connected to  the  computer  using  a

parallel  printer cable. This presents a practical problem  since

the  ViewPort  VGA interface is also connected  to  the  parallel

port.  An  A/B  switchbox can be  used  to  facilitate  switching

between the interface and the printer. The switch MUST be set  to

the  interface  when the FAX480 program is booted  and  when  you

desire  to receive or transmit a picture. When you want to  print

an  image, you must switch to the printer prior to  invoking  the

print routine.

Hitting the <P> key will initiate the printing process.  The

screen  will  clear  to  white and  the  display  image  will  be

repositioned to the center of the screen. Printing a picture will

require  several  minutes with a typical printer while  the  time

required with the DeskJet will be significantly longer. In either

case,  the result is a printout of very high quality compared  to

other printing routines.

If  you  operational  computer  is not  linked  to  a  laser

printer,  you can copy the FAX480 program and any  desired  image

files  to  floppy  disks  and print out  pictures  on  any  other

computer  that  does have the proper printer. You  will  get  the

error  message  when you boot the software because you  will  not

have  the  A&A  interface connected but  that  will  not  inhibit

loading images from disk and printing them.

SSTV OPERATION

The  basic  fax functions have so much in common  with  SSTV

that  it made a lot of sense to add an SSTV option to  the  basic

FAX480  program.  The  program  will  receive  and  display   the

following formats using a high-quality 64-step grayscale:

8/8.5 second B&W

36 second B&W

12 second Robot color

36 second Robot color

76 second Robot color

Scottie 1 color

Scottie 2 color

AVT 94 color

In addition, the system will transmit image in either the 8 or 36

second  B&W  format. Since the ViewPort VGA system comes  with  a

color SSTV operating program, you might well ask why bother  with

a  piece  of software that only displays monochrome  images?  For

someone  who wants to get on every once and a while  and  operate

SSTV, there are a couple of very good reasons why you may wish to

use this part of the program:

IMAGE  QUALITY - if your VGA monitor will only  display  256

color values, the quality of the color image display is marginal.

If  the received signal is slightly-mistuned, you will  also  get

undesirable  color tints on your images. That same monitor (or  a

monochrome VGA monitor or a VGA laptop) will do an excellent  job

of displaying grayscale images. Mistuning will only cause  subtle

shifts in the grayscale display - not a color tint!

CONVENIENCE  - there is no configuration file to  play  with

since everything is clocked and all operations are independent of

your CPU and its clock speed. This program is also much easier to

use since almost everything works from a single menu.

IMAGE  STORAGE  - all images stored with  the  ViewPort  VGA

program require 123K of disk space. Monochrome images are  stored

in  color format and will be over-laid by color when called  back

up  from disk. This program stores all images in 64K  disk  files

and  save  and  load  almost instantly with  no  color  tint.  In

addition,   information   on  the  original   image   format   is

automatically stored in the image disk file.

ESCAPE - With the ViewPort VGA software, you can get trapped

in  a  receive  routine if the system doesn't get  a  valid  sync

pulse.  This  program features single key exit from  any  routine

without the need to re-boot the computer.

The  program  is simple to use and  the  high-quality  monochrome

image  display  will  typically generate a  nicer  image  with  a

standard  VGA display system then will the color option. You  may

well  find  the  program to be your  preferred  operating  system

unless  you  become a "hard-core" SSTV operator. Of  course,  the

ViewPort  VGA software can be used at any time for color  display

and  transmission  and the fax clock modification  will  have  no

effect  on  the  operation of the  interface  with  the  ViewPort

software.

SSTV MENU

The  SSTV  Menu is organized like the fax  menu,  supporting

both receive and other functions.

RECEIVE FUNCTIONS

The Main Menu provides seven receive display modes. They all

share a number of operational features in common. When a mode  is

selected,   the   screen  will  format  for  SSTV   display,   an

abbreviation  of  the operating mode will appear along  the  left

edge,  as  will  a small white square in the lower  left  of  the

display.  This  square  indicates that the  program  is  actively

processing an image. If display completes in the normal  fashion,

the  square  will  disappear.  In  a  direct  sense,  the  square

functions  to perform the same function as the  <**ACTIVE**>  and

<**COMMAND**>  indicators for fax. If you need to  abort  display

for  any  reason, simply key <Q> and the  square  indicator  will

disappear, leaving you in the equivalent of the fax COMMAND mode.

See the section on Post-Display Options for what you can do  with

the displayed image.

Reliable   SSTV  display  requires  that  the   station   be

accurately  tuned. Results will depend on noise and  interference

levels.  Mis-tuning will result in varying degrees of  line  mis-

triggering,  as will noise or QRM (depending on the  strength  of

the SSTV signal. Absent interference, the noise generated by your

computer system (on HF) will set the noise floor. My Zenith  lap-

top, for example, requires an S4-S5 signal to overcome the effect

of  computer  generated  RF (see the section  on  COMPUTER  NOISE

SUPPRESSION).

<1> 8/8.5 Second B&W

This is the standard low-resolution SSTV image format. While

the  images  are limited in detail, they are  acceptable  if  the

subject is chosen with care and have the advantage that they  can

be  transmitted  very  quickly. They  are  also  compatible  with

virtually  every SSTV system ever built, including  early  analog

equipment.  This receive mode will also display Robot  12  second

color images.

<2> 36 Second B&W <S>

This  is the high-resolution (240 line)  monochrome  display

mode. This format provides a combination of the highest available

SSTV resolution in the shortest possible transmission time.

Note  the  <S>  following this mode. If  you  are  ready  to

receive  another FAX480-equipped station, key <S> when the  image

starts instead of <2>. This will lock the display to the  crystal

clock  and  completely  avoid  the  possibility  of  false   line

triggering.  You  can  only use this  option  with  other  FAX480

stations  since  other SSTV stations  employ  slightly  different

clocking schemes. With most other SSTV systems, the clock is far

enough off to result in a "tilt" to the picture that can range

from mild to severe. Under difficult band conditions, you can use

the synchronous option is the slant is not too bad. Some individual

SSTV units will be almost on the right frequency and the results will

be quite satisfactory. With other FAX480 systems, the images will be

perfectly synchronized if the clock has been adjusted with reasonable

care.

<3> 36 Second Robot Color

This is a popular format because it allows the  transmission

of  240 line images in a short period of time. Image  quality  is

distinctly lower, however, that with either the 36 second B&W  or

72 second Robot Color format.

<4> 72 Second Robot Color

This  is  the  highest quality display in  the  Robot  Color

system and the resulting image will be equivalent to the 36  line

B&W  format. Having the other station send a 36 second B&W  image

will  save time, but most operators will wish to  transmit  color

for the benefit of others on the frequency.

<5> Scottie 1

This  is a popular color display mode that requires about  2

minutes  for  the display of a 240 line  image,  with  resolution

equivalent to the 36 second B&W and Robot 72 second color modes.

<6> Scottie 2

This mode provides results comparable to or slightly  better

than the 36 second Robot Color mode. The main advantage, compared

with  Scottie  1. is the shorter transmission time.  In  general,

Scottie 1 will provide the better quality.

<7> Amiga AVT 94

This  is  a synchronous system that is  difficult  for  most

non-Amiga  systems to handle because the format does  not  employ

sync  pulses and clocking requirements are quite  stringent.  The

FAX480  hardware clock is sufficiently accurate, when tuned  with

care, and the software compensates for the different clock rates,

yielding essentially perfect synchronization. If you want optimum

image  noise immunity when working stations equipped with  Amiga,

Pasokon, or up-graded Robot systems, this is the mode of  choice.

When  working  other  stations using  the  FAX480  software,  the

synchronous  36 second B&W option will provide  equivalent  image

quality in about 1/3 the transmission time.

Reception of AVT94 images requires that the transmitting

station be tuned VERY accurately if the software is to lock

properly to the image header. Tuning errors of only 100 Hz will

result in image display being aborted. If the system has locked

on to the header properly, you will see the display start at the

top of the screen. You should key up the mode just as the VIS

data ends and the 5 second header begins for best results. If the

system fails to lock on to the header data, the square block in

the lower left of the screen will go out without the image being

displayed. There is no way to display the image if the receiver

was mis-tuned during the header interval and the <Q> key should

be used to return you to the SSTV Menu.

OTHER FUNCTIONS

<R>ecall Last Image

Whenever you use the <Q> key to exit the display screen (see

POST DISPLAY OPTIONS), the image is not lost but is saved to disk

as a 64K file (TEMP.STV). This image can be retrieved at any time

using  the <R> key. The complete range of functions described  in

the section on POST DISPLAY OPTIONS are then available.

<L>oad Image from Disk

When  you key <L> to invoke this function, the names of  all

SSTV   image  files  on  the  default  directory/drive  will   be

displayed.  Each of these will have an ".STV" extension (see  the

section on IMAGE FILES). Enter the name of the file to load (less

the extension) and the image file will be loaded, the system will

switch  to  the  image  display screen, and  the  image  will  be

transferred to the screen. See the POST DISPLAY OPTIONS for  what

you can do at this point.

<G>rayscale Display

Keying  <G>  will  fill the entire  screen  with  a  16-step

grayscale display that is ideal for adjusting the brightness  and

contrast  of the VGA display or for setting camera exposures  for

taking  photographs (see PHOTOGRAPHING THE DISPLAY). Hitting  any

key will return you to the Main Menu.

<P>rint Files

Printing files takes a fair amount of time, using the  high-

resolution  image printing routine and it makes little  sense  to

devote  an  entire page to SSTV images. This routine is  a  batch

printing  routine  that will place up to four SSTV  images  on  a

sheet  of paper. When you activate this option with the <P>  key,

you will get a display of the available SSTV image files and  you

will  be  requested  to supply four file names  in  sequence  (no

extensions).  Any image can be skipped by hitting <ENTER> at  the

request.  In  effect,  you can print from  one  to  four  images.

Printing  four  at a time is  a better investment,  in  terms  of

printing time, but the choice is yours. If the format of the four

images is important to you, image 1 is in the upper left, image 2

is in the upper right, image 3 is in the lower left, and image  4

is in the lower right of the printed page.

Once file names have been selected, the four images will  be

formatted on the screen. This will take several minutes, followed

by  the  actual  printing routine. You will  find  the  resulting

prints  quite satisfactory, compared with other SSTV image  print

routines.

<C>alibration Tone

This routine causes your interface to transmit one of  three

possible calibration tones (1200 Hz Sync, 1500 Hz Black, or  2300

Hz  White) to assist the other station in precision  tuning  your

signal. You select the tone, depending on the needs of the  other

station.  For communicating with other FAX480 stations,  use  the

1500  Hz  (Black)  option.  The calibration  tone  will  be  sent

continuously until you press the <Q> key. Generally, 5-10 seconds

will  be  adequate  to allow the other  station  to  make  tuning

adjustments.

<T>une Display

Proper  grayscale  rendition  in both  fax  and  SSTV  modes

requires  accurate  tuning/carrier insertion  at  your  receiver.

Usually you can achieve sufficient accuracy by tuning for natural

voice  quality,  but  this can be ambiguous at  times.  The  tune

display  provides  a  graphic  aid  for  precision  tuning.   Two

vertical, parallel lines define the range from 1200 Hz (Sync)  at

the  top of the display to 2300 Hz (White) at the bottom. If  you

are uncertain about the accuracy of your tuning, request that the

transmitting station transmit a 1500 Hz (Black) calibration  tone

prior  to sending an image. Key <T> for the tone display and  you

will  note  a  horizontal line between the  two  vertical  lines.

Carefully adjust the receiver tuning (use the RIT control in  the

case of a transceiver) until the horizontal line is lined up with

the two arrows. These arrows are positioned at the 1500 Hz  point

on the frequency display. When the line is properly aligned  with

the arrows, your are precisely tuned for image reception.

To use the tune display for SSTV, you have several  options.

If  the other station can transmit a short calibration tone  (all

stations  using  the  FAX480 software can do  so),  ask  them  to

transmit a black (1500 Hz) tone and tune as noted above for  fax.

Stations  can also be tuned while transmitting SSTV. If  you  can

note  the bar at the top of the display that corresponds to  sync

(1200 Hz), tune to the point where this is positioned JUST at the

top. Multiple bars should also be noted in the range between  the

arrows  (black) and the bottom of the display (white). The  black

threshold can usually be noted because of the clustering of bars.

If you can see the black threshold, carefully adjust tuning to it

aligns with the two arrows. The rapidly shifting frequency of  an

SSTV signal results in the bars, corresponding to the  individual

frequencies, being fairly faint. They may not be readily  visible

on  a  laptop computer with an LCD screen because of  the  slower

response  time  of  such  a  display.  LCD  displays  will   work

well with the single-tone tuning aid.

No  matter what your tuning mode, hitting the <Q>  key  will

return you to the fax menu.

<F>ax Menu

Keying <F> will return you to the fax menu.

<Q>uit SSTV

Hitting  <Q>  will cause you to exit the  SSTV  program  and

return to the Main Menu.

SSTV POST-DISPLAY OPTIONS

When  an image has been received, recalled, or  loaded  from

disk,  you  will note letters arrayed vertically along  the  left

side of the display:

<S>

<T>

<Q>

<C>

<R>

The  five  letters  correspond to your options  at  the  point  -

<S>ave, <T>ransmit, <Q>uit, <C>ontrast expansion, and <R>epair.

<Q>uit

Keying  <Q> will return you to the SSTV menu.  The  received

image will NOT be lost and can be recovered at any time using the

<R>ecall Last Image option from the SSTV menu.

<S>ave Image

Keying <S> will switch you to a screen prompting you for the

name  to be used for the image save. This name can be  any  valid

DOS  filename  (characters max.) and should not  incorporate  any

extensions. If you are using a floppy drive, be sure a disk is in

the  drive with enough capacity to hold the 64K  image.  Normally

the file will be created in the default drive/directory. You  can

save to another drive/directory by including the appropriate path

data  in the file name you provide. NOTE: Files save  to  another

drive/directory  will NOT appear on the listing when you  attempt

to  load  images from the Main Menu. If you want  access  to  the

image  you  have saved, the file must be transferred at  a  later

time.  See  the IMAGE FILES section before attempting to  do  so.

When the save is complete, you will be returned to the Main Menu.

<T>ransmit Image

Hitting  the <T> key will cause the screen to clear and  you

will be presented with the following transmit options:

<1> 8/8.5 Second B&W

<2> 36 Second B&W

<Q>uit Transmit

Keying <Q> will cause you to exit the transmit menu and  the

image will be re-posted. Keying either <1> or <2> will cause  the

image to be re-posted, the interface will key up, and the current

image will be transmitted in the selected format.

When  you use this option for the first time, you will  need

to adjust the MIKE GAIN control on the interface for at RF output

reading  on your transceiver that is between 1/2 and 2/3  of  the

PEAK  reading normally observed with voice transmission.  Keeping

the  audio level down will reduce distortion and the  possibility

of  over-driving the transmitter. It is also important since  fax

transmission  (like SSTV) involves a 100% duty-cycle and  you  do

not want to overheat your final amplifier stage(s). Using a dummy

load  to  make  the gain adjustment is  highly  recommended.  The

setting  of  the  MIKE  GAIN control  has  no  effect  on  normal

microphone  audio,  which  is normally set by  a  level  or  gain

control  on  your  transceiver.  If  you  change  this   control,

readjustment of the MIKE GAIN control will probably be required.

Transmission  can be aborted at any time by hitting the  <Q>

key,  at which point the interface will drop out of the  transmit

mode and you will be returned to the Main Menu. When transmission

is  completed (in either 8.5 or 36 seconds), the  interface  will

drop  out  of the transmit mode and you will return to  the  Main

Menu.

Both  transmit image formats incorporate the standard  Robot

VIS codes in the vertical sync pulse. This means that Robot 400C,

450C,  and 1200C scan converters, Amiga AVT systems, the  Pasokon

system  for the IBM, and any other display systems which use  the

VIS codes will be able to automatically switch to the proper mode

with the receipt of your vertical sync pulse. Those with  display

systems  that require manual display mode selection need to  know

what  mode  you will be using, so you should clearly  state  your

option prior to beginning transmission. Even those whose  systems

normally  trigger on the VIS-coded may need the mode to  manually

trigger their system if noise or QRM disrupts the VIS interval.

<C>ontrast Expansion

Some  SSTV  images you will encounter will  have  relatively

poor contrast. This may be because of the nature or the  original

subject, poor lighting or other factors at the transmitting  end,

or slight mistuning at your end. Regardless of the cause,  keying

<C>  from the post-display menu will give you the option to  look

at  the  pixel  distribution of the image  and  correct  contrast

deficiencies  if you wish. The option takes a little time,  so  I

suggest you use it for images you have saved to disk, rather than

attempting to adjust pictures while you are on the air.

When  you key <C> the system will BEEP, indicating that  the

command was recognized. NOTHING else will happen for the next few

minutes  but  be patient. When analysis is complete,  the  screen

will clear and you will be presented with a histogram showing the

distribution  of pixel values from black at the left to white  on

the  right. The further the lines extend down from the  baseline,

the more pixels have that particular value. If the vertical lines

are relatively widely spaced and about 16 in number, the original

image  was probably transmitted with 16 grayscale steps.  If  you

key  <Q> at this point, you can exit without changing the  image.

Otherwise, proceed as follows.

The  first step is to indicate where you want the new  black

cutoff  to  be. You will notice a short vertical  line  extending

above  the  baseline at the far left. Pressing the <+>  key  will

move  this  mark toward the right (white end  of  the  grayscale)

while pressing <-> will move it back toward black (assuming it is

not already at the far left. If there are few pixels at the black

end of the grayscale, move the marker to the right (using the <+>

key,  until you get to a point where the number of pixels  begins

to  increase  -  then press the <B> key to  mark  the  new  black

threshold.  If there is a good clustering of pixels at  the  left

side of the display, simply key <B> with the marker positioned at

the far left.

Once  you have keyed <B>, the marker will re-appear  at  the

white  (right) end of the display. Move the mark ( using  <+>/<->

to the point where you want the white cutoff be be and key <W> to

mark the white threshold.

At this point the display will clear and the original  image

will  re-appear. As you watch, it will be overwritten  using  the

new pixel distribution. When complete, you can save the image  or

quite  in  the normal manner. If the contrast-expanded  image  is

superior,  save  the image using the original file name.  If  you

aren't sure, save it with another name and compare the two  using

the Disk Image Load routine.

For those of you with some image processing background, this

routine performs a "pixel slide" to the black threshold, followed

by a pixel expansion keyed to the new white threshold.

<R>epair

One of the common ways in which an image can be degraded  is

improper  line triggering. This can be caused by improper  tuning

or by interference or noise. Commonly, the line or lines will  be

offset  to  the  right.  The  <R>epair  routine  allows  you   to

reposition such offset lines, greatly improving the appearance of

the image. The routine takes a bit of time, but is worthwhile  in

the  cases  where  you want the best possible  appearance  for  a

significant image.

When  you key <R> a short white line will appear at the  top

and  to  the left of the image. This line marks the  line  to  be

repaired.  Keying  <D> will move the reference  line  DOWN  while

keying  <U>  will  move  it UP. When  the  marker  is  positioned

opposite  an offset line (look CAREFULLY), pressing the  <A>  key

will  cause the line to be offset to the LEFT by one  pixel.  The

<A> key can be pressed as often as desired to properly align  the

offset  image line. The <U> or <D> keys can then be used to  move

the scribe to other lines, where the process can be repeated.

When the lines have been repositioned to your  satisfaction,

hitting <Q> will terminate the repair routine. At this point, you

will  be  prompted  for whether you want the  repaired  image  to

replace  the  original image file. Keying <Y>  will  replace  the

original  image  while keying <N> will retain the  original  disk

file. At that point you will be returned to the normal SSTV post-

display screen. If you elected NOT to replace the original image,

it can be save to another file-name using the <S> routine.

SSTV IMAGE FILES

All  images  are  saved  as  64K  binary  files,  permitting

extremely  fast loads and saves. Each image is also labeled  with

an  .STV file extension. Image files can be deleted,  copied,  or

re-named,  just  like any other data file. If you rename  a  file

under DOS, be sure to add the extension to the name. If you  fail

to  do  so,  the image will not appear  on  the  image  directory

listing when you load a file.

FILE CONVERSION MENU

Keying  <3>  from the Main Menu will route you to  the  File

Conversion  Menu.  Prior  to  discussion  of  the  various   file

conversion options, some discussion is in order regarding sources

of  images and steps preparatory to conversion. Lets  Start  with

the latter.

SCREEN IMAGE CAPTURE

The  key  to all the conversion options is  the  ability  to

capture  computer  images from various sources into  a  form  the

conversion  programs can manipulate. This is accomplished with  a

family of programs called "screen capture" software. As noted  in

the  original  article on the fax standard, I prefer  the  VGACAP

program available on the DRIG BBS (see the February 1993 issue of

QST  for information on this system). This is a TSR program  that

sits  in  the background until you have an  image  displayed,  at

which point you invoke the "capture" by keying <Alt-F4). At  this

point,  VGACAP takes over and converts the screen image  (without

altering it) to a binary data file. If the image was in the 640 x

480  x  16 format (suitable for fax), the result will be  a  307K

binary  file containing all the data for the screen  image.  This

file will reside in the current directory and will be names:

screenxx.raw

where  xx  is  a number from 01 to 99, depending  upon  how  many

screen  captures  you have executed since booting  VGACAP.  There

will also be a matching palette file (screenxx.plt), which we  do

not  need.  Other  capture programs will  operate  in  a  similar

manner,  but  I  would suggest the use of  VGACAP  for  the  most

predictable  results. The file conversion routine has one  option

for converting these 307K files to 480 line FAX480 files.

If  the  original image was in the 320 x 200 x (16,  64,  or

256)  format, the result will be a 64K binary file with the  same

name convention as the larger 307K files, but with the  extension

".bld"  instead of ".raw". The file conversion module contains  a

routine  for  converting  these 64K files to  FAX480  SSTV  image

files.

IMAGE VIEWERS

Many  image  options (see below)  will  support  full-screen

display of images they create, but if this is not the case,  they

will  support conversion of the images to one of several  popular

formats  (.GIF,  .PCX, .TIF, .IMG, or .TGA). In such  cases,  you

need  a  viewing  program that will import the  image  files  and

provide  the  full-screen  display needed for  capture  as  noted

above. There are many programs that will do this, but one I  have

had  good success with is VUIMAGE, a shareware package  available

on  the DRIG BBS. This is an excellent program  supporting  .GIF,

.PCX, and .TIF display, a wide range of monitor modes, and a very

wide range of scaling options. Most of my images, from a  variety

of  sources,  are displayed using VUIMAGE to execute  the  actual

screen capture. I set the mode to 640x480x16 for high  resolution

images  I  intend to capture for fax files,  or  320x200x256  for

generating  64K  files for direct conversion to SSTV  format.  By

adjusting the scale, I can get just the screen display I wish  to

capture,  gaining  considerable control over the  dimensions  and

coverage of the ultimate image file.

SOURCES OF IMAGES

Hand  scanners  and  flatbed scanners can also  be  used  to

generate images and most of these devices come with programs that

support the 640 by 480 x 16 VGA mode. Programs such as VGACAP can

be used to capture the screen images as described above.

TV  frame  grabbers  can  be  used  if  they  support   high

resolution  image  capture.  Most inexpensive  boards  will  only

capture 240-256 video lines. In such cases, it makes little  sense

to transmit these as fax, since 240 line SSTV formats will  match

the  resolution  of  the  captured images.  Such  images  can  be

captures  directly  from  the  application  or  using  a   viewer

(320x200). Higher quality frame grabbers will capture a 512  line

image (using both interlaced frames that make up a TV picture). A

high  quality  black  and white  camera  will  typically  produce

sharper  images  than  a  comparable  color  camera.  These  high

resolution  frame grabbers usually support 640 x 480 VGA  display

so  programs  like VGACAP can capture the  images.

Other sources, such as paint programs, image libraries,  BBS

sources,  etc. can all provide images suitable for conversion  to

either  fax  (if the original has a  high-enough  resolution)  or

SSTV.

COLOR VS. MONOCHROME

Image conversion is quite predictable with monochrome images

and most unpredictable with color. B&W hand-scanners and TV image

capture boards generate such monochrome images automatically  and

present no problems when used with viewers such as VUIMAGE.

Color images must be converted to monochrome prior to screen

capture  is acceptable results are to be obtained.  Many  viewing

programs  allow  color images to be displayed with  a  monochrome

palette (VUIMAGE, despite its other useful qualities, does NOT!),

in  which  case you can screen grab with  good  results.  Lacking

other options, color images can be converted to monochrome  using

the  ViewPort  VGA  software  that  comes  with  your  interface,

provided the .PCX, or .TGA images have the correct resolution  to

start with (see your program documentation). Assuming the  VGACAP

program is resident, follow the steps below:

(1) Boot the ViewPort software

(2) Use the <ESC> key to get the OTHER OPTIONS menu.

(3) Load the image (<F7> for .TGA or <Shift-F7> for .PCX)

(4) Once the image has loaded, use <ESC> to get back to  the

OTHER OPTIONS menu.

(5) Use <F10> to get to the test and diagnostic routines

(6) Key <Y> to display the Luminance data

(7) Key <Alt-F4> to capture the screen

Images  captured  in  this  way will  produce  64K  binary  files

suitable for SSTV conversion.

All of this sounds very complex to start with, but you can't

hurt  anything  by  experimenting.  Once  you  have  played  with

grabbers,  viewers,  and  images for a bit, it  will  all  become

second nature.

IMAGE CONVERSION OPTIONS

The  Image  Conversion  menu  supports  three  options   (in

addition  to  <Q> to exit the conversion menu).  All  share  some

common characteristics on that they will ask for the name of  the

file  to convert and the name for the converted file.  The  three

options are:

<1> 307K binary files to 480 line fax

This  program will convert the 307K screen binary  files  to

FAX480 fax image files. Be sure to specify the entire name of the

file  to convert (ex. screen05.raw), as well as directory  and/or

path  data  if  the  source  image file  is  not  in  the  FAX480

directory.  Conversion will take several minutes but  image  line

numbers  will  be  posted  to let you know  how  the  process  is

proceeding.  When complete, you can retreat to the fax  menu  and

view  the new image. To save disk space, the routine  erases  the

original 307K binary file when conversion is complete.

<2> 480 line fax to SSTV

This  routine will create an SSTV image file from  a  FAX480

fax  file. The original fax image file is retained and  the  SSTV

image is posted on the screen as conversion proceeds.

<3> 64K binary files to SSTV

This  will convert 64K binary screen grab files directly  to

SSTV  format. The SSTV image is displayed as conversion  proceeds

and  the  original  binary  file is  erased  when  conversion  is

complete.

Note that routines <1> and <3> do erase the original binary files

(307K and 64K, respectively, but they don't so anything with  the

smaller  palette files (.plt extension). You should  periodically

sweep your directories and erase such files. The

erase screen*.*

command  should  do the job as long as you have  no  un-converted

screen files in the same directory.

PHOTOGRAPHING IMAGES

Photography provides another way to archive images you  have

received. A 35 mm camera, equipped with a close-up or macro  lens

and  black  and white film, will do the job if  positioned  on  a

tripod. The display image on the VGA monitor should be framed and

focused  with  the camera so that it fills most of the  field  of

view.  To avoid a diagonal artifact on your pictures,  caused  by

the  camera focal plane shutter, set the exposure time to 1/4  to

1/2  second. The f-stop can be adjusted for the  proper  exposure

using the grayscale display from the main menu and your  camera's

built-in light metering.

When making photographs, keep the ambient light level low so

you  don't "wash out" the screen. You also need to be  sure  that

bright objects such as windows or lamps are no reflecting off the

monitor  screen  as viewed in the camera viewfinder. Due  to  the

high resolution of the fax format, the resulting photographs will

tolerate  a  high degree of enlargement (typically up to 8  x  10

inches) while retaining acceptable image sharpness.

FUTURE OPTIONS

When  I first created the FAX480 program, it supported  only

480 line fax operation. When I realized how effective the  system

was  going  to be, I added a few basic SSTV options to  make  the

program  more  attractive.  The present for of  the  SSTV  module

reflects  added features I wanted when I actually put the  system

on the air. It is safe to say that the SSTV module has grown  far

beyond that which I had originally intended. Since the program is

being  made  available as freeware, you cannot expect  be  to  be

quite  as  aggressive in adding new features as I would be  if  I

were selling the program. None-the-less, there are a few  options

that are likely candidates for future upgrades:

(1) HF fax receive modes

The  ViewPort interface provides all the hardware  resources

needed  for  reception  and display  of  wirephotos  and  weather

satellite photos and charts transmitted on HF. No transmission in

these modes will be supported since all are too long, in terms of

the time required for transmission.

(2) Amiga AVT fax modes

The  Amiga  AVT system supports a 640 x 400 fax  mode  which

could be added without too much difficulty.

The  extent  to  which I do upgrades  is  dependent  on  two

factors - the size of the program user base and the enthusiasm of

users  for  specific  features. If lots of folks  are  using  the

program and I get comments on-the-air or through my BBS (517-676-

0368), I will be more prone to make upgrades available.

One  thing I won't do is mess with color display. The  whole

intent of this project was to make an affordable system available

for  high-resolution imagery, with the SSTV features added  as  a

bonus.  John  Montalbano is doing an excellent job  in  providing

color options using the ViewPort interface and I can see no point

in  re-inventing  the wheel, particularly since his  software  is

exceptionally reasonable in terms of price.

Mason, MI                                January 1993

###########################################################################################