������� ������� �������� ������� ����� ���������

���߱�� ���߱�� �������� �������  ����  �߱�����

������� ������� ��� ���� �����    ���     ���

������� ���߱�� ���  ��� �����    ���     ���

���     ��� ��� �������� ������� �����    ���

��      ��  ��  �������  ������  ����     ��

������� ���     ����� �������  ���   ��� ������� ������� �������

������� ���      ���� �������  ��۱�ܱ�� ���߱�� ���߱�� �������

������� ���      ���  ���      ��۱�۱�� ������� ������� �����

��߱�� ���      ���  ���      ��۱�۱�� ���߱�� ��۱��� �����

������� ������� ����� �������  ��������� ��� ��� ��� ��� �������

������  ������  ����  ������   ��������  ��  ��  ��  ��  ������

Registered U.S. Trademark

PageIt Ver 1.7 Registration $15.00  (504) 288-6550

Gary M. Raymond

P.O. Box 8184

New Orleans, La.  70182

CompuServe 70613,3165

Voice Support 504-288-6550

Welcome to PageIt Reader Beeper modem message transmitter, Ver 1.7

PageIt is designed to run on any IBM/Clone PC/MS-DOS PC using dos 3.3

or later. It requires 128k of RAM and a Hayes compatable modem capable

of running 300 baud. PageIt supports COM1 thru COM4 at IBM default

addresses of COM1(3F8/IRQ4), COM2(2F8/IRQ3), COM3(3E8/IRQ4) and

COM4(2E8/IRQ3).

TECH STUFF: This software is based on the IXO Telocator protocol

and has been tested on most major brands of alpha numeric pagers.

DO NOT confuse Alpha Numeric with ordinary digital pagers.

==========================================================================

With PAGEIT, your PC and Modem, you may send script messages directly

to alpha numeric beepers without paying the fee of a local dispatcher.

PAGEIT is designed to operate simply and easily and can be learned by

anyone within minutes.

PAGEIT uses the latest coding techniques to directly address the serial

hardware of your PC, thus insuring greater compatability with a wider

range of computers and operating systems. PAGEIT also allows changing

of the modem initialization string (contained within the program module)

to provide even greater modem compatability.

==========================================================================

How PAGEIT operates:

*******************************************************

PAGEIT has two general modes of operation. Automatic and Manual. In

the automatic mode, you can create a simple three line ascii text

file that contains your Systems ID code on the first line (use a

capital "M" if you don't know it) followed by your pager's digital

modem access number on the second line and your pager phone number on

the third line. Some of this information is obtained from the dealer

where you purchased the Reader Beeper. When PAGEIT # is typed at the

dos prompt, it automatically looks for PAGEIT.DIR (note: a sample

PAGEIT.DIR file is contained in the self uncompressing archive

file called PAGEDIR.EXE) and if found, loads the three lines of

information into memory and immediately ask for the 80 character

message to send. This eliminates the user from having to manually

type the first three data fields and saves time when constantly

calling the same reader beeper. If the file (PAGEIT.DIR) is not

found, PAGEIT reverts to manual mode and prompts for the System ID

Code (remember, use capital M if you don't know it) followed by the

System Modem access phone number and your pager's phone number.

Finally, you will enter the message to transmit which can be up to

eighty characters.

CAUTION: Don't unpack the PAGEDIR.EXE unless you intend to use it.

If the file is present, PAGEIT will NOT operate MANUALLY!

Example of content of PAGEIT.DIR

--------------------------------

M                               <-- System ID (Use capitol M if unsure)

*70,1-407-339-8588              <-- System Modem Access number

645-9083                        <-- Your pagers phone number

--------------------------------

Note that the Call Waiting temporary defeat switch (*70,) has been

added to line two to prevent disruption if you get an incomming

call while PageIt is active.

To begin using PAGEIT, you must determine what free serial port exist

on your PC. This must be a port without other serial devices competing

for the same serial interrupt vector. Example: If you have a mouse on

COM1, you cannot also use COM3 because they share the same IRQ4 line.

Your modem must also be configured on a hardware level to be compatable

with your PC. Generally, if you are already using a serial communications

program like ProCom etc. you need only determine what port the comm

is sucessfully using.

The next step is to tell PAGEIT what port to use. This is as easy as

typing the port number after PAGEIT, i.e. "C:>PAGEIT 4". In this

example, comport 4 is selected.

A second parameter may be passed to PAGEIT to force it into a MONOCHROME

screen mode that is more suitable for using on some PC's and LAPTOPS.

This is accompolished as "C:>PAGEIT 4 M" where the "M" tells it to

remain in monochrome (black and white) mode.

A third parameter is used ONLY when it is necessary to reburn a new

modem initialization string into the body of the code. Example, some

modems will error out with the &D2 command and others will not hang

up correctly without it. Some modem manual reading and or

experimentation may be necessary. A quick way to test a modem command

string is to enter it from terminal mode via your favorite comm

program (example : ATM1) and see if you get an OK or ERROR reply.

PAGEIT comes with a generic string already loaded, which is

"ATM1V1X4S7=45". It may also be necessary, depending on the brand of

modem, to add &C1 and or &D2 and or N0 and or B1. If it is necessary

to reburn a new string, it must be the THIRD parameter on the command

line as in this example: "C:>PAGEIT 4 M ATM1&D2V1X4S7=46S11=70"

1 2 3.....................

The newer high speed (9600/14400) modems generall need the N0 and or

B1 switch to force the modem into the old Bell 300 baud standard

rather than the later CCITT protocol.

When reburning a new string, you will note the drive light turn on for

a few seconds as the new code replaces the old. The next time you run

PAGEIT, the new string will be automatically used.

EXAMPLE OF OPENING SCREEN and three prompts:

...............................................................................

� PageIt Ver 1.7 Copy(c)right 1993 Gary Raymond    504-288-6550 to order! �

UnRegistered Evaluation Copy - Expires Dec 30, 1993

P. O. Box 8184, New Orleans, La. 70182

Gary M. Raymond, Programmer

Enter System User ID -> M

System Modem Access  -> *70,733-7766

Enter Pager Phone #  -> 831-5110

Enter Message on line below, (80 character limit), Hit <Enter> to begin Sending

#-----------------------------------------------------------------------------#

Next service call: Mrs Jones 123 Apple Lane, go to rear door and knock #26543

...............................................................................

When you complete your message (limited to 80 characters) hit the

Carriage Return or <Enter> key to begin transmission. You will then

see on screen the modem being initialized, you should hear the modem

begin to dial, a CONNECT prompt appear if a good hardware handshake

occured, an ID= request, a pager number request, a message request,

each automatically filled in by PAGEIT until a final prompt advises

that your message was sucessfully transmitted and PAGEIT terminates

the connection and restors your dos prompt.

NOTES: PAGEIT is smart enough to recognise a BUSY signal and attempt

to redial again until a ring is detected. You may break out of

ANY part of PAGEIT's operations by hitting ANY key!

*******************************************************

The Registration is $15.00. You will receive the latest version which

will include any new advanced features and the datelock restriction

removed. The registered version will also allow messages up to 255

characters for those Pagers that support it.

PAGEIT was written in Pascal 7.0  Source code is also available. PUR

Name_____________________________________________________________

Mailing Address__________________________________________________

City & Statre ___________________________________________________

ZIP _____________________________ Phone _________________________

Shipped on 5.25 DSDD floppy unless other noted here: ____________

Send $15.00 check, cash or money order to:

********************************************

Gary M. Raymond

P.O. Box 8184

New Orleans, La.  70182

CompuServe 70613,3165

Voice Support 504-288-6550

EOF()