TN210-8.DOC

USER COMMAND LIST, PT II

****************************

PARMS  (P) - (Parameters) Issuing this command will yield a status  listing

of the node's  parameters.   There are 23 SYSOP  settable parms in version 2.10

and the node response may look like this:

SKY:W7YB-3}  86 100 255 6 5 1800 6 7 10 1 0 30 3 0 1 100 32 240 2 3 180 4 4

Each parameter  affects the  node operation  in one way  or another.    The

values chosen for one  node will impact the operation of the other nodes in the

network.   The convention is to number the parameters from left to right in the

example above, starting 1, 2, 3, etc.   Users will not be able to change any of

the parms unless recognized as a SYSOP.    However  users may find various parm

values  of  interest, such as in finding path  quality and timeliness of routes

between nodes.

ROUTES (R)  -   This command yields a listing of all radio line-of-sight or

wire-connected nodes  "known"  to the node, including those  set  by the  SYSOP

routes   locking  commands.    Due to AX.25 protocol  variations,  Thenet  does

not automatically recognize KA-Nodes,  ROSE, TEXNET,  nor FLEXNET nodes in it's

ROUTES list.  It will recognize G8BPQ, MSYS and TCP/IP nodes.  A typical ROUTES

display may look like:

WTEX:W5CDM-3} Routes:

0 ODSA01:W5CDM-1 192 30

> 0 DKC:W2RRY-2 192 23

> 0 MAF:KE5PL-1 192 10

0 FST:N5DCZ-1 192 16

0 RKN:KE5PL-3 192 11

0 BGS:KE5PL-4 192 5

Shown in  column 1,  all of the connections are  via direct radio  port (0)

paths.   The "right arrow"  indicator  tells us  two of the paths are either in

use  or have had  activity within  the past 15 minutes.  All radio paths show a

standard  path quality value of 192.  The last column indicates the most active

node in the list  is ODSA01   with  30 destination  routes showing.   The least

active is BGS with 5 destination routes.

It is  possible  for  the NodeOp  to lock a non-node alias or callsign into

the ROUTES list.    This is done to indicate  to the observer that a non-TheNet

node is available.  An example might be a KA-node gateway:

MIST:KB7IVK-1} Routes:

> 0 AST:W7FBM-1 192 54

0 V1KING 192 0!

0 AST2:W7FBM-8 192 1

In  this example V1KING is a KA-node  gateway from the local LAN to 28.103.

The  "0" in the number of destination routes column normally indicates a failed

path.   An astute observer would have checked the INFO section and learned that

V1KING  was a gateway.    Also he would  have  noticed  the V1KING entry in the

ROUTES  response was a locked pseudo  call-sign that should  tip him  off there

was something unusual about the entry.    A connect from  MIST  to V1KING would

result in the user being connected to the KA-node.

USERS (U)  -  A command response might appear as follows:

AZ:N7OO-3} TheNet Plus 2.10st (713)

Uplink(AA5EK)  <-L4->  Downlink(AA5EK K1MEA-7)

Circuit(N5KGP-1 N5MYI)  <-L4->  Circuit(ONT:VE3KYZ-4 N5MYI)

Thrulink(BISBEE:WA7KYT-1)  <-L3->  Thrulink(ONT:VE3KYZ-4)

Circuit(SVALAN:N7OO-4 K1SC)

Uplink(DF2AU)  <..>  CQ(DF2AU-15)

The first line identifies the node type and version.    This information is

valuable  to  the  experienced user since it provides insight into the commands

supported by the node.     Command support  is similar among the different node

types and their versions, but there are significant differences.  The number in

parentheses indicates the free buffers  (36 byte segments) currently available.

The  amount of free buffers  varies and is dependent upon the total destination

and neighbor nodes listed in the tables,  as well as the number and activity of

it's users.    Free buffers can be used as a "health indicator".   Upon initial

startup, the value will be in the "720" range.     Around the "450 - 500" area,

free  buffer  depletion  may  cause the node to respond with a  "Node busy"  to

command requests.

Version  2.10 introduces the <-L3->, <-L4-> symbols and the term "Thrulink"

to the  USERS  display.    In the example above, the term "Uplink" refers to an

OSI  level  2  connect  from a local user to the network node.    "Downlink" is

defined  as  an  OSI level 4 network connect to a local user.    A "Circuit" is

either an OSI  level 4 network connect from a source node TO the local node, or

it  may  be  a connect  FROM the local node to a destination node.   "Thrulink"

displays an OSI level 3 circuit from a source node to a destination node.  As a

level 3 circuit, there may be more than one user traversing it bi-directionally

from end-to-end.   User identity can be accomplished by connecting to either of

the  nodes  listed  and performing a USERS command.   Inactive level 3 circuits

will timeout after 5 minutes.