Int09 version 0.1 (beta)

By Celso Minnitti Jr. Public domain software. Copy freely.

INT09_01.ZIP has these files:

�����������������������������������������������������ͻ

� INT09.ASM     8086 Assembly language source code    �

� INT09.COM     Executable program                    �

� INT09.DOC     This document                         �

� I.BAT         Batch to compile and link INT09.ASM   �

�����������������������������������������������������ͼ

INT09 is a very simple program designed to explain to everyone how

the Interrupt 9 works. The INT 9 is the Interrupt ReQuest #1 (IRQ1) of the

Intel's 8259 Programmable Interrupt Controller (PIC) that you have in your

PC. So every time one key is pressed AND the bit 1 of I/O port 21h is clear

,set to 0, one int 9 will occur.

The port 60h will carry the scan code data of the key that has been

pressed. For instance, the key 'A'=1Eh, 'S'=1Fh, 'D'=20h, 'F'=21h, and so on.

Is the BIOS task to translate the scan code data of key 'A' to the Keyboard

Buffer at 40:1Eh.

That means you will not read the ASCII value 41h for the key 'A'

at the I/O port 60h. Instead it will be 1Eh.

See the INT09.ASM for more details...

IRQ List

��������

IRQ0  INT 08	Timer interrupt

IRQ1  INT 09	Keyboard Interrupt

IRQ2  INT 0A	LPT2

IRQ3  INT 0B	Serial Communication (COM2)

IRQ4  INT 0C	Serial Communication (COM1)

IRQ5  INT 0D	Fixed Disk

IRQ6  INT 0E	Diskette controller

IRQ7  INT 0F	Parallel printer

How to contact the Author

�������������������������

If you have any comments or improvement suggestions about INT09

I would like to hear from you. Please call me at 617-235-4018 or write to:

Celso Minnitti Jr

139 Linden St

Wellesley, MA 02181

And I will be glad to release a more comprehensive, user-friendly,

and much more self explanatory version of INT09 in very single aspect...

So please feel free to call at any time.

Thanks a lot.

P.S. As you can see this DOC needs a lot of work.