QBserial version 3.10

Serial I/O Routines for QB 4.x, BC 7.x, VBDOS, & FOSSIL Drivers

This library will provide you with serial I/O communications

routines for use in QuickBASIC 4.x (with or without PDQ), the

Microsoft Basic Compiler 7.x Professional Development System, and

Visual Basic for DOS 1.x. No longer are you forced to use the poor

communications support provided by QB.  This program will allow you

to control 8250 and 16450 type communications ports (UARTS) at

speeds of up to 115200 baud. Communication ports 1 - 4, and non-

standard addresses are supported. You will no longer have problems

with the DTR signal. DTR is left in the same state it was in when

before you called this driver, or it can be controlled by your

program. The serial driver includes XON/XOFF and CTS/RTS

handshaking. Serial input is interrupt driven, using any IRQ you

specify (1 - 15), with incoming XOFF flow control (if enabled), or

RTS flow control (if enabled) to prevent overrunning the input

buffer. This is referred to as "Normal" mode.

This driver also has a mode where it will operate with FOSSIL

drivers (INT 14h interface). Using FOSSIL mode allows you to use

virtually any type of communications hardware, since the type of

hardware accessed is dependant on the FOSSIL driver. This mode

requires that you have a FOSSIL driver loaded in advance. This is

referred to as "FOSSIL" mode.

The driver was written with Microsoft's C, and compiled with

version 6.0a of that compiler. This driver is useable with

QuickBASIC version 4.x, and Basic Compiler 6.x & 7.x PDS  (With or

without PDQ), and Visual Basic for DOS version 1.x. Basic Compiler

1.x, QuickBASIC 2.x, and 3.x are not supported by this driver. The

reason is that versions of QuickBASIC prior to 4.0 do not support

the extensive multi-language interface that QB4.x has (via the

DECLARE statement and Microsoft language extensions). Throughout

this manual QB will be used to refer to both QuickBASIC, the Basic

Compilers (6.x & 7.x PDS), and Visual Basic for DOS.

Before the driver can be used, the following INCLUDE statement must

be added to the beginning of your QB program:

'    $INCLUDE: 'qbserial.dec'

The declarations included by this statement specify all the entry

points into the serial driver. DO NOT change them or the driver

will not function.

Refer to the included sample programs SIMPLE.BAS, PCBDOOR.BAS, and

OFFHOOK.BAS. they use most of the calls described below.

QBSERIAL User Manual - V 3.10                              Page 1

Qbserial has two (2) modes of operation: Normal and FOSSIL. The

mode you operate in is specified in the OpenComm statement (The

FOS% parameter). In the following sections, where applicable, each

parameter will be explained as to how it operates in both Normal

and FOSSIL mode.

Port Initialization

OpenComm Port%, IRQ/Stat%, Wordlen%, Parity%, Baudrate&, HS%, FOS%

Parameter:     Port%

Normal:   If the specified port is 1 to 4, the driver opens that

port. If you specify the port as ZERO (0) the driver

enters "LOCAL" mode. This allows you to call the driver

with data, but the driver won't send anything. This is

useful when working with "doors". if you specify the port

as any other value, QBserial will use that as the base

address for the port. This allows you to work with non-

standard I/O addresses. Note however that if you open a

non-standard port, you must specify an IRQ value from 1

to 15 - There is no default IRQ when used in this manner.

FOSSIL:   Unlike normal mode, "port" does not necessarily relate to

a specific communications port or address. Using a "1"

for a port value does NOT necessarily mean COM1. Also

unlike QBserial's normal mode, a value other than 1 - 4

does NOT refer to a base address of a UART.

When using FOSSIL mode, "port" refers to a logical

device. Port 0 is the first port of a FOSSIL device

driver, Port 1 the second, and so on. When used with

drivers such as X00.SYS (assuming you didn't remap the

ports), port 0 refers to COM1, and port 1 refers to COM2.

This will vary depending on the FOSSIL driver used and

the device(s) referenced. In most cases it is determined

when the FOSSIL driver is installed through some sort of

configuration file (or command line options) that relates

the physical devices to logical port numbers. However, in

order to make the FOSSIL implementation as compatible as

possible with "normal" mode, the port number passed will

have a value of ONE (1) subtracted from it before

referring to the actual FOSSIL port. Therefore to access

the first port (port 0), you pass a 1 as you did with in

normal mode. Internally, one (1) is subtracted so that

port 0 is referenced. This makes the implementation or

normal and FOSSIL modes totally transparent.

Specifying a value of ZERO (0) for the port puts the

driver in "local" mode. This is identical to normal mode.

QBSERIAL User Manual - V 3.10                              Page 2

Parameter:     IRQ/Stat%

Normal:   Specifies which interrupt to use with this port. If the

value of IRQ% is ZERO (0), then the default IRQ values

are used (COM1 & COM3 use IRQ4, COM2 & COM4 use IRQ3).

This is what most applications will use. Specify an IRQ

value of 1 through 15 when you want to use an IRQ value

other than the default. Such as when you want to use

IRQ15 with COM3. NOTE: Be careful when choosing an IRQ

value other than the default. Most machines use some of

the other IRQ inputs for other machine functions such as

the Hard drive and system clock. QBserial DOES NOT chain

the interrupt, it takes it over entirely. If you choose

an IRQ that is used for something already, your machine

will most certainly operate improperly.

FOSSIL:   No IRQ value is necessary in FOSSIL mode since all

communications tasks are handled outside of QBserial. The

IRQ used with a particular "port" will be determined at

the time the FOSSIL driver is installed, probably through

the same configuration method mentioned above.

When using FOSSIL mode, this field is now Status. This is

a value RETURNED to the programmer when OpenComm has

completed. It indicates whether or not the FOSSIL driver

initialized properly and is ready to perform

communications tasks. A value of 1954h (HEX) or 6484

(DECIMAL) indicates a successful initialization. ANY

OTHER value indicates a initialization failure. You

should terminate your program and inform the user of your

program of the problem. Failures could result from the

FOSSIL driver not being installed, or not installed

properly. If you pass an invalid baud rate (see below),

Status will be returned as a ZERO. Otherwise Status will

be the actual value returned from the FOSSIL driver in

response to initialization. These values other than

1954h/6484 may offer a clue as to why the driver did not

initialize. These "possible" values would be documented

in the documentation for the FOSSIL driver.

Parameter:     Wordlen%

An integer specifying the word length of the serial data.

It has a value of 7 or 8. Identical in both modes.

Parameter:     Parity%

An integer, 0 = NONE, 1 = ODD, 2 = EVEN. Identical in

both modes.

QBSERIAL User Manual - V 3.10                              Page 3

Parameter:     BaudRate&

Normal:   This is a LONG INTEGER representing the desired baud

rate. Valid numbers are 0, 300, 1200, 2400, 4800, 9600,

19200, 38400, 57600, and 115200. The value you send is

NOT checked and I assume you could generate crazy baud

rates if desired. 115200 is the absolute maximum rate

that can be generated. If you specify a baud rate of ZERO

then the serial port is used AS-IS. The rate, word

length, & parity are NOT changed. This is useful for door

operation since the port is already initialized at the

proper settings when you get control.

FOSSIL:   Identical to normal mode with the exception being that

the range of selectable baudrates is limited. This is

limited by the current FOSSIL specification. The valid

baud rates are: 0, 300, 1200, 2400, 4800, 9600, 19200,

and 38400. If you pass a value other than one of these

valid rates, the driver will not initialize and return a

ZERO for status. In FOSSIL mode, if you specify a baud

rate of ZERO (0), the Baudrate initialization call is not

performed by OpenComm. This *SHOULD* have the same effect

that normal mode does by not changing the baudrate, word

length, & parity. However, since this is an undefined

condition in the FOSSIL specification, there is no way of

guaranteeing that the current settings will not be

changed during initialization of the driver. If you want

a 100% guarantee of the rate setting, you should provide

a speed when you do an OpenComm. This is only being done

for compatibility with Normal mode.

Parameter:     HS%

An integer specifying the type of handshake you wish to

use between the CPU and Modem (or destination device).

Valid numbers are: 0 = NO handshake, 1 = XON/XOFF, 2 =

CTS/RTS, 3 = XON/XOFF and CTS/RTS. Identical in both

modes.

Parameter:     FOS%

An integer that specifies which mode the driver will

operate in. Zero (0) specifies "Normal" mode, and One (1)

specifies "FOSSIL" mode.

Serial Output

To send a string of data out the serial port, you use the

"Transmit" call as follows:

Temp$ = "Transmit this..."

QBSERIAL User Manual - V 3.10                              Page 4

Transmit Temp$

or

Transmit "Send this also...."

If you want to transmit single characters, you can use the

"WriteChar" function. Transmit calls WriteChar to send the actual

characters to the port. If you wish to use WriteChar, it has the

following format:

Status% = WriteChar(Char%)

Char% is the integer value of the character you want to send, it is

not a string. Status% is an integer returned by WriteChar, it is

NON-ZERO if the character was actually sent to the port, and ZERO

if the character was not sent. WriteChar will loop indefinitely on

CTS hold, XOFF hold, or Transmit Buffer Busy. These loops will exit

if carrier is lost in FULL or PARTIAL modes and return a ZERO

status% (This occurs in normal mode ONLY. What happens in FOSSIL

mode is undefined). WriteChar gives you more low level control of

the sending of characters, but you must send each character

separately and monitor its status.

The Transmit function monitors the status of its calls to WriteChar

and triggers QuickBASIC's User Event trap (UEVENT) if a carrier

loss occurs in FULL mode. The UEVENT trap (FULL mode) is used by

default. You can disable the use of this event trap in the event

you already use the UEVENT trap for something else (PARTIAL mode).

Therefore, by default, carrier loss detection in your program is

done with the ON UEVENT GOSUB/UEVENT ON statements. The example

programs show the use of this method. By default UEVENT is also

used for catching carrier loss on keyboard input. this is described

below.

Serial Input

Serial input is managed with three functions: DataWaiting,

ReadChar, & ClearInputBuffer. DataWaiting is used to test the input

buffer to see if any characters need to be read from it. ReadChar

is then called to get the characters. ClearInputBuffer flushes the

input buffer of any existing characters. Both example programs

contain a subroutine called "KeyboardInput" that demonstrates the

use of these calls. It also properly handles input from the local

and remote keyboards, and can be the routine you use in your

programs if you wish. Basically the procedure for reading the

remote keyboard (serial input) is as follows:

QBSERIAL User Manual - V 3.10                              Page 5

IF DataWaiting THEN

C$ = CHR$(ReadChar)

END IF

The DataWaiting function also monitors the state of the carrier

detect signal and triggers, in FULL mode, the user event trap

(UEVENT) if carrier is lost. As you can see a singular trap routine

catches carrier loss on output as well as input. You will notice

that the "KeyboardInput" subroutine (in SIMPLE.BAS) loops using

DataWaiting, and as such, carrier is constantly checked while

waiting for input. If the ReadChar function is called when no data

is available (as indicated by DataWaiting) a NULL character is

returned.

In "Normal" mode, if XOFF/XON, CTS/RTS, or BOTH is enabled, then

the serial buffer will be protected from overfills. When the buffer

reaches 75% of capacity, an XOFF will be sent on the serial output

or RTS will be dropped. When the buffer empties out to 25% of

capacity, an XON will be sent or RTS raised. The capacity of the

input buffer is 2048 bytes. In FOSSIL mode, the threshold points

where flow control occurs and the buffer size, are determined by

the FOSSIL driver.

Carrier Detect Control

There are three modes of carrier detection: Full, Partial, & None.

Each has it own distinct use for a particular purpose. The driver

defaults to FULL carrier detection unless you change it. The

following call is used to change control modes:

CarrierDetect 0 [or] 1 [or] 2

Specifying a TWO (2) sets carrier detection to FULL. This is the

default mode. When a loss of carrier occurs while transmitting,

polling for input data, or sitting in a wait loop (XOFF/XON or

CTS), the driver will return to the calling program and trip the

UEVENT flag. Tripping the UEVENT flag causes Basic to go to the

User Event trap routine (providing ON UEVENT GOSUB/UEVENT ON has

been set up in the user program). You should use this mode only if

you want to use the UEVENT trap for carrier loss detection. You

cannot transmit data (with the Transmit function) in this mode if

there is no carrier. You can send characters in this mode with the

WriteChar function.

Specifying a ONE (1) sets carrier detection to PARTIAL. This mode

is similar to FULL with the exception that the UEVENT flag is not

tripped when carrier is lost. This mode should be used if UEVENT is

used for something else in your program, or you don't wish to

utilize UEVENT.

QBSERIAL User Manual - V 3.10                              Page 6

Specifying a ZERO (0) sets carrier detection to NONE. An example of

this mode would be where "plain Jane" serial I/O is required (such

as communicating with a "dumb" terminal). In this application there

are no modem handshake signals (3 wire EIA/RS-232). This mode can

also be used for talking to a modem when there is no carrier signal

present (such as dialing and initialization commands). The Transmit

and WriteChar routines will always transmit data in this mode with

or without carrier present. Carrier detection must be disabled in

this mode to prevent calls to Uevent and disable the no carrier

escape mechanisms built into the XON/XOFF loops. Important: If you

are using "Normal" mode, and you are using the XOFF/XON protocol,

with "CarrierDetect 0" set, and receive an XOFF while transmitting,

you will sit forever waiting for an XON. This is a normal

condition, with the exception that there is no escape from this

loop other than XON. If you are using "FOSSIL" mode, this action is

undefined.

Carrier Loss Detection by Polling

As mentioned above, by default, the UEVENT trap is used to detect

when a loss of carrier occurs. There is also a function that allows

you to detect loss of carrier without using the UEVENT trap. While

I feel that using UEVENT is the simplest way to catch a carrier

loss in a program, it is possible that UEVENT might need to be used

by something else. This function is:

X% = CarrierLost

This function call allows to you to 'poll' for the state of

carrier. CarrierLost returns a NON-ZERO value when there is NO

carrier, and a ZERO when there IS carrier. CarrierLost is a real

time function, it returns the current carrier condition at the time

of the call. This allows you to code simple IF or DO loops to

detect loss:

IF CarrierLost THEN

..process carrier loss

END IF

Modem Status

NOTE:     This function ONLY works in Normal mode. It has no

functional equivalent in FOSSIL mode.

The Modem Status register contains bits that convey information

about the Modem's current state, and what has happened since the

last status query. In most instances these bits will not be needed

QBSERIAL User Manual - V 3.10                              Page 7

for normal serial operations. This was added to provide additional

information about the device in use.

Normal Mode:

Modem Status Register:

7       6       5       4       3       2       1       0   Bit

���������������������������������������������������������������Ŀ

�  128  �  64   �  32   �  16   �   8   �   4   �   2   �   1   �

�Carrier� Ring  �DataSet� Clear � Delta � Delta � Delta � Delta �

�Detect �Signal � Ready �To Send�  CD   �  RI   �  DSR  �  CTS  �

�����������������������������������������������������������������

Bits 0 - 3 of this register report a change in the state of their

respective RS-232 (signal) pins. A "1" in any of these bits means

that it's input has changed since last status read. Reading this

register clears bits 0 - 3. Bits 4 - 7 report the absolute state of

their respective RS-232 inputs (These bits are "real time").

To use this function, the Basic statement would be:

value% = ModemStatus

The value returned by ModemStatus depends on the particular bit(s)

set in the Modem Status register. The corresponding decimal weights

are listed in the boxes in the above figure. As an example, if you

were monitoring the register and looking for a Ring Indication, you

would be looking for a value of 64. NOTE: If you are looking for a

particular value, you must AND the value you wish to test against

what ModemStatus returns. So, if you were looking for a Ring

Indication, you would use:

St% = ModemStatus

if St% AND 64 then

... Ring detected

Program Termination

Since "OpenComm" seizes an interrupt vector, this vector needs to

be restored BEFORE your program ends. If you neglect to restore

this vector, you could be risking a crash. The routine used to

reset the vector is: CloseComm. Only use CloseComm if you specified

a port value in the OpenComm call (Normal or FOSSIL modes). If you

specified a port value of ZERO (Local mode) the interrupt vector is

NOT grabbed, and does not need to be reset. In Normal mode,

CloseComm resets any UART registers to their original value. In

FOSSIL mode, how the UART registers are left is undefined.

QBSERIAL User Manual - V 3.10                              Page 8

Data Terminal Ready (DTR)

Everybody that has used QuickBASIC's OPEN COMn statement is very

familiar with the problems of the DTR signal. Apparently Microsoft

felt that DTR would never be needed after a program terminates.

Anybody who has ever tried to write a door, or chain from one

program to another has cursed this decision at some time during

their programs development. This driver handles the DTR with a

different view. When you close the comm channel the DTR signal is

left in the same state it was when you opened the comm channel. For

door applications, this is a must.

A function call is provided however that allows your program to

control the DTR signal:

DTRcontrol 1 [or] 0           ' 0 = DTR OFF, 1 = DTR ON

Additionally when you use DTRcontrol to change the state of the DTR

signal it also instructs the driver to leave it this way when the

CloseComm function is called. Remember that the driver leaves DTR

the way it found it when OpenComm was called. If you used some

other method to change DTR after OpenComm was called, it would

automatically return to that original state when you called

CloseComm. Therefor only DTRcontrol should be used if you wish to

change the state of DTR. This appears to operate identically in

normal and FOSSIL modes.

Request To Send (RTS)

NOTE:     This function ONLY works in Normal mode. It has no

functional equivalent in FOSSIL mode.

Request To Send (RTS) is a hardware flow control signal used to

tell the remote device to STOP sending data to us. Normally RTS is

set "on" and data will always flow (or the signal is ignored at the

remote end). With some communications programs RTS is used to

temporarily stop a modem from sending data while I/O is done to a

slow disk. This is especially true with high speed communications

where there is very little time between interrupts. This function

is independent of the automatic RTS flow control selected by

handshake 2 or 3 which tells the remote device to stop sending when

the input buffer is almost full. This function allows you to

manually control the state of RTS so you could, for instance, go

off and write the data somewhere without fear of loosing data.

RTScontrol 1 [or] 0           ' 0 = RTS OFF, 1 = RTS ON

When you use RTScontrol to change the state of the RTS signal it

also instructs the driver to leave it this way when the CloseComm

function is called. The driver leaves RTS the way it found it when

QBSERIAL User Manual - V 3.10                              Page 9

OpenComm was called. If you used some other method to change RTS

after OpenComm was called, it will automatically return to that

original state when you call CloseComm. Therefor only RTScontrol

should be used if you wish to change the state of RTS. This is

similar in the way DTRcontrol works.

Driver Version and Copyright

A function is available to access the drivers version and copyright

string. This would be used if you wish to display in your final

product the version of QBserial that your are using, and/or my

copyright along with yours. In order to access the copyright

string, the following lines should be added to your Basic program:

X& = DriverCopyright

WHILE (PEEK(X&))

CP$ = CP$ + CHR$(PEEK(X&))

X& = X& + 1

WEND

This places the version/copyright string into string variable CP$.

This string may now be handled in whatever method you choose.

Crescent's PDQ Library

An object module for use with PDQ is also included. PDQ does not

support UEVENT, so all references to UEVENT in the text should be

ignored. You will need to do carrier checking by polling with the

CarrierLost function. If you use FULL mode and carrier is lost it

will operate the same way as PARTIAL mode. The only exception to

this is the Transmit function, which when in FULL or PARTIAL mode,

will not transmit data.

Linking

Since object modules are provided for QB4.x, BC7, VBDOS, and PDQ,

you will need to select the proper one to use for your program. The

file QBSER.OBJ should be used for programs compiled with

QuickBASIC. BC7SER.OBJ is to be used for programs compiled with BC7

PDS, and Visual Basic for DOS. QBSERPDQ.OBJ is for PDQ users.

Typical link command lines would look like this: (Please note that

your command lines will probably be different if you use additional

libraries to generate your code). The term "yourprog" is meant to

imply any code modules necessary for linking your project. This

could one .OBJ file, or many. Please note that compiling any

linking projects from within the QB/QBX/VBDOS development

environment(s) is not supported.

QBSERIAL User Manual - V 3.10                             Page 10

Link yourprog qbser;               (For QuickBASIC)

Link yourprog bc7ser;              (For BC7 PDS and Visual Basic)

Link yourprog qbserpdq acrtused;   (For PDQ)

^

����� See note about acrtused.obj

NOTE:     In the past the file ACRTUSED.OBJ was required to be

linked in when using the PDQ replacement libraries. This

was necessary to satisfy a dummy external call. It

appears that it may no longer be necessary to link with

this file anymore. You should try linking without

acrtused, and see if a link error occurs. If no error

occurs, then you do not need to specify it.

Quick Libraries

If you wish to develop programs from within the environment, you

will first need to create a Quick library using one of the object

modules QBSER.OBJ, BC7SER.OBJ, or QBSERPDQ.OBJ. To create a Quick

lib for QuickBASIC 4.x, use the following Link command:

Link QBSER,,,BQLB4x/q

Where "BQLB4x" is one of the following: BQLB40.LIB, BQLB41.LIB, or

BQLB45.LIB. Use the one that came with your compiler. If you use

the wrong BQLB file you might get an "Invalid Format" error when

attempting to start QuickBASIC. QBSER.OBJ may also be added to

Quick Libraries containing other routines and OBJ's as well.

To create a Quick Library for use within QBX (BC7's environment)

use the following link command (IMPORTANT: be sure to use Link

v5.xx that came with BC7 or you will receive linking errors):

Link BC7SER,,,QBXQLB/q;

To create a Quick Library for use within Visual Basic's environment

(VBDOS) use the following link command (IMPORTANT: be sure to use

Linker that came with VBDOS or you will receive linking errors):

Link BC7SER,,,VBDOSQLB/q;

For PDQ, use the method for QB4.x, since you will be using the

built in run time routines built into the environment. You ONLY use

QBSERPDQ.OBJ when you link the EXEcutable file.

Remember too, that you can also add BC7SER.OBJ to Quicklibs

containing other routines.

QBSERIAL User Manual - V 3.10                             Page 11

Specifications

The following port addresses and default interrupts are used in

Normal mode:

Comm      Base Address        IRQ (Default)

1          3F8 hex            4

2          2F8 hex            3

3          3E8 hex            4

4          2E8 hex            3

Please read the section explaining the OpenComm function for

information on how to use addresses or IRQ's other than the

defaults.

Registration and Support

QBserial is distributed as shareware. You may copy and distribute

it freely, as long as you do NOT modify it or add (or remove) any

files to it.

QBserial is not free software, and requires registration. There are

two classes of registration which are dependant upon use:

Commercial and Non-Commercial.

Registration of QBserial is $25 for non-commercial use. Non-

commercial uses are: Personal use, or use in another shareware

product such as doors and utilities where you allows others to try

the product first before buying and request a nominal registration

fee. Source code is available as an option for an additional $50.

Registration of QBserial for commercial use is $75. Commercial uses

are: Inclusion of QBserial into ANY product that is to be sold for

profit, or any use of this program in a business environment. Any

program that must be paid for in advance before the product is

delivered, or use in a corporate environment fits this category.

Commercial registration includes the source code for QBserial in C

(Microsoft C), if desired.

Please register your copy!

Software and Shareware distribution houses may charge a nominal

distribution and copying fee not to exceed $5.00. They may not sell

this program outright or ask you for the registration fee directly,

they must tell you that registration is required by the author.

If you have access to a BBS that is an ILINK network member, you

can reach me in the respective Basic conferences. If you do not

have ILINK access anywhere nearby, then you can always contact me

on the SailBoard BBS at the number listed below. I would prefer if

QBSERIAL User Manual - V 3.10                             Page 12

you contact me directly on the SailBoard however, as mail networks

aren't always as reliable as I'd like, and I have missed messages.

I'll do the best I can to help if you have questions. Bugs will be

tended to if required, and good suggestions tend to be implemented.

Source Code

The source code for QBserial is available. The source is available

by signing at the appropriate place on the registration form. The

source may be used for you own internal uses only. The source may

not be distributed by you to any other person, even if you make

changes to it. Nor can the resulting object code be sold or

distributed. Modifications can only be used in your end product,

and without any type of royalty. The source is available only after

you register QBserial. The license fee for the QBserial source code

is $50.00 for Non-Commercial users. Commercial users receive the

source as part of their registration fee if they desire it.

Jeff Sumberg

Sysop of the SailBoard BBS

Ringwood, NJ, 201-831-8152

[ or ]

Box 212

Ringwood, NJ, 07456

Acknowledgments

Thanks go out to Mark "Sparky" Herring for using these routines in

his highly successful Qmail door.

Thanks to Brent Yandell for using these routines in his PCBoard

File Viewer PCBFV and PCBFX programs.

Thanks to Mark Wilson for some very good ideas in version 2.0.

Thanks to Michael Conley for finding a real time sensitive

interrupt problem and for providing very useful information.

Thanks to Jeffrey Morley (author of ZipLab) for testing.

Distribution File Contents

QBSER.OBJ     QBserial object module for use with QuickBASIC

version 4.x.

QBSERPDQ.OBJ  QBserial object module for use with PDQ and

QuickBASIC 4.x or PDQ with Basic Compiler 7.x (NEAR

string model).

QBSERIAL User Manual - V 3.10                             Page 13

BC7SER.OBJ    QBserial object module for use with Basic Compiler

6.x, Basic Compiler 7.x (NEAR or FAR string models),

and Visual Basic for DOS.

ACRTUSED.OBJ  Dummy object module to satisfy an external reference

when linking with PDQ files. May no longer be

necessary. See Text.

OFFHOOK.BAS   Sample Basic program. Used to take modem off hook.

References PCBoard data files for port info. Can

also be used stand alone.

PCBDOOR.BAS   Very simple frame for creating a door in PCBoard.

Should be used ONLY as a guide.

SIMPLE.BAS    Very simple "do nothing" program offering examples

of how to call QBserial functions.

QBSERIAL.DEC  Function declarations for QBserial.

QBSERIAL.DOC  QBserial Documentation (this file).

READ.ME       Latest changes to QBserial.

REGISTER.FRM  QBserial Registration form. Please Register your

copy.

FILE_ID.DIZ

DESC.SDI      BBS description files.

Changes and Revisions

06/09/89  1.0  Initial Release

06/21/89  1.1  Added carrier detection control. Calling program can

turn detection off and on as desired. Necessary if

data sending/receiving required if carrier isn't

present.

09/01/89  1.2  Fixed a bug in Readchar where 'extended' characters

(ASCII 128 to 255) were causing an Illegal Function

Call in the CHR$() conversion (because they were

being returned as negative numbers). Extended

character may now be received properly. Added three

new functions: CDtrap, CarrierLost, and DTRcontrol.

03/24/90  1.5  Added support for Basic Compiler 7.0 PDS and PDQ.

Added sections to manual about Linking, Source code

availability, and updated section on Quicklibs.

QBSERIAL User Manual - V 3.10                             Page 14

Separate object files now supplied for QB, BC7, and

PDQ.

05/01/90  1.6  Oops! PDQ users found out fast that I forgot to

compile the PDQ module with stack checking removed.

I also forgot to include ACRTUSED.OBJ to satisfy

linking requirements. Sorry 'bout that! Released

only as beta to those that needed it.

11/01/90  2.0  Added capability to specify non-standard port

addresses and which IRQ to use when the port is

opened. Removed CDtrap function. Created new carrier

detection modes using the CarrierDetect function. (0

= NONE, 1 = PARTIAL, 2 = FULL). Also added

DriverCopyright function to allow program access to

the driver version.

02/08/92  2.10 Added RTS flow control support. RTS will be used to

control data coming in to QBserial when the

handshake is set to 2 (CTS/RTS). Removed necessity

for SERIAL.LIB library file by adding inline

assembler that performed the routines that were

previously included in that library. QBserial is now

compiled with Microsoft C 6.0 which will give

smaller, faster, and more efficient code than QuickC

could generate. This is especially true in the

interrupt area. Added new function RTScontrol, to

allow program to control RTS line directly.

Increased size of input buffer from 1024 to 2048

bytes. Changed method in which interrupt controller

mask bits were set & cleared. Corrected error found

in documentation describing the function of

CarrierDetect.

06/20/92  2.20 Added support for extended IRQs. QBserial now

supports IRQ 8 - 15. Added a new sample program to

the QBserial distribution package (OFFHOOK.BAS) to

demonstrate usage of QBserial.

07/05/92  2.25 While experimenting with some programs of my own I

found a some errors in the disabling of the IRQ

masks on the Interrupt Controller when exiting from

QBserial. The program has been changed so that if

the IRQ you specify is already turned ON when

OpenComm executes, QBserial will leave it ON when

you do a CloseComm. In versions since 2.10, the IRQ

mask was always turned OFF when CloseComm was

executed. This caused other communications programs

such as BBS's that don't reset the port when

returning from QBserial to appear to "lock up" when

they regained control.

QBSERIAL User Manual - V 3.10                             Page 15

11/01/92  3.00 Added the ability for QBserial to use FOSSIL

drivers. Modified the OpenComm call to allow the

programmer to specify which mode to operate in. The

QBSERIAL.DEC file has changed and the one provided

with this version MUST be used when upgrading to

this version. Added new function: ModemStatus, to

read the Modem Status register of the UART. This is

a normal mode function only.

06/12/93  3.10 This version is a bug fix version. Several minor

errors and omissions have been reported by users,

and these have been corrected in this version. These

include:

- Modified the QBSERIAL.DEC file to allow Qbserial

to work with Visual Basic for DOS. This is because

of an unknown, undocumented change in the way

DECLARE works in Visual Basic for DOS.

- ClearInputBuffer did not work in FOSSIL mode.

- Modified OpenComm to reset flow control flags,

buffer pointers, etc. It was found that if a port

was opened, used, closed, then opened again without

program termination or chain from program to

program, that the Flow Control mode would be wrong

or inoperative. The input buffer also was not

flushed when a port was opened. This was because

initialization to these flags and pointers was only

being done when the EXE was loaded, not during

OpenComm. These are done within OpenComm.

QBSERIAL User Manual - V 3.10                             Page 16