The	Hitchhiker's Guide to X386/XFree86 Video Timing

(or, Tweaking	your Monitor for Fun and Profit)

Eric S. Raymond esr@snark.thyrsus.com	1

This is version 1.0, Jan 8th 1993.

1.  Introduction

Please direct	comments, criticism, and suggestions for improvement to

esr@snark.thyrsus.com.

The XFree86 server allows users to configure their video subsystem and

thus encourages best use of existing hardware.  This tutorial	is

intended to help you learn how to generate your own timing numbers to

make optimum use of your video card and monitor.

We'll	present	a method for getting something that works, and then show

you how you can experiment starting from that	base to	develop	settings

that optimize	for your taste.

If you already have a	mode that almost works (in particular, if one of

predefined VESA modes	gives you a stable display but one that's

displaced right or left, or too small, or too	large) you can go

straight to the section on Fixing Problems.  This will enlighten you

on ways to tweak the timing numbers to achieve particular effects.

XFree86 allows you to	hot-key	between	different modes	defined	in

XF86Config (see XF86Config.man for details).	Use this capability to

save yourself	hassles!  When you want	to test	a new mode, give it a

unique mode label and	add it to the end your hot-key list.  Leave a

known-good mode as the default to fall back on if the	test mode

doesn't work.	 The Xconfig section at	the end	of the second Example

Calculation provides a good example of how to	record your experiments

in a way that	will help you quickly converge on a solution.

First	check out the Monitors file in lib/X11/doc If your monitor is in

it, you can probably skip the	rest of	this document!	You may	need to

scale	some of	the timing numbers if the clock	used to	generate the

mode in the database doesn't match what your card has	available, but

that's easy.

2.  How Video	Displays Work

Knowing how the display works	is essential to	understanding what

numbers to put in the	various	fields in the file Xconfig.  Those

values are used in the lowest	levels of controlling the display by the

XFree86 server.

The display generates	a picture from a series	of dots.  The dots are

arranged from	left to	right to form lines.  The lines	are arranged

from top to bottom to	form the picture.  The dots emit light when they

are struck by	the electron beam inside the display.  To make the beam

strike each dot for an equal amount of time, the beam	is swept across

the display in a constant pattern.

The pattern starts at	the top	left of	the screen, goes across	the

screen to the	right in a straight line, and stops temporarily	on the

right	side of	the screen.  Then the beam is swept back to the	left

side of the display, but down	one line.  The new line	is swept from

left to right	just as	the first line was.  This pattern is repeated

-----------

1. (from an	original by Chin Fang

fangchin@leland.stanford.edu;	portions derive	from a how-

to by	Bob Crosson crosson@cam.nist.gov,

until	the bottom line	on the display has been	swept.	Then the beam is

moved	from the bottom	right corner of	the display to the top left

corner, and the pattern is started over again.

Starting the beam at the top left of the display is called the

beginning of a frame.	 The frame ends	when the beam reaches the the

top left corner again	as it comes from the bottom right corner of the

display.  A frame is made up of all of the lines the beam traced from

the top of the display to the	bottom.

If the electron beam were on all of the time it was sweeping through

the frame, all of the	dots on	the display would be illuminated.  There

would	be no black border around the edges of the display.  At	the

edges	of the display the picture would become	distorted because the

beam is hard to control there.  To reduce the	distortion, the	dots

around the edges of the display are not illuminated by the beam even

though the beam may be pointing at them.  The	viewable area of the

display is reduced this way.

Another important thing to understand	is what	becomes	of the beam when

no spot is being painted on the visible area.	 The time the beam would

have been illuminating the side borders of the display is used for

sweeping the beam back from the right	edge to	the left and moving the

beam down to the next	line.  The time	the beam would have been

illuminating the top and bottom borders of the display is used for

moving the beam from the bottom-right	corner of the display to the

top-left corner.

The adapter card generates the signals which cause the display to turn

on the electron beam at each dot to generate a picture.  The card also

controls when	the display moves the beam from	the right side to the

left and down	a line by generating a signal called the horizontal sync

(for synchronization)	pulse.	One horizontal sync pulse occurs at the

end of every line.  The adapter also generates a vertical sync pulse

which	signals	the display to move the	beam to	the top-left corner of

the display.	A vertical sync	pulse is generated near	the end	of every

frame.

The display requires that there be short time	periods	both before and

after	the horizontal and vertical sync pulses	so that	the position of

the electron beam can	stabilize.  If the beam	can't stabilize, the

picture will not be steady.

In a later section, we'll come back to these basics with definitions,

formulas and examples	to help	you use	them.

3.  Basic Things to Know about your Display and Adapter

There	are some fundamental things you	need to	know before hacking an

Xconfig entry.  These	are:

1.	your monitor's horizontal and vertical sync frequency options

2.	your video adapter's driving clock frequency, or "dot clock"

3.	your monitor's bandwidth

The monitor sync frequencies:

The horizontal sync frequency	are just the number of times per second

the monitor can write	a horizontal scan line;	it is the single most

important statistic about your monitor.  The vertical	sync frequency

is the number	of times per second the	monitor	can traverse its beam

vertically.

Sync frequencies are usually listed on the specifications page of your

monitor manual.  The vertical	sync frequency number is typically

calibrated in	Hz (cycles per second),	the horizontal one in KHz

(kilocycles per second).  The	usual ranges are between 50 and	80Hz

vertical, and	between	31 and 135KHz horizontal.

If you have a	multisync monitor, these frequencies will be given as

ranges.  Some	monitors, especially lower-end ones, have multiple fixed

frequencies.	These can be configured	too, but your options will be

severely limited by the built-in monitor characteristics.  Choose the

highest frequency pair for best resolution.  And be careful --- trying

to clock a fixed-frequency monitor at	a higher speed than it's

designed for can damage it.

The card driving clock frequency:

Your video adapter manual's spec page	will usually give you the card's

dot clock (that is, the total	number of pixels per second it can write

to the screen).  If you don't	have this information, the X server will

get it for you.  Even	if your	X locks	up your	monitor, it will emit a

line of clock	and other info to standard output.  If you redirect this

to a file, it	should be saved	even if	you have to reboot to get your

console back.

If you're using SGCS X, the line will	look something like the

following example, collected from a Swan local-bus S3	adapter.

XFree86 uses a slightly different multi-line format.

WGA: 86C911 (mem: 1024k clocks: 25 28 40	3 50 77	36 45 0	0 79 31	94 65 75 71)

---  ------	 -----	       --------------------------------------------

|     |		   |		     Possible driving frequencies in MHz

|     |		   +-- Size of on-board	frame-buffer RAM

|     +-- Chip type

+-- Server type

Note:	do this	with your machine unloaded (if at all possible).

Because X is an application, its timing loops	can collide with disk

activity, rendering the numbers above	inaccurate.  Do	it several times

and watch for	the numbers to stabilize; if they don't, start killing

processes until they do.  SVr4 users:	the mousemgr process is

particularly likely to mess you up.

In order to avoid the	clock-probe inaccuracy,	you should clip	out the

clock	timings	and put	them in	your Xconfig as	the value of the Clocks

property --- this suppresses the timing loop and gives X an exact list

of the clock values it can try.  Using the data from the example

above:

wga

Clocks    25 28 40 3 50 77 36 45 0 0 79 31 94 65 75	71

On systems with a highly variable load, this may help	you avoid

mysterious X startup failures.  It's possible	for X to come up, get

its timings wrong due	to system load,	and then not be	able to	find a

matching dot clock in	its config database ---	or find	the wrong one!

The monitor's	video bandwidth:

Finally, it's	useful to know your monitor's video bandwidth, so you

know approximately what the highest dot clock	you can	use is.	 There's

a lot	of give	here, though --- some monitors can run as much as 30%

over their nominal bandwidth.

Knowing the bandwidth	will enable you	to make	more intelligent choices

between possible configurations.  It may affect your display's visual

quality (esp.	 sharpness for fine details).

Your monitor's video bandwidth should	be included on the manual's spec

page.	 If it's not, look at the monitor's highest rated resolution.

As a rule of thumb, here's how to translate these into bandwidth

estimates (and thus into rough upper bounds for the dot clock	you can

use):

640x480		25

800x600		36

1024x768	   65

1024x768 interlaced	45

1280x1024	   110

BTW, there's nothing magic about this	table; these numbers are just

the lowest dot clocks	per resolution in the standard XFree86 Modes

database.  The bandwidth of your monitor may be higher than the

minimum needed for its top resolution, so don't be afraid to try a dot

clock	a few MHz higher.

Also note that bandwidth is seldom an	issue for dot clocks under 65MHz

or so.  With an SVGA and most	hi-res monitors, you can't get anywhere

near the limit of your monitor's video bandwidth.  The following are

examples:

Brand		     Video Bandwidth

----------		     ---------------

NEC	4D		     75Mhz

Nano 907a		50Mhz

Nano 9080i		     60Mhz

Mitsubishi HL6615	     110Mhz

Mitsubishi Diamond Scan	  100Mhz

IDEK MF-5117	     65Mhz

IOCOMM Thinksync-17	CM-7126	  136Mhz

HP D1188A		100Mhz

Philips SC-17AS		  110Mhz

Swan SW617		     85Mhz

Even low-end monitors	usually	aren't terribly	bandwidth-constrained

for their rated resolutions.	The NEC	Multisync II makes a good

example --- it can't even display 800x600 per	its spec.  It can only

display 800x560.  For	such low resolutions you don't need high dot

clocks or a lot of bandwidth;	probably the best you can do is	32Mhz or

36Mhz, both of them are still	not too	far from the monitor's rated

video	bandwidth of 30Mhz.

At these two driving frequencies, your screen	image may not be as

sharp	as it should be, but definitely	of tolerable quality. Of course

it would be nicer if NEC Multisync II	had a video bandwidth higher

than,	say, 36Mhz.  But this is not critical for common tasks like text

editing, as long as the difference is	not so significant as to cause

severe image distortion (your	eyes would tell	you right away if this

were so).

What these control:

The sync frequency ranges of your monitor, together with your	video

adapter's dot	clock, determine the ultimate resolution that you can

use.	But it's up to the driver to tap the potential of your hardware.

A superior hardware combination without an equally competent device

driver is a waste of money.  On the other hand, with a versatile

device driver	but less capable hardware, you can push	the hardware's

envelope a little.  This is the design philosophy of XFree86.

4.  Interpreting the Basic Specifications

This section explains	what the specifications	above mean, and	some

other	things you'll need to know.  First, some definitions.  Next to

each in parens is the	variable name we'll use	for it when doing

calculations

horizontal	sync frequency (HSF)

Horizontal scans per second (see above).

vertical sync frequency (VSF)

Vertical scans per second (see above).	Mainly important as the

upper limit on your refresh rate.

dot clock	(DCF)

More formally, `driving	clock frequency'; sometimes loosely

called `bandwidth'.  The frequency of the crystal or VCO on your

adaptor	--- the	maximum	dots-per-second	it can emit.

video bandwidth (VB)

The highest frequency at which your monitor's video signal can

change.	 This constrains the highest dot clock you can use and

the overall sharpness of fine details in the video image.

frame length (HFL,	VFL)

Horizontal frame length	(HFL) is the number of dot-clock ticks

needed for your	monitor's electron gun to scan one horizontal

line, *including the inactive left and right borders*.	Vertical

frame length (VFL) is the number of scan lines in the *entire*

image, including the inactive top and bottom borders.

screen refresh rate (RR)

The number of times per	second your screen is repainted.  Higher

frequencies are	better,	as they	reduce flicker.	 60Hz is good,

VESA-standard 72Hz is better.  Compute it as

RR = DCF / (HFL * VFL)

Note that the product in the denominator is *not* the same as

the monitor's visible resolution, but typically	somewhat larger.

We'll get to the details of this below.

About	Bandwidth:

Monitor makers like to advertise high	bandwidth because it constrains

the sharpness	of intensity and color changes on the screen.  A high

bandwidth means smaller visible details.

Your monitor uses electronic signals to present an image to your eyes.

Such signals always come in in wave form once	they are converted into

analog form from digitized form.  They can be	considered as

combinations of many simpler wave forms each one of which has	a fixed

frequency, many of them are in the Mhz range,	eg, 20Mhz, 40Mhz, or

even 70Mhz.  Your monitor video bandwidth is,	effectively, the

highest-frequency analog signal it can handle	without	distortion.

For our purposes, bandwidth is mainly	important as an	approximate

cutoff point for the highest dot clock you can use.

Sync Frequencies snd the Refresh Rate:

Each horizontal scan line on the display is just the visible portion

of a frame-length scan.  At any instant there	is actually only one dot

active on the	screen,	but with a fast	enough refresh rate your eye's

persistence of vision	enables	you to "see" the whole image.

Here are some	pictures to help:

_______________________

|			|     The horizontal frame length

|->->->->->->->->->->->	|     is the time in dot clocks

|		       )|     required for the

|<-----<-----<-----<---	|     electron beam to trace

|			|     a	pattern	like this

|			|

|			|

|			|

|_______________________|

_______________________

|	 ^		|     The vertical frame length

|	^ |		|     is the time in dot clocks

|	| v		|     required for the

|	^ |		|     electron beam to trace

|	| |		|     a	pattern	like this

|	^ |		|

|	| v		|

|	^ |		|

|_______|_v_____________|

Remember that	the actual raster scan is a very tight zigzag pattern;

that is, the beam moves left <-> right and at	the same time up <->

down.

Now we can see how the dot clock and frame size relates to refresh

rate.	 By definition,	one hertz (hz) is one cycle per	second.	 So, if

your horizontal frame	length is HFL and your vertical	frame length is

VFL, then to cover the entire	screen takes (HFL * VFL) ticks.	 Since

your card emits DCF ticks per	second by definition, then obviously

your monitor's electron gun(s) can sweep the screen from left	to right

and back and from bottom to top and back DCF / (HFL *	VFL) times/sec.

This is your screen's	refresh	rate, because it's how many times your

screen can be	updated	thus REFRESHED per second!

You need to understand this concept to design	a configuration	which

trades off resolution	against	flicker	in whatever way	suits your

needs.

5.  Tradeoffs	in Configuring your System

Another way to look at the formula we	derived	above is

DCF =	RR * HFL * VFL

That is, your	dot clock is fixed.  You can use those dots per	second

to buy either	refresh	rate, horizontal resolution, or	vertical

resolution.  If one of those increases, one or both of the others must

decrease.

Note,	though,	that your refresh rate cannot be greater than the

maximum vertical sync	frequency of your monitor.  Thus, for any given

monitor at a given dot clock,	there is a minimum product of frame

lengths below	which you can't	force it.

In choosing your settings, remember: if you set RR too low, you will

get mugged by	screen flicker.

You probably do not want to pull your	refresh	rate below 60Hz.  This

is the flicker rate of fluorescent lights; if	you're sensitive to

those, you need to hang with 72MHz, the VESA ergonomic standard.

Flicker is very eye-fatiguing, though	human eyes are adaptable and

peoples' tolerance for it varies widely.  If you face	your monitor at

a 90%	viewing	angle, are using a dark	background and a good

contrasting color for	foreground, and	stick with low to medium

intensity, you *may* be comfortable at as little as 45Hz.

The acid test	is this: open a	xterm with pure	white back-ground and

black	foreground using xterm -bg white -fg black and make it so large

as to	cover the entire viewable area.	 Now turn your monitor's

intensity to 3/4 of its maximum setting, and turn your face away from

the monitor.	Try peeking at your monitor sideways (bringing the more

sensitive peripheral-vision cells into play).	 If you	don't sense any

flicker or if	you feel the flickering	is tolerable, then that	refresh

rate is fine with you.  Otherwise you	better configure a higher

refresh rate,	because	that semi-invisible flicker is going to	fatigue

your eyes like crazy and give	you headaches, even if the screen looks

OK to	normal vision.

So let's say you've picked a minimum acceptable refresh rate.	 In

choosing your	HFL and	VFL, you'll have some room for maneuver.

6.  Memory Requirements

Available frame-buffer RAM may limit the resolution you can achieve on

color	or gray-scale displays.	 It probably isn't a factor on displays

that have only two colors, white and black with no shades of gray in

between.

For 256-color	displays, a byte of video memory is required for each

visible dot to be shown.  This byte contains the information that

determines what mix of red, green, and blue is generated for its dot.

To get the amount of memory required,	multiply the number of visible

dots per line	by the number of visible lines.	 For a display with a

resolution of	800x600, this would be 800 x 600 = 480,000, which is the

number of visible dots on the	display.  This is also,	at one byte per

dot, the number of bytes of video memory that	are necessary on your

adapter card.

Thus,	your memory requirement	will typically be (HR *	VR)/1024 Kbytes

of VRAM, rounded up.	In the example case, we'd need (936 * 702)/1024

= 642K.  So if you have one meg, you'll have extra for virtual-screen

panning.

However, if you only have 512K on board, then	you can't use this

resolution.  Even if you have	a good monitor,	without	enough video

ram, you can't take advantage	of your	monitor's potential.  On the

other	hand, if your SVGA has one meg,	but your monitor can display at

most 800x600,	then high resolution is	beyond your reach anyway.

Don't	worry if you have more memory than required; XFree86 will make

use of it by allowing	you to scroll your viewable area (see the

Xconfig file documentation on	the virtual screen size	parameter).

Remember also	that a card with 512K bytes of memory really doesn't

have 512,000 bytes installed,	it has 512 x 1024 = 524,288 bytes.

If you're running SGCS X using an S3 card, and are willing to	live

with 16 colors (4 bits per pixel), you can set depth 4 in Xconfig and

effectively double the resolution your card can handle.  S3 cards, for

example, normally do 1024x768x256.  You can make them	do 1280x1024x16

with depth 4.

7.  Computing	Frame Sizes

Warning: this	method was developed for multisync monitors.  It will

probably work	with fixed-frequency monitors as well, but no

guarantees!

Start	by dividing DCF	by your	highest	available HSF to get the number

of horizontal	sweeps per second available.

For example; suppose you have	a Sigma	Legend SVGA with a 65MHz dot

clock, and your monitor has a	55KHz horizontal scan frequency.  The

quantity (DCF	/ HSF) is then 1181.

Now for our first bit	of black magic.	 You need to round this	figure

to the nearest multiple of 8.	 This has to do	with the VGA hardware

controller used by SVGA and S3 cards;	it uses	an 8-bit register, left-

shifted 3 bits, for what's really an 11-bit quantity.	 Other card

types	such as	ATI 8514/A may not have	this requirement, but we don't

know and the correction can't	hurt.  So round	the usable horizontal

scans	per second figure to 1176.

This figure (DCF / HSF rounded to a multiple of 8) is	the minimum HFL

you can use.	You can	get longer HFLs	(and thus, possibly, more

horizontal dots on the screen) by setting the	sync pulse to produce a

lower	HSF.  But you'll pay with a slower and more visible flicker

rate.

As a rule of thumb, 80% of the horizontal frame length is available

for horizontal resolution, the visible part of the horizontal	scan

line (this allows, roughly, for borders and sweepback	time --	that is,

the time required for	the beam to move from the right	screen edge to

the left edge	of the next raster line).  In this example, that's 944

ticks.

Now, to get the normal 4:3 screen aspect ratio, set your vertical

resolution to	3/4ths of the horizontal resolution you	just calculated.

For this example, that's 708 ticks.  To get your actual VFL, multiply

that by 1.05 to get 743 ticks.

About	that 4:3 --- a ratio of	4:3 for	width to height	of the displayed

area approximates the	Golden Section,	(1 + sqrt(5))/2.  Human	beings

seem to be wired to find this	kind of	rectangle pleasant to look at;

accordingly, video tubes and the standard resolutions	such as	800x600,

640x480 and 1024x768 all approximate it.  Though it's	psychologically

magic, it's not technically magic; nothing prevents you from using a

non-Golden-Section ratio if that will	get the	best use out of	your

screen real estate.

So, HFL=1176 and VFL=743.  Dividing 65MHz by the product of the two

gives	us a nice, healthy 74.4Hz refresh rate.	 Excellent!  Better than

VESA standard!  And you got 944x708 to boot, more than the 800 by 600

you were probably expecting.	Not bad	at all!

You can even improve the refresh rate	further, to almost 76 Hz, by

using	the fact that monitors can often sync horizontally at 2khz or so

higher than rated, and by lowering VFL somewhat (that	is, taking less

than 75% of 944 in the example above).  But before you try this

"overdriving"	maneuver, if you do, make *sure* that your monitor

electron guns	can sync up to 76 Hz vertical.	(the popular NEC 4D, for

instance, cannot.  It	goes only up to	75 Hz VSF).

So far, most of this is simple arithmetic and	basic facts about raster

displays.  Hardly any	black magic at all!

8.  Black Magic and Sync Pulses

OK, now you've computed HFL/VFL numbers for your chosen dot clock,

found	the refresh rate acceptable, and checked that you have enough

VRAM.	 Now for the real black	magic -- you need to know when and where

to place synchronization pulses.

The sync pulses actually control the horizontal and vertical scan

frequencies of the monitor.  The HSF and VSF you've pulled off the

spec sheet are nominal, approximate maximum sync frequencies.	 The

sync pulse in	the signal from	the adapter card tells the monitor how

fast to actually run.

Recall the two pictures above?  Only part of the time	required for

raster-scanning a frame is used for displaying viewable image	(ie.

your resolution).

Horizontal Sync:

By previous definition, it takes HFL ticks to	trace the a horizontal

scan line.  Let's call the visible tick count	(your horizontal screen

resolution) HR.  Then	Obviously, HR <	HFL by definition.  For

concreteness,	let's assume both start	at the same instant as shown

below:

|___	__ __ __ __ __ __ __ __	__ __ __ __

|_ _	_ _ _ _	_ _ _ _	_ _		   |

|_______________________|_______________|_____

0			   ^		   ^	 unit: ticks

|   ^       ^   |

HR  |       |  HFL

|   |<----->|   |

|<->|  HSP  |<->|

HGT1		HGT2

Now, we would	like to	place a	sync pulse of length HSP as shown above,

ie, between the end of clock ticks for display data and the end of

clock	ticks for the entire frame.  Why so?  because if we can	achieve

this,	then your screen image won't shift to the right	or to the left.

It will be where it supposed to be on	the screen, covering squarely

the monitor's	viewable area.

Furthermore, we want about 30	ticks of "guard	time" on either	side of

the sync pulse.  This	is represented by HGT1 and HGT2.  In a typical

configuration	HGT1 !=	HGT2, but if you're building a configuration

from scratch,	you want to start your experimentation with them equal

(that	is, with the sync pulse	centered).

The symptom of a misplaced sync pulse	is that	the image is displaced

on the screen, with one border excessively wide and the other	side of

the image wrapped around the screen edge, producing a	white edge line

and a	band of	"ghost image" on that side.  A way-out-of-place	vertical

sync pulse can actually cause	the image to roll like a TV with a mis-

adjusted vertical hold (in fact, it's	the same phenomenon at work).

If you're lucky, your	monitor's sync pulse widths will be documented

on its specification page.  If not, here's where the real black magic

starts...

You'll have to do a little trial and error for this part.  But most of

the time, we can safely assume that a	sync pulse is about 3.5	to 4.0

microsecond in length.

For concreteness again, let's	take HSP to be 3.8 microseconds	(which

btw, is not a	bad value to start with	when experimenting).

Now, using the 65Mhz clock timing above, we know HSP is equivalent to

247 clock ticks (= 65x10**6 *	3.8 *10**(-6)) recall M=10**6,

micro=10**(-6)

Vertical Sync:

Going	back to	the picture above, how do we place the 247 clock ticks

as shown in the picture?

Using	our example, HR	is 944 and HFL is 1176.	 The difference	between

the two is 1176-944=232 < 247!  Obviously we have to do some

adjustment here.  What can we	do?

The first thing is to	raise 1176 to 1184, and	lower 944 to 936.  Now

the difference = 1184-936= 248. Hmm, closer.

Next,	instead	using 3.8, we use 3.5 for calculating HSP; then, we have

65*3.5=227.  Looks better.  But 248 is not much higher than 227.  It's

normally necessary to	have 30	or so clock ticks between HR and the

start	of SP, and the same for	the end	of SP and HFL.	AND they have to

be multiple of eight!	 Are we	stuck?

No!  let's do	this, 936% 8==0, (936+32)% 8==0	too.  But 936+32=968,

968+227=1195,	1195+32=1227.  Hmm.. this looks	not too	bad.  But it's

not a	multiple of 8, so lets round it	up to 1232.

But now we have potential trouble, the sync pulse is no longer placed

right	in the middle between h	and H any more.	 Happily, using	our

calculator we	find 1232-32=1200 is also a multiple of	8 and

(1232-32)-968=232 corresponding using	a sync pulse of	3.57 micro

second long, still reasonable.

In addition, 936/1232	0.76 or	76%, still not far from	80%, so	it

should be all	right.

Furthermore, using the current horizontal frame length, we basically

ask our monitor to sync at 52.7khz(=65Mhz/1232) which	is within its

capability.  No problems.

Using	rules of thumb we mentioned before, 936*75%=702, This is our new

vertical resolution.	702*1.05=737, our new vertical frame length.

Screen refresh rate =	65Mhz/(737*1232)=71.6 Hz.  This	is still

excellent.

Figuring the vertical	sync pulse layout is similar:

|___ __	__ __ __ __ __ __ __ __	__ __ __

|_ _ _ _ _ _ _ _ _ _ _ _		|

|_______________________|_______________|_____

0		       VR	       VFL     unit: ticks

^   ^	    ^

|   |	    |

|<->|<----->|

VGT	VSP

We start the sync pulse just past the	end of the vertical display data

ticks.  VGT is the vertical guard time required for the sync pulse.

Most monitors	are comfortable	with a VGT of 0	(no guard time)	and

we'll	use that in this example.  A few need two or three ticks of

guard	time, and it usually doesn't hurt to add that.

Returning to the example: since by the definition of frame length, a

vertical tick	is the time for	tracing	a complete HORIZONTAL frame,

therefore in our example, it is 1232/65Mhz=18.95us.

Experience shows that	a vertical sync	pulse should be	in the range of

50us and 300us.  As an example let's use 150us, which	translates into

8 vertical clock ticks (150us/18.95us	8).

9.  Putting it All Together

The Xconfig file Table of Video Modes	contains lines of numbers, with

each line being a complete specification for one mode	of X-server

operation.  The fields are grouped into four sections, the name

section, the clock frequency section,	the horizontal section,	and the

vertical section.

The name section contains one	field, the name	of the video mode

specified by the rest	of the line.  This name	is referred to on the

"Modes" line of the Graphics Driver Setup section of the Xconfig file.

The name field may be	omitted	if the name of a previous line is the

same as the current line.

The dot clock	section	contains only the dot clock (what we've	called

DCF) field of	the video mode line.  The number in this field specifies

what dot clock was used to generate the numbers in the following

sections.

The horizontal section consists of four fields which specify how each

horizontal line on the display is to be generated.  The first	field of

the section contains the number of dots per line which will be

illuminated to form the picture (what	we've called HR).  The second

field	of the section indicates at which dot the horizontal sync pulse

will begin.  The third field indicates at which dot the horizontal

sync pulse will end.	The fourth field specifies the total horizontal

frame	length (HFL).

The vertical section also contains four fields.  The first field

contains the number of visible lines which will appear on the	display

(VR).	 The second field indicates the	line number at which the

vertical sync	pulse will begin.  The third field specifies the line

number at which the vertical sync pulse will end.  The fourth	field

contains the total vertical frame length (VFL).

Example:

#Modename	 clock	horizontal timing  vertical timing

"752x564"	  40	752 784	 944 1088  564 567 569 611

44.5  752 792  976 1240	564 567	570 600

(Note: stock X11R5 doesn't support fractional	dot clocks.)

For Xconfig, all of the numbers just mentioned - the number of

illuminated dots on the line,	the number of dots separating the

illuminated dots from	the beginning of the sync pulse, the number of

dots representing the	duration of the	pulse, and the number of dots

after	the end	of the sync pulse - are	added to produce the number of

dots per line.  The number of	horizontal dots	must be	evenly divisible

by eight.

Example:

horizontal numbers: 800 864 1024 1088

The sample line has the number of illuminated	dots (800) followed by

the number of	the dot	when the sync pulse starts (864), followed by

the number of	the dot	when the sync pulse ends (1024), followed by the

number of the	last dot on the	horizontal line	(1088).

Note again that all of the horizontal	numbers	(800, 864, 1024, and

1088)	are divisible by eight!	 This is not required of the vertical

numbers.

The number of	lines from the top of the display to the bottom	form the

frame.  The basic timing signal for a	frame is the line.  A number of

lines	will contain the picture.  After the last illuminated line has

been displayed, a delay of a number of lines will occur before the

vertical sync	pulse is generated.  Then the sync pulse will last for a

few lines, and finally the last lines	in the frame, the delay	required

after	the pulse, will	be generated.  The numbers that	specify	this

mode of operation are	entered	in a manner similar to the following

example.

Example:

vertical numbers: 600	603 609	630

This example indicates that there are	600 visible lines on the

display, that	the vertical sync pulse	starts with the	603rd line and

ends with the	609th, and that	there are 630 total lines being	used.

Note that the	vertical numbers don't have to be divisible by eight!

Let's	return to the example we've been working.  According to	the

above, all we	need to	do from	now on is to write our result into

Xconfig as follows:

< name >	  DCF	  HR  SH1 SH2	HFL   VR  SV1 SV2 VFL

where	SH1 is the start tick of the horizontal	sync pulse and SH2 is

its end tick;	similarly, SV1 is the start tick of the	vertical sync

pulse	and SV2	is its end tick.

#name	clock	horizontal timing   vertical timing    flag

936x702	65	936 968	1200 1232   702	702 710	737

No special flag necessary; this is a non-interlaced mode.  Now we are

really done.

10.  Questions and Answers

Q. The example you gave is not a standard screen size, can I use it?

A. Why not?  There is	NO reason whatsoever why you have to use

640x480, 800x600, or even 1024x768.  XFree86 driver lets you config

your hardware	with a lot of freedom.	It usually takes two to	three

minutes to come up the right one.  The important thing to shoot for is

high refresh rate with reasonable viewing area. not high resolution at

the price of eye-tearing flicker!

Q. It	this the *only*	resolution given the 65Mhz dot clock and 55Khz

HSF?

A. Absolutely	not!  You are encouraged to follow the general procedure

and do some trial-and-error to come up with a	setting	that's really to

your liking.	Experimenting with this	can be lots of fun.  Most

settings may just give you nasty video hash, but nothing you do can

actually damage a multi-sync monitor (unless you somehow force your

card to clock	it at way above	its bandwidth --- if you stick

reasonably close to the highest resolution the monitor is documented

to support this can't	happen).  Beware fixed-frequency monitors!  This

kind of hacking around *can* damage them.

Q. You just mentioned	two standard resolutions. In Xconfig, there are

many standard	resolutions available, can you tell me whether there's

any point in tinkering with timings?

A. Absolutely!  Take,	for example, the "standard" 640x480 listed in

the current Xconfig.	It employs 25Mhz driving frequency, frame

lengths are 800 and 525 => refresh rate   59.5Hz. Not	too bad.  But

28Mhz	is a commonly available	driving	frequency from many SVGA boards.

If we	use it to drive	640x480, following the procedure we discussed

above, you would get frame lengths like 812 and 505.	Now the	refresh

rate is raised to 68Hz, a quite significant improvement over the

standard one.

Q. But how about interlace/non-interlace?

A. At	a fixed	dot clock, an interlaced display is going to flicker

worse	than a non-interlaced one, which is why	the market has moved

away from them.  What	you buy	with the increased flicker is higher

resolution with a slower dot clock.  If the DCF were fast enough (say

90MHz	or up) even interlacing	wouldn't produce flicker -- but	at

present speeds, interlaced monitors are a bad	idea for X.

Q. Can you summarize what we have discussed so far?

A. In	a nutshell:

1.	for any	fixed driving frequency, raising max resolution	incurs

the penalty of lowering	refresh	rate and thus introducing more

flicker.

2.	if high	resolution is desirable	and your monitor supports it,

try to get a SVGA card that provides a matching	dot clock or

DCF. The higher, the better!

11.  Two More	Example	Calculations

Here's another hypothetical:

An adapter card has a	40 MHz clock rate.  A display has a range of

horizontal sync rates	from 30	KHz to 37 KHz.	The minimum number of

dots per line	is 40,000,000/37,000 = 1,081.081, or approximately 1,081

dots per line.

We'll	use this number	of dots	per line in the	following calculations.

So each line on our display must have	at least 1081 dots.  We	round

this up to 1088 to make it divisible evenly by eight.	 Now let's

assume that the horizontal sync pulse	should be 3.8 microseconds long.

We need to find out how many dots it takes to	make a 3.8 microsecond

pulse.  We do	this by	first finding out how many microseconds	are in

one dot.  Since there	are 40,000,000 dots per	second,	1/40,000,000 is

the number of	seconds	per dot.

1/40,000,000 = .000000025 = .025 microseconds	per dot

Thus the number of dots for a	3.8 microsecond	sync pulse is

3.8 microseconds = D dots x .025 microseconds/dot

or

D dots = (3.8	microseconds) /	(.025 microseconds/dot)	= 152 dots

So we	have 1088 dots per line, 800 of	which are the illuminated ones,

with 152 for the sync	pulse.	(Note that 152 is evenly divisible by

eight.  If it	weren't	we would round it up until it was evenly

divisible.)  This leaves us the task of calculating the time before

and after the	sync pulse that	is necessary for the display.

The rule of thumb for	this is	that we	need about 30 ticks of guard

time.	 In this particular case, allocating 32	dots is	convenient,

because all the other	quantities are divisible by 8.	This results in

the timing being 800 dots for	the viewable area, 152 dots for	the

pulse, and 1088 - (800 + 152)	= 136 dots to divide between the two

other	times.	Half of	136 is 68 dots,	so 68 dots are placed between

the illuminated dots and the sync pulse, and 68 dots are placed after

the sync pulse.  The horizontal numbers in the Xconfig line then

become

800 (800+68) (800+68+152) (800+68+152+68)

or

800 868 1020 1088

Now we want to calculate the vertical	numbers.  To begin, we must

remember that	the vertical numbers are not in	terms of dots or

microseconds per dot,	but are	expressed as numbers of	lines!	So we

have to calculate how	much time it takes to display a	single line.

That's easy, because we know each line is 1088 dots and each dot is

.025 microsecond.  Each line is, therefore,

(1088	dots/line) x (.025 microseconds/dot) = 27.2 microseconds/line

Since	we chose 800 visible dots per line, let's choose the number of

lines	to be such that	the ratio of horizontal	to vertical is 4 to 3.

Thus,	800 is 4 x 200,	so the number of visible lines should be 3 x 200

= 600.  Our target resolution	is 800x600.

We know that a vertical sync pulse should be in the range of 50 to 300

microseconds.	 If we chose 150 microseconds as a typical sync	pulse,

we find how many lines 150 microseconds is by	dividing 150 by	27.2

microseconds per line.

(150 microseconds/pulse) / (27.2 microseconds/line) =	5.51 lines/pulse

By rounding up (never	down) to 6 lines/pulse we now have the vertical

sync pulse width.

To guess at the total	number of lines	per frame (illuminated lines

plus nonilluminated lines in the border) we assume (from

"Videotiming...") that the total number of lines will	be 5% more than

the number of	viewable lines.	 So the	total number of	lines is

(600 lines) x	(1.05) = 630 total lines per frame

So now we must place the pulse in the	time between the end of	the

illuminated lines and	the end	of the frame.  Since we	have 630 total

lines, 600 illuminated lines,	and 6 lines for	the pulse, we have

630 -	600 - 6	= 24 lines left

Some displays	don't mind if the pulse	begins immediately after the

illuminated lines, but others	might want a line or two between the

last illuminated line	and the	beginning of the sync pulse.  Taking the

latter course	just to	be safe, we add	three lines between the	last

illuminated line and the beginning of	the pulse.  The	rest of	the

lines	are added after	the pulse ends.	 So the	vertical timing	numbers

become

600 (600+3) (600+3+6)	(600+3+6+21)

or

600 603 609 630

Before we do anything	else, we must check that the display can handle

630 lines/frame at 27.2 microseconds/line.  We do this by calculating

how many frames per second our configuration will generate, and

comparing it to the display manual's entry for vertical sync rate.

For 630 lines/frame at 27.2 micro- seconds/line, we have 630 x 27.2 =

17,136 microseconds/frame.  17,136 microseconds/frame	is 0.017136

seconds/frame, or 1/0.017892 frames/second.

1 / (0.017136	seconds/frame) = 58.4 frames/second

If the manual	says the vertical sync rate is 58.4 Hz,	or 58.4	Hz is in

the range of the display's vertical sync rate, we are	fine.  If the

display cannot handle	this rate, we'll have to change	the number of

lines	per frame by adjusting all of the timings proportionally.

Now we combine the horizontal	and vertical timing numbers together

with the resolution and clock	values to produce a test configuration

for Xconfig.	Our line becomes

"800x600"  40	 800 868 1020 1088  600	603 609	630

Now we have a	configuration of XFree86 to try.  It may not work if any

of our assumptions were grossly wrong, but in	most cases it should at

least	give us	a stable display.  Now it takes	a little experimentation

to produce something pleasing.

An actual calculation

My adapter card has a	40 MHz crystal on it so	I started with a 40 MHz

clock	rate.  My display's maximum horizontal sync rate is 37 KHz, so

the minimum dots per line are	40,000,000/37,000 = 1081.  My display's

vertical sync	rate is	the range from 50 Hz to	90 Hz.

My display's manual says that	the largest horizontal sync pulse is

3.92 microseconds.  With 0.025 microseconds per dot, the pulse is

(3.92	microseconds) /	(.025 microseconds/dot)	= 156.8	dots

Rounding this	up to the nearest number evenly	divisible by eight gives

160 dots.

The manual also says that the	time between the last illuminated dot

and the beginning of the sync	pulse must be at least 0.67

microseconds.	 The number of dots in 0.67 microseconds at a 40 MHz

clock	rate - remember	40 MHz is .025 microseconds/dot	- is

D dots = (0.67 microseconds) / (.025 microseconds/dot) = 26.8	dots

Since	26.8 is	not evenly divisible by	eight, round it	up to 32 dots.

My display's manual says the time after the sync pulse should	be 3.56

microseconds or more.	 In dots, 3.56 microseconds is

D dots = (3.56 microseconds) / (.025 microseconds/dot) = 142.4 dots

Round	142.4 up to 144, so that it's evenly divisible by eight.

So now for a horizontal line we have 800 illuminated dots, 32	dots

between the illuminated dots and the sync pulse, 152 dots for	the sync

pulse, and 144 dots after the	sync pulse.

800 +	32 + 160 + 144 = 1136

We now have a	line that is 1136 dots long.  This is greater than the

1088 we previously calculated, but remember that 1088	was the	MINIMUM

number of dots that could be on a line.  So 1136 dots	per line is okay

for starters.

The numbers to enter on the Xconfig line so far are

"800x?"  40  800 (800+32) (800+32+160) (800+32+160+144)...

or

"800x?"  40  800 832 992 1136...

A line of 1136 dots at .025 microsecond/dot means that a line

represents 1136 x .025 = 28.4	microseconds.

Since	we chose 800 dots/line horizontal resolution, we choose	600

lines/frame as the vertical resolution.

My display's manual says that	the vertical sync pulse	must be	at least

64 microseconds long.	 In terms of lines, 64 microseconds is

(64 microseconds/pulse) / (28.4 microseconds/line) = 2.25 lines/pulse

We round 2.25	up to 3	lines for the vertical sync pulse.

The manual says the time between the last displayed line and the start

of the sync pulse must be at least 318 microseconds, and the delay

after	the end	of the pulse must be at	least 630 microseconds.	 We

calculate how	many lines each	of these time periods represents as

follows.

(318 microseconds) / (28.4 microseconds/line)	= 11.20	lines

(630 microseconds) / (28.4 microseconds/line)	= 22.18	lines

We round each	of the times up	to become 12 lines before the sync pulse

and 23 lines after the pulse.	 This makes our	vertical timing	numbers

600 (600+12) (600+12+3) (600+12+3+23)

or

600 612 615 638

Checking the frame rate to see if it falls within the	rate of	the

display, we see that 638 lines/frame at 28.4 microseconds/line is

18,119 microseconds/frame, which is 55.19 frames/second.  My display

can handle anything from 50 Hz to 90 Hz, so the timing is all	right.

Putting the resolution, clock, horizontal, vertical timing numbers

together on a	video mode line	in Xconfig results in

"800x600"  40	 800 832 992 1136  600 612 615 638

This was the first video mode	I tried.  It turned out	not to be very

satisfactory because there was too much flicker.  I tried other

timings both above and below this setting as shown in	the following

example.  I finally settled on the "784x614" mode as a compromise

between flicker and resolution.

You'll notice	that almost all	of the clock frequencies are 40	MHz.

Through experimentation I found that higher frequencies were beyond my

adapter card's capabilities, and that	lower frequencies didn't provide

the resolution I wanted.

Example: Timings I have tried:

# the following line works but is right of center

"752x564" 40  752  784  944	1088  564 567 569 611

#	       44.5  752 792  976 1240	564 567	570 600

#

# this line	fixes the problem with the previous line

#"752x564"	   40  752  816	 976 1088  564 567 569 611

#

# trying to	increase the vertical display size, it works

#"752x614"	   40  752  816	 976 1088  614 617 619 661

#

# trying to	increase the horiz. display size, it works

#"784x564"	   40  784  816	 976 1088  564 567 569 611

#

# the following works but is to the	right of center

#"784x614"	   40  784  816	 976 1088  614 617 619 661

#

# the following corrects the uncentered problem of the previous one

"784x614" 40  784  848 1008	1088  614 617 619 661

#

# trying to	increase the display size

# the following works, the display is slightly off center to the left

#"800x614"	   40  800  864	1024 1088  614 617 619 661

#

# the following corrects the problem of the	previous entry

"800x614" 40  800  864 1024	1104  614 617 619 661

#

# increase the display size, it works

"816x614" 40  816  880 1040	1120  614 617 619 661

#

# increase the display size, it works

"800x620" 40  800  864 1024	1104  620 623 625 661

#

# increase the display size, it works

"816x620" 40  816  880 1040	1120  620 623 625 661

#

# increase the display size, it works

"832x630" 40  832  896 1056	1136  630 633 635 661

#

# change the display size, it works	but flickers badly

"848x618" 40  848  912 1072	1152  618 621 623 661

12.  Fixing Problems with the	Image.

OK, so you've	got your X configuration numbers.  You put them	in

Xconfig with a test mode label.  You fire up X, hot-key to the new

mode,	... and	the image doesn't look right.  What do you do?	Here's a

list of common problems and how to fix them.

You *move* the image by changing the sync pulse timing.  You *scale*

it by	changing the frame length (you need to move the	sync pulse to

keep it in the same relative position, otherwise scaling will	move the

image	as well).  Here	are some more specific recipes:

The horizontal and vertical positions	are independent.  That is,

moving the image horizontally	doesn't	affect placement vertically, or

vice-versa.  However,	the same is not	quite true of scaling.	While

changing the horizontal size does nothing to the vertical size or vice

versa, the total change in both may be limited.  In particular, if

your image is	too large in both dimensions you will probably have to

go to	a higher dot clock to fix it.  Since this raises the usable

resolution, it is seldom a problem!

The image is displaced to the	left or	right

To fix this, move the	horizontal sync	pulse.	That is, increment or

decrement (by	a multiple of 8) the middle two	numbers	of the

horizontal timing section that define	the leading and	trailing edge of

the horizontal sync pulse.

If the image is shifted left (right border too large,	you want to move

the image to the right) decrement the	numbers.  If the image is

shifted right	(left border too large,	you want it to move left)

increment the	sync pulse.

The image is displaced up or down

To fix this, move the	vertical sync pulse.  That is, increment or

decrement the	middle two numbers of the vertical timing section that

define the leading and trailing edge of the vertical sync pulse.

If the image is shifted up (lower border too large, you want to move

the image down) decrement the	numbers.  If the image is shifted down

(top border too large, you want it to	move up) increment the numbers.

The image is too wide	(too narrow) horizontally

To fix this, increase	(decrease) the horizontal frame	length.	 That

is, change the fourth	number in the first timing section.  To	avoid

moving the image, also move the sync pulse (second and third numbers)

half as far, to keep it in the same relative position.

The image is too deep	(too shallow) vertically

To fix this, decrease	(increase) the vertical	frame length.  That is,

change the fourth number in the second timing	section.  To avoid

moving the image, also move the sync pulse (second and third numbers)

half as far, to keep it in the same relative position.

Any distortion that can't be handled by combining these techniques is

probably evidence of something more basically	wrong, like a

calculation mistake or a faster dot clock than the monitor can handle.

Finally, remember that increasing either frame length	will decrease

your refresh rate, and vice-versa.

$XConsortium:	VidModes.sgml,v	1.3 95/01/27 16:14:33 kaleb Exp	$

Generated from XFree86: xc/programs/Xserver/hw/xfree86/doc/sgml/VidModes.sgml,v 3.5	1995/01/28 16:02:40 dawes Exp $